3GPP TSG-RAN WG3 Meeting #128	R3-253512

St. Julian’s, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1496 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of aerial UE’s altitude information reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3, TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on TS 23.256, for the aerial UEs, SA2 specified new procedures to enable the altitude reporting from NG-RAN to CN. Moreover, RAN3 agreed in RAN3#127 to reuse the Location Reporting Control and Location Report procedures to configure and perform the aerial UE’s altitude reporting, respectively, and that the altitude reporting can be configured to be periodic, threshold-based or event-based based on NG-RAN implementation. This CR follows the above mentioned agreements and introduces the necessary enhancements for the Handover Preparation and Retrieve UE Context procedures for the purpose of aerial UE’s altiture reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the Handover Preparation and Retrieve UE Context procedures to: - introduce a new codepoint ’aerial UE altitude information reporting’ for the existing Event Type IE within the Location Reporting Information IE of the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages, to indicate the start of the aerial UE’s altitude reporting; - new Aerial UE Altitude Information Reporting Configuration IE within the Location Reporting Information IE of the HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages, to configure altitude thresholds for reporting and, optionally, reporting periodicity; - updated ASN.1 accordingly. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous release of the specification because the changes only affect the Handover Preparation and Retrieve UE Context procedures to enable aerial UE’s altitude reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The aerial UE’s altitude reporting is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.47, 9.2.3.xx (new), 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.410 CR 0054 TS 38.413 CR 1284 TS 38.300 draftCR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Changes ***

#### 9.2.3.47	Location Reporting Information

This information element indicates how the location information should be reported.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED ( report upon change of serving cell, report UE moving presence into or out of the Area of Interest, …, report upon change of serving cell and Area of Interest, aerial UE altitude information reporting) |  | – |  |
| Report Area | M |  | ENUMERATED (Cell, …) |  | – |  |
| Area of Interest Information | O |  | 9.2.3.48 |  | – |  |
| Additional Location Information | O |  | ENUMERATED (Include PSCell, ...) |  | YES | Ignore |
| Aerial UE Altitude Information Reporting Configuration | C-ifEventTypeisAerialUEAltitudeInformationReporting |  | 9.2.3.xx |  | YES | Ignore |

| Condition | Explanation |
| --- | --- |
| ifEventTypeisAerialUEAltitudeInformationReporting | This IE shall be present if the Event Type IE is set to the value "aerial UE altitude information reporting". |

*** Next Change ***

#### 9.2.3.xx	Aerial UE Altitude Information Reporting Configuration

This IE indicates the configuration to be handled by the NG-RAN node to configure the aerial UE to report its altitude information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Minimum Altitude Threshold | M |  | INTEGER (-420..10000,…) | Corresponds to information provided in the Altitude IE, as defined in TS 38.331 [18], and it represents the minimum threshold for which the altitude reporting is initiated when the actual altitude goes below this value. Unit [meters] |
| Maximum Altitude Threshold | M |  | INTEGER (-420..10000,…) | Corresponds to information provided in the Altitude IE, as defined in TS 38.331 [18], and it represents the maximum threshold for which the altitude reporting is initiated when the actual altitude goes above this value. Unit [meters] |
| Reporting Periodicity | O |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 20480ms, 40960ms, 1min, 6mis, 12min, 30min, …) | Corresponds to information provided in the ReportInterval IE, as defined in TS 38.331 [18], and it indicates the periodicity for the reporting of altitude information that is applied if either of the thresholds are met. |

*** Next Change ***

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

*** skip unmodified parts ***

id-NRPPaPositioningInformation,

id-AerialUEAltitudeInformationReportingConfiguration,

maxEARFCN,

*** skip unmodified parts ***

maxnoofRSPPQoSFlows

FROM XnAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM XnAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES

FROM XnAP-Containers;

-- A

*** skip unmodified text ***

AerialUEAltitudeInformation ::= INTEGER (-420..10000, ...)

AerialUEAltitudeInformationReportingConfiguration ::= SEQUENCE {

minimumAltitudeThreshold						AerialUEAltitudeInformation,

maximumAltitudeThreshold						AerialUEAltitudeInformation,

reportingPeriodicity							ReportingPeriodicity														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AerialUEAltitudeInformationReportingConfiguration-ExtIEs} }		OPTIONAL,

...

}

AerialUEAltitudeInformationReportingConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

*** Next Change ***

#### -- E

*** skip unmodified parts ***

EventType ::= ENUMERATED {

report-upon-change-of-serving-cell,

report-UE-moving-presence-into-or-out-of-the-Area-of-Interest,

...,

report-upon-change-of-serving-cell-and-Area-of-Interest,

aerial-ue-altitude-information-reporting

}

*** Next Change ***

#### -- L

*** skip unmodified parts ***

LocationReportingInformation ::= SEQUENCE {

eventType			EventType,

reportArea			ReportArea,

areaOfInterest		AreaOfInterestInformation			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LocationReportingInformation-ExtIEs} } OPTIONAL,

...

}

LocationReportingInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-AdditionLocationInformation	CRITICALITY ignore	EXTENSION AdditionLocationInformation	PRESENCE optional},

{ ID id-AerialUEAltitudeInformationReportingConfiguration CRITICALITY ignore EXTENSION AerialUEAltitudeInformationReportingConfiguration  PRESENCE conditional

-- The above IE shall be present if the Event Type IE is set to the value “aerial UE altitude information reporting”},

...

}

*** Next Change ***

-- R

*** skip unmodified text ***

ReportingPeriodicity ::= ENUMERATED {

120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 20480ms, 40960ms, 1min, 6mis, 12min, 30min,

...

}

*** Next Change ***

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

*** skip unmodified text ***

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

*** skip unmodified text ***

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-AerialUEAltitudeInformationReportingConfiguration												ProtocolIE-ID ::= aaa

*** End of Changes ***