3GPP TSG-RAN WG3 Meeting #128	R3-253520

St. Julien's, Malta, 19 - 23 May, 2025

Agenda Item:		13.2

Source:		Ofinno

Title:	Open Issues on SN initiated Inter-CU SCG LTM

Document for:		Discussion and Decision

# Introduction

In RAN3 #127bis meeting, the following progress has been achieved on the agenda item 13.2, i.e., support for Inter-CU LTM NR-DC:

RAN3#127bis:

Include the following information in SN Change Required (Source SN to MN) message:

LTM request indication (explicit way)

Suggested PScell list (implicit way in INM, FFS on explicit way)

Measurement results (implicit way in INM)

Upper limit number of PScell(s) for each candidate SN (FFS)

CSI Resource configuration request (explicit way)

Early sync information request (explicit way)

Include the following information in SN Addition Request (MN to Candidate SN) message:

LTM request indication (explicit way)

Suggested PScell list (implicit way in INM, FFS on explicit way)

Measurement results (implicit way in INM)

Upper limit number of PScell(s) for the candidate SN (explicit way)

CSI Resource configuration request (explicit way)

Early sync information request (explicit way)

It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message.

For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN.

The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN.

Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message.

Continue to work on the solutions comparison and details…

This contribution discusses some open issues on SN initiated Inter-CU SCG LTM.

# Discussion

## 2.1	Access Success Notification for Inter-SN SCG LTM

In last RAN3 meeting, a baseline CR for TS 37. 340 was endorsed [2]. FIG.1 shows the procedure for SN initiated inter-SN SCG LTM procedure.

FIG. 1 SN-initiated inter-SN LTM (Figure 10.5.2-5 in BLCR of 37.340 [2]).

In the procedure description, there is an editor’s note on Step 23 to 29.

“23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.”

In step 23, currently it shows that target SN send an “Access Success” message to source SN directly. However, it is not an accurate expression. Since the Xn interface may not exist between source SN and target SN, the signalling should always be sent from target SN to MN and then MN pass it to the source SN. On example is shown in Fig.2 as option 1. HANDOVER SUCCESS message can be used instead of defining a new message.

In addition, option 2 is also shown in FIG.2. When MN receives RRC Reconfiguration Complete message successfully from the UE, it can know that the UE has accessed to a target SN successfully. At this time, MN may send an Access Success message, or SN Modification/Release Request message directly to the source SN.

FIG. 2 Proposed change on step 23 in FIG.1.

Based on the analysis above, the following proposals are suggested to RAN3:

Proposal 1: To revise step 23, i.e., access success message, for SN-initiated inter-SN LTM procedure and consider the following two options:

- Option 1: Access Success/Handover Success message is sent from target SN to MN and then MN pass it to the source SN.

- Option 2: When MN receives RRC Reconfiguration Complete message successfully from the UE, MN sends an Access Success/Handover Success message or SN Modification/Release Request message to the source SN.

Proposal 2: To adopt one of the two options proposed in FIG. 2 for BLCR to TS 37.340 (Figure 10.5.2-5 in [2]).

## 2.2	Cell Switch Notification message

As shown in FIG. 1, step 18 and 19 was agreed in stage 2. However, the stage 3 details for Cell Switch Notification message applied to dual connectivity scenario has not been reflected in the BLCR.

The following text proposal for TS 38.423 is proposed for agreement in the BLCR.

# Text Proposal for BLCR of TS 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.y.1	Cell Switch Notification

Editor’s note: Details need to be continued.

#### 8.y.1.1	General

The purpose of the Cell Switch Notification procedure is to enable the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the source NG-RAN node to the target NG-RAN node. The procedure uses UE-associated signalling.

For dual connectivity, the Cell Switch Notification procedure is to enable the source S-NG-RAN node to inform the target S-NG-RAN node, via the M-NG-RAN node, about the initiation of the cell switch command to the UE as specified in TS 37.340 [8]. This procedure is also used to transfer the selected TCI state from the source S-NG-RAN node to the target S-NG-RAN node, via the the M-NG-RAN node. The procedure uses UE-associated signalling.

#### 8.y.1.2	Successful Operation

Figure 8.y.1.2-1: Cell Switch Notification procedure. Successful operation.

Figure 8.y.1.2-2: Cell Switch Notification procedure for dual connectivity. Successful operation.

Handover

The source NG-RAN node initiates the procedure by sending a CELL SWITCH NOTIFICATION message.

Upon reception of the CELL SWITCH NOTIFICATION message, the target NG-RAN node shall, if supported, consider that a cell switch command was sent to the UE where one cell of the target NG-RAN node is indicated by the included Target Cell Global ID IE.

Dual Connectivity

The source S-NG-RAN node initiates the procedure by sending a CELL SWITCH NOTIFICATION message to the M-NG-RAN node. The M-NG-RAN node forwards the CELL SWITCH NOTIFICATION to the target S-NG-RAN node.

Upon reception of the CELL SWITCH NOTIFICATION message, the target S-NG-RAN node shall, if supported, consider that a cell switch command was sent to the UE where one PSCell of the target S-NG-RAN node is indicated by the included Target PSCell ID IE.

<<<<<<<<<<<<<<<<<<<< End of first Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Second Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

This message is also sent by the source S-NG-RAN node to the M-NG-RAN node or from the M-NG-RAN node to target the S-NG-RAN node to inform the target S-NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Direction: M-NG-RAN node  target S-NG-RAN node or source S-NG-RAN node  M-NG-RAN node (Dual Connectivity).

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node, at the source S-NG-RAN node, or at the M-NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node, at the M-NG-RAN node, or at the target S-NG-RAN node. | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 | Or target PSCell ID. | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |

<<<<<<<<<<<<<<<<<<<< End of Second Change >>>>>>>>>>>>>>>>>>>>

Proposal 3: To agree the stage 3 change above about CELL SWITCH NOTIFICATION message for TS 38.423.

# Proposals

Proposal 1: To revise step 23, i.e., access success message, for SN-initiated inter-SN LTM procedure and consider the following two options:

- Option 1: Access Success/Handover Success message is sent from target SN to MN and then MN pass it to the source SN.

- Option 2: When MN receives RRC Reconfiguration Complete message successfully from the UE, MN sends an Access Success/Handover Success message or SN Modification/Release Request message to the source SN.

Proposal 2: To adopt one of the two options proposed in FIG. 2 for BLCR to TS 37.340 (Figure 10.5.2-5 in [2]).

Proposal 3: To agree the stage 3 change above about CELL SWITCH NOTIFICATION message for TS 38.423.

# Reference

[1] R3-252320, Summary of offline discussion on Mobility Enh

[2] R3-253084, (BL CR to 37.340) Stage 2 for inter-CU LTM in NR-DC

[3] R3-253089, (BL CR to 38.423) Xn support for inter-CU LTM