3GPP TSG-RAN WG3 Meeting #128	R3-253521

St. Julien's, Malta, 19 - 23 May, 2025

Agenda Item:		21.3

Source:		Ofinno

Title:	Support of Uplink Rate Control for XR

Document for:		Discussion and Decision

# Introduction

In RAN #107 meeting, the following objectives were added for WID on XR Phase 3 [2]:

-  Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

-  Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

-  Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

-  QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and burst size to be provided over GTP-U

-  Support of exposure of available data rate [RAN3]

Note: Coordination with SA2, as needed

This contribution discusses some of the open issues on support of XR uplink Rate Control.

# Discussion

In last RAN3 meeting, the following agreements were achieved for uplink rate control, but the impacts on F1 are still open.

RAN3#127bis:

For Specify uplink congestion signalling:

Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

Introduce the same indication in XnAP to inform target gNB.

Continue the discussion on UL rate control.

Two options exist based on RAN2 agreements shown as follows.

RAN2 agreement on XR rate control … - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. …

2.1 Option 1: flows are indicated by MAC CE

Option 1 is that QoS flows are indicated by MAC CE. The main concern is that DU may not derive the bit rate for each QoS flow in case of M to 1 mapping to a DRB.

On the other hand, to support this option it is necessary for CU to notify to DU: Alt.1) which DRB is subject to rate control (DU determines the QoS flow identifier and the recommended bit rate), or Alt.2) which QoS flow is subject to rate control (DU determines the recommended bit rate for the QoS flow).

For example, one implementation of Alt. 1 is to add a new IE in UE CONTEXT SETUP/MODIFICATION REQUEST message, as shown as follows.

--------------------------------------------------------------------

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| … | … |  |  |  | … | … |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >> Indication of Bit Rate Adaptation | O |  | ENUMERATED (uplink, …) | Indicates that the DRB allows rate adaptation in the indicated direction. | YES | ignore |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| … |  |  |  |  |  |  |

For example, one implementation of Alt. 2 is to follow the agreement as in NG and Xn, i.e., add the Indication of Bit Rate Adaptation IE into the QoS Flow Level QoS Parameters.

#### -------------------------------------------------------------------

#### 9.3.1.45	QoS Flow Level QoS Parameters

This IE defines the QoS to be applied to a QoS flow, or to a DRB, or to a BH RLC channel, or to a Uu Relay RLC channel, or to a PC5 Relay RLC channel, or to a MRB.

NOTE:	For a BH RLC channel, the listed mandatory IEs and the GBR QoS Flow Information IE are applicable, where GBR QoS Flow Information IE may be present if BH RLC channel conveys the traffic belonging to a GBR QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.49 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.47 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.48 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.46 | This IE shall be present for GBR QoS Flows only and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Details in TS 23.501 [21]. This IE applies to non-GBR flows only and is ignored otherwise. | - |  |
| PDU Session ID | O |  | INTEGER (0 ..255) | As specified in TS 23.501 [21]. | YES | ignore |
| UL PDU Session Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.22 | The PDU session Aggregate Maximum Bit Rate Uplink which is associated with the involved PDU session. | YES | ignore |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| PDCP Terminating Node DL Transport Layer Address | O |  | Transport Layer Address 9.3.2.3 | DL Transport Layer Address of node terminating PDCP. Included for MN-terminated SCG bearers and SN-terminated MCG bearers. | YES | ignore |
| PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.319 |  | - |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.319 |  | - |  |
| Indication of Bit Rate Adaptation | O |  | ENUMERATED (uplink, …) | Indicates that the QoS Flow allows rate adaptation in the indicated direction. | YES | ignore |

Proposal 1: To consider the following two alternatives for supporting the first option, i.e., QoS flows are indicated by MAC CE:

Alt.1) CU to notify to DU which DRB is subject to rate control (DU determines the QoS flow identifier and the recommended bit rate), or

Alt.2) CU to notify to DU which QoS flow is subject to rate control (DU determines the recommended bit rate for the QoS flow).

2.2 Option 2: flows are indicated by RRC while MAC CE is per DRB

Option 2 is that QoS flows are indicated by RRC while MAC CE is per DRB. The main concern is that RRC may be not as quick as MAC CE for UE to know the QoS flow for uplink rate adaption.

Normally DU is the node to detect that congestion happens. DU may generate a recommended bit rate for a DRB. This can be sent to the UE via MAC CE for the DRB.

CU may not know what happens in DU and whether DU has sent MAC CE per DRB or not. In order to let CU knows that this situation happen or will happen soon, DU may notify to CU the recommended bit rate of the DRB to CU vis user plane. CU then figures out the QFI of a QoS flow mapped to the DRB. CU can indicate the QFI and the recommended bit rate to UE via RRC.

For example, one implementation is to add a new field in ASSISTANCE INFORMATION DATA frame in TS 38.425, as shown as follows.

#### -------------------------------------------------------------------

#### 5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 |  | 3 | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  |  | PDCP Dupl. Ind. | Assistance Info. Ind. | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  |  |  | Recommended Bit Rate |  | UL Congestion Information Ind. | DL Congestion Information Ind. | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Delay DU Result |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Congestion Information |  |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  |  | 0 or 2 |
| Recommended Bit Rate |  |  |  |  |  |  |  |  | FFS |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

Proposal 2: To enhance ASSISTANCE INFORMATION DATA frame for supporting RRC option.

2.3 Dual Connectivity Scenario

FIG. 1

In current assumption, it is possible that an XR service may be mapped to one DRB in RAN side. The DRB may be split and served by a master node and a secondary node as shown in FIG. 1. Thus, each node has its own MAC entity. Master node and secondary node may send its own recommended bit rate to UE. UE may be confused on how to report it to its upper layer for the same DRB to apply the rate control.

On the other hand, even though only master node or secondary node determines to send a recommended bit rate to UE, the other node may not know the situation and waste the resources allocated for that or lack of information to increase the resources for a potential increased bit rate.

Therefore, some least coordination/notification is necessary for master node and secondary node.

Proposal 3: To consider least coordination/notification between master node and secondary node for the determined recommended bit rate.

# Proposals

Proposal 1: To consider the following two alternatives for supporting the first option, i.e., QoS flows are indicated by MAC CE:

Alt.1) CU to notify to DU which DRB is subject to rate control (DU determines the QoS flow identifier and the recommended bit rate), or

Alt.2) CU to notify to DU which QoS flow is subject to rate control (DU determines the recommended bit rate for the QoS flow).

Proposal 2: To enhance ASSISTANCE INFORMATION DATA frame for supporting RRC option.

Proposal 3: To consider least coordination/notification between master node and secondary node for the determined recommended bit rate.

# Reference

[1] R3-251501, RAN3#127bis Meeting Agenda

[2] RP-240107, Revised WID on XR for NR Phase 3

[3] R3-252352, Summary of offline discussion on XR