3GPP TSG-RAN WG3 Meeting #128	R3-253523

St Julian’s, Malta, 19 – 23 May, 2025

Agenda Item:	20.2

Source:	ZTE Corporation

Title:	[TP to 38.455] Support of AI/ML assisted Positioning (case 3a)

Document for:	Other

- Introduction

The objectives of AI/ML for air interface WI [1] are listed as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Based on the current discussions in RAN1 and RAN2, the topic that requires normative work in RAN3 is AI/ML-based positioning. Further analysis indicates that Case 3a and Case 3b may have impacts on RAN3.

In previous meetings, the initial RAN3 progress was as follows:

The existing LCS framework is re-used for AI/ML positioning.

Case 3a agreements:

- LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

- How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

- NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

- RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

- Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

- WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

For case 3b:

- NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

In RAN3#127 meeting, further agreements for case 3a are reached:

Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement.

For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs.

WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

In RAN3#128, following agreements for case 3a are reached:

An NRPPA based solution could be used to collect UE location information in an opportunistic way, i.e. from UEs that are being positioned.

FFS on Whether to introduce an NGAP based procedure to collect UE location information in a proactive way, i.e. from UEs that are being served by the NG-RAN. (Updated Option 3 as in the LS)

In this contribution, we provide our understanding on the case 3a about assisted AI/ML positioning.

- Discussion

  - Case 3a (AI/ML assisted positioning)

2.2.1 Model training

The following opportunistic approach has been adopted as one of the supported methods for data collection:

Figure 1. Positioning Information collection for UEs under positioning

This solution can be used to collect data from both served and non-served UEs, while introducing minimal impact on legacy positioning procedures.

Due to the sensitive nature of Part B information, which may involve user privacy concerns, the LMF needs to indicate to the NG-RAN node whether it is able to provide such data.

Under the current opportunistic solution, there are two feasible options for this indication:

- Alternative 1: The LMF may inform the NG-RAN node prior to receiving the data collection request in the measurement response, or

- Alternative 2: The LMF may inform the NG-RAN node after the data collection request in the measurement response is received.

Both approaches are workable from a procedural standpoint

- It is proposed that RAN3 further discuss how the LMF should notify the NG-RAN node regarding the availability of Part B data (i.e., whether it can be provided or not).

If we go forward Alternative 2, there are two potential approaches to implement the failure indication:

- One option is to extend the existing NRPPa POSITIONING COLLECTION REPORT message to include a failure indication.

- The other option is to introduce a new NRPPa POSITIONING COLLECTION FAILURE message dedicated to reporting such failures.

Both options are technically feasible; however, we believe that introducing a new POSITIONING COLLECTION FAILURE procedure provides a more explicit and intuitive solution. This new message can also carry an appropriate cause value to indicate the reason why the Part B (label) information cannot be provided—such as high load conditions or user privacy concerns.

- Introduce a new NRPPa POSITIONING COLLECTION FAILURE message dedicated to reporting such failures, enabling LMF inform gNB part B is not available.

Following the RAN1 agreement, for the opportunistic approach, NG-RAN nodes can correlate Part A and Part B information using time stamp. Therefore, the matching issue is resolved and does not require further discussion in RAN3.

- RAN3 is not required to further discuss the correlation between Part A and Part B for the optimistic solution.

2.2.2 Model performance evaluation

In the previous meeting, RAN3 has achieved a working assumption that RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

And following is the RAN1 agreement on this issue:

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

RAN3 now agreed that NG-RAN node performs monitoring metric calculation for its own model. Now, there are 2 different ways for model performance evaluation：

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

For model performance evaluation, RAN3 discussed this topic during the R17 and R18 stages and concluded that model performance is out of RAN3's scope. This wording to reflect this into stage2 specification is still under discussion regarding whether it needs further revision, but the principle is to avoid transmitting model performance metrics over NG-RAN interfaces (e.g., loss, accuracy, etc.).

- Option 1 (gNB performs model monitoring metric calculation and makes the monitoring decision), also referred to as Option A, is the appropriate approach for model performance evaluation in Case 3a.

RAN1 requests confirmation from RAN3 regarding the performance monitoring metric calculation.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

  - Stage 2 impact

For the AI/ML topic, it is important to specify which network entity is responsible for model training and model inference. In the case of AI/ML for NG-RAN, the deployment is already documented in the specification. Therefore, the deployment of AI/ML-assisted positioning should also be explicitly mentioned in the specification.

For Case 3a, where model inference is performed at the gNB side, if the architecture follows a CU/DU split, then — in line with the established AI/ML for NG-RAN principles — the model inference is expected to be performed at the CU side.

For AI/ML-assisted positioning (Case 3a):

- Model training and Model inference is located in the NG-RAN node.

- Capture the deployment for AI/ML assisted positioning in the TS 38.401.

- Conclusion

Following is our proposals and observations.

- It is proposed that RAN3 further discuss how the LMF should notify the NG-RAN node regarding the availability of Part B data (i.e., whether it can be provided or not).

- Introduce a new NRPPa POSITIONING COLLECTION FAILURE message dedicated to reporting such failures, enabling LMF inform gNB part B is not available.

- RAN3 is not required to further discuss the correlation between Part A and Part B for the optimistic solution.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- Option 1 (gNB performs model monitoring metric calculation and makes the monitoring decision), also referred to as Option A, is the appropriate approach for model performance evaluation in Case 3a.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

- Capture the deployment for AI/ML assisted positioning in the TS 38.401.

- Text Proposal to TS 38.455

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Start of Changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Data Collection Transfer. This function enables the LMF to provide data information to the NG-RAN node in order to support AI/ML-assisted positioning.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Data Collection Transfer | Data Collection Update |

8	NRPPa procedures

8.1	Elementary procedures

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Data Collection Update | DATA COLLECTION UPDATE |

.5	Measurement Information Transfer

8.5.1	Measurement

8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider this as the time window(s) within which to perform the positioning measurements, as specified in TS 38.305 [18].

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

8.5.1.3	Unsuccessful Operation

Figure 8.5.1.3.1: Measurement procedure. Unsuccessful operation.

If the NG-RAN node cannot configure any of the requested measurements for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, it shall respond with a MEASUREMENT FAILURE message with an appropriate cause value.

8.5.1.4	Abnormal Conditions

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message but the NG-RAN node is unable to provide the measurement results within the indicated time, the NG-RAN node shall, if supported, respond with a MEASUREMENT FAILURE message with an appropriate cause value.

8.5.2	Data Collection Update

8.5.2.1	General

The Data Collection Update procedure allows the LMF to report AI/ML assisted positioning measurements to the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

8.5.2.2	Successful Operation

Figure 8.5.2.2.1: Measurement Report procedure. Successful operation.

8.5.2.3	Abnormal Conditions

Void.

9.1.4.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Data Collection Request | O |  | ENUMERATE (true, …) |  | YES | ignore |

9.1.4.5	DATA COLLECTION UPDATE

This message is sent by the LMF to report the positioning data for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Data Collection List |  | 0..1 |  |  | YES | reject |
| > Data Collection Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >> Label Information | M |  | FFS |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End of Changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

- Draft LS to RAN1

3GPP TSG-RAN WG3 Meeting #128														R3-25xxxx

St Julian’s, Malta, 19 – 23 May, 2025

Title:	[Draft] LS on AI/ML assisted Positioning about model performance monitoring

Response to:	-

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	ZTE Corporation (will be RAN3)

To:	RAN1

Cc:	RAN2

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: N/A

- Overall description

RAN3 discussed the following options for model performance monitoring in the case of case 3a,

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

- Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

- Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision

RAN3 prefers the option 1 that gNB performs model monitoring metric calculation and makes monitoring decision to avoid transmitting model performance metrics over NG-RAN interfaces.

- Actions

To RAN1:

ACTION: 	RAN3 kindly asks RAN1 to take the above information into consideration.

3	Dates of next TSG SA WG 5 meetings

RAN3#128	17 February 	- 21 February   2025		China