3GPP TSG-RAN WG3 Meeting #128	R3-253539

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:	17.3

Source:	Ericsson. Deutsche Telekom, China Unicom, Jio Platforms

Title:	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Other

# Introduction

The revised Work Item Description (WID) for “Enhancements of network energy savings for NR” [1] in Rel-19 states the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

In this contribution, we discuss issues and solutions regarding Objective 2 of the work item (see highlighted text above), focusing on the impact on the Xn interface.

In particular, we discuss the main principles of the inter NG-RAN node signaling to enable on-demand SIB1 (OD-SIB1) transmission, considering non-split architecture.

# General background for discussion

## General scenario

Figure 1 illustrates an example deployment scenario for OD-SIB1 transmission, where the “NES Cell” of gNB1 is meant to operate in OD-SIB1 transmission mode and the “Cell A” of gNB2 provides the UL Wake-Up Signal (WUS) configuration (that is needed to request SIB1 transmission in the NES Cell) to UEs. The scenario corresponds to Case 2 as defined by RAN1, assumed for normative work according to the WID.

Figure 1: An example deployment scenario for OD-SIB1 transmission (corresponding to Case 2).

We assume that, before the NES Cell operates in OD-SIB1 transmission mode, gNB1 provides the UL WUS configuration of its NES Cell to gNB2 in step 1. In step 2, gNB2 provides the UL WUS configuration of the NES Cell to UEs camping on (or connected to) Cell A. In step 3, when needed, a UE (in RRC Idle/Inactive mode) sends the UL WUS to the NES Cell and, in response, receives SIB1 from the NES Cell in step 4.

## Previous agreements

In relation to the impact on the Xn interface, the following agreements have been captured:

- RAN3#125 meeting [2]:

- NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

- RAN3#125-bis meeting [3]:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- RAN3#126 meeting [4]:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

The procedure uses non UE-associated signaling.

Cell A stores the UL WUS configuration information after it has received it.

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

- RAN3#127 meeting [5]:

It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

- RAN3#127-bis meeting [6]:

Step4 is not needed.

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that:

- One Choice “Start with UL WUS Condiguration”

- One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

These agreements can be summarized as follows:

- New class 1 Xn procedure for signaling the UL WUS configuration from NES-Cell gNB to Cell-A gNB

  - The initiating (request) message has the choice “start with UL WUS Configuration”, which carries the UL WUS configuration for the NES Cell and triggers Cell-A gNB to start broadcasting the UL WUS configuration in SIBx of Cell A.

  - After accepting the request, Cell-A gNB stores the UL WUS configuration for the NES Cell and broadcasts the UL WUS configuration in SIBx at the first opportunity.

  - The response message (successful or unsuccessful outcome) from Cell-A gNB indicates if the UL WUS configuration is to be broadcasted in Cell A or not.

  - The initiating (request) message has the choice “stop”, which triggers Cell-A gNB to stop broadcasting the UL WUS configuration in SIBx. All related information is removed from local memory in Cell-A gNB.

- New class 2 Xn procedure for signaling the interruption of UL WUS configuration broadcast by Cell-A gNB

  - Cell-A gNB signals to NES-Cell gNB that it stops the UL WUS configuration broadcast in SIBx.

In the following, we address some open issues relating to both the new class 1 and the new class 2 procedures.

# Discussion on the new procedures

## General aspects

Before discussing the details of the new procedures, we would like to make a few general comments that apply to the work we are doing in relation to OD-SIB1.

For the design of the new procedures, it is essential to prioritize simplicity to ensure robustness and minimize the potential for errors. Several contributions in the past meetings have proposed a multiplicity of options for the new procedures. While they may offer flexibility, they also introduce certain potential error cases and complicate the design process, so the benefit is not clear. By initially focusing on core functionality, we can establish a solid foundation that is both reliable and easy to implement. With such foundation, enhancements can be introduced in future releases.

RAN3 to agree to design simple and robust procedures for enabling UL WUS configuration provisioning.

The energy-saving benefits of OD-SIB1 largely come from enabling hardware to enter deeper sleep modes rather than from reduced energy expenditure associated with not transmitting SIB1 periodically. These savings, however, are contingent on the absence of concurrent processes that might prevent deeper sleep. To fully leverage these benefits, OD-SIB1 should be utilized in scenarios characterized by extended periods of low traffic, where only a few UEs sporadically and briefly connect to the network. By targeting these low-traffic scenarios, we can provide a robust framework for the NG-RAN to use the OD-SIB1 feature where it is most beneficial.

A consequence of the previous statement is that a NES Cell would only enter and exit OD-SIB1 mode a few times per day. The OD-SIB1 mode would be active during the night and other low traffic periods, and the NES Cell may exit this mode if a certain, sustained increase in traffic appears. In summary, we expect that the OD-SIB1 mode would be semi-static, and either active or inactive for very long periods of time.

The NES Cell is expected to be either in OD-SIB1 mode or periodic SIB1 transmission for very long periods of time, i.e., very few interactions between NES Cell and Cell A.

Finally, we would like to bring to RAN3 attention that, according to RAN1 discussion, the size of the UL WUS configuration is around 30 octets (240 bits) [7][8]. This is a very small amount of data to signal over RAN3 (mostly wired) interfaces.

The size of the UL WUS configuration is around 30 octets (240 bits) according to RAN1. This is a negligible amount of data to signal over RAN3 interfaces.

RAN3 to agree the UL WUS configuration is semi static and due to the small size of UL WUS Configuration, it is no issue to always send it when needed.

## Design of the new Class 1 procedure as start/stop request

### Agreed structure of the new Class 1 procedure

As already agreed, the new Class 1 procedure over Xn has two options: start and stop.

- The “start” option carries the UL WUS configuration information and is used when NES-Cell gNB intends to put the NES Cell into OD-SIB1 operation and cells of another gNB (Cell-A gNB) need to assist.

- The “stop” option is used by NES-Cell gNB to signal Cell-A gNB that it can stop providing the UL WUS configuration for a previously accepted request.

### No need to “pause/resume” the UL WUS configuration provisioning

Several contributions in the past meetings have proposed the possibility that the NES-Cell gNB can signal the Cell-A gNB to pause (or resume) the provisioning of the UL WUS configuration in SIBx. We believe that this functionality is not needed as we explain below.

A functionality to “pause/resume” the provisioning of the UL WUS configuration implicitly assumes that the periods when the provisioning is paused are relatively short. It does not make sense to pause the provisioning, e.g., for several hours. This has several disadvantages:

- Cell-A gNB would have to store in memory the context related to the Class 1 procedure and the UL WUS configuration itself, even when OD-SIB1 mode is off for long periods, which is a burden.

- A Cell-A gNB itself could not remove the said context from memory, and if it would, a resume request from NES-Cell gNB would lead to an error.

- Since the UL WUS configuration may potentially change, NES-Cell gNB would need to remember which version of the UL WUS configuration a Cell-A gNB has stored.

- Assume the SIBx in Cell A is full and NES-Cell gNB pauses the provisioning, if the now available space in SIBx is used for the UL WUS configuration of another cell, it may not be possible for NES-Cell gNB to resume the provisioning due to lack of space in SIBx.

Allowing the new Class 1 procedure to “pause” the broadcasting of the UL WUS configuration for long periods of time results in multiple complications.

As a result, it stands to reason that one could only safely pause the provisioning of the UL WUS configuration for a few seconds, or a few minutes at most. However, as stated in Observation 1, the NES Cell is expected to be either in OD-SIB1 mode or in periodic SIB1 transmission mode for very long periods of time in the relevant scenarios. There is ideally no need to “pause/resume” the provisioning.

Besides, as stated in Observation 2, the size of the UL WUS configuration is very small in RAN3 terms. Even if the NES Cell enters and exits the OD-SIB1 mode several times, the added overhead of signaling the UL WUS configuration over the Xn interface is almost negligible.

Therefore, there is no strong motivation to have the option to “pause” the provisioning of the UL WUS configuration from the point of view of interface overhead.

There is not sufficient motivation to introduce the option to “pause/resume” the provisioning of the UL WUS configuration from the point of view of interface overhead.

In the contributions that proposed having the “pause/resume” functionality, there are two arguments in favor of it. First, it is argued that when a NES-Cell gNB receives the UL WUS from a UE and starts provisioning the OD-SIB1 in response, it should pause the UL WUS configuration provisioning in the Cell A. This is not necessary since RAN2 agreed during the RAN2#127bis meeting that:

• A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

According to RAN2, Cell-A gNB can broadcast the UL WUS configuration of a NES Cell independently of the state of the NES Cell (i.e., both in OD-SIB1 mode or periodic SIB1 mode).

A second argument in favor of this functionality is that, by pausing the UL WUS configuration provisioning in the Cell A, the energy consumption of Cell-A gNB can be reduced. However, we recall that the basic scenario studied here assumes that the NES Cell is a capacity “booster” cell which is completely immersed in a larger coverage Cell A. Given that Cell A is providing coverage, it may be unlikely that the Cell-A gNB can significantly reduce its energy consumption by not provisioning the UL WUS configuration. Even if the NES-Cell’s UL WUS configuration is the only one in the SIBx of Cell A, which means that the SIBx might not be transmitted if the UL WUS configuration is removed, this might not lead to considerably longer new sleep opportunities. As stated in the objectives of the WID:

- Cell A: A cell that is periodically transmitting at least its own SIB1

Given that the Cell A is most likely a coverage cell, and it is periodically transmitting its own SIB1, it is unlikely that pausing the provisioning of the UL WUS configuration can result in a meaningful reduction of the energy consumption of Cell-A gNB. Moreover, Cell A could provide the SIBx on-demand which would further reduce any energy impact in Cell-A gNB.

It is unlikely that pausing the provisioning of the UL WUS configuration would result in a meaningful reduction of the energy consumption of Cell-A gNB.

Our analysis reveals that there is no motivation or gain in introducing the option to “pause/resume” the provisioning of the UL WUS configuration from any point of view, such as, the relevant scenarios, the interface overhead, RAN2 requirements, or Cell-A gNB energy consumption.

RAN3 to agree not to introduce a “pause/resume” or similar functionality in the new Class 1 procedure for UL WUS configuration provisioning.

As a consequence of Proposal 3, we further propose that the initiating (i.e., request) message for the new Class 1 procedure always carries the UL WUS configuration whenever NES-Cell gNB requests Cell-A gNB to start broadcasting the UL WUS configuration information in its SIBx. Therefore, the codepoint “start with UL WUS configuration” should be renamed “start” to reflect this behavior.

RAN3 to agree that the choice “start with UL WUS configuration” is renamed “start”, and that the UL WUS configuration information is mandatorily included in the request message whenever this choice is used.

### No need to “update” the UL WUS configuration provisioning

A few contributions in the past meetings have proposed the possibility that NES-Cell gNB can signal Cell-A gNB an updated UL WUS configuration for an accepted provisioning. Similarly to the case of “pause/resume”, we believe that this functionality is not needed as we explain below.

In line with Proposal 1, the simplest and most robust approach to “update” an UL WUS configuration is to first stop the ongoing provisioning, and then to request a new provisioning with the updated UL WUS configuration.

It is simplest and most robust to “update” an UL WUS configuration by first stopping the ongoing provisioning and then starting a new provisioning.

If RAN3 decides to include an explicit functionality to “update” a provisioning, then it needs to be discussed which parameters are allowed to be updated (e.g., NES Cells identities, Cell A identities, UL WUS configuration) and how to address possible error cases. For example, let us assume that an update to a provisioning does not fit in the available space in the SIBx of Cell A, should the “update” fail the whole procedure? As already stated, the simplest and most robust approach to “update” an UL WUS configuration is overwrite everything with a new start.

RAN3 to agree not to introduce an explicit “update” or similar functionality in the new Class 1 procedure for UL WUS configuration provisioning.

However, to streamline the signaling, the “stop” request for an ongoing provisioning and “start” request for a new provisioning (as suggested above) may be conveyed via a single request message. One way to realize this is the following: If the NES-Cell gNB sends a “start” request for an ongoing provisioning (e.g., by referring to an existing Provision ID), the Cell-A gNB interprets this request as an “overwrite” request, and as such, the Cell-A gNB stops the ongoing provisioning and starts a new provisioning according to the information that is included in the request message. This way, an update of an existing UL WUS configuration provisioning can be achieved without introducing an explicit functionality to do so, thus, without encountering the potential issues mentioned above.

RAN3 to agree that a “start” request can be sent for an ongoing UL WUS configuration provisioning (by referring to an existing provisioning context) and that such “start” request is interpreted by the receiving gNB as a stop the ongoing broadcasting for the ongoing provisioning plus a start request for a new provisioning and considering the information included in the request message.

### Further discussion related to the new Class 1 procedure

In summary, we propose to design the procedure as a simple start/stop request, sent by NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The Cell-A gNB can accept or reject the “start” request. The Cell-A gNB cannot reject the “stop” request, which is similar to, e.g., the PDU session resource release procedure (where the “release” will never fail).

RAN3 to agree to the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

In line with Proposal 1, the “start” request should include only one UL WUS Configuration. When multiple UL WUS Configuration exist, e.g., if NES-Cell gNB has multiple NES Cells with different configurations, the NES-Cell gNB can simply send multiple messages. If more than one configuration is included in the request message, it may be possible that the SIBx does not have enough available space for all the signaled configurations. By only including one UL WUS Configuration, RAN3 avoids discussing complex procedures with partial success. We further note that RAN2 agreed, during the RAN2#129bis meeting, that:

• WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

Considering this, we propose the “start” request may include a list of NES Cells to which the signaled UL WUS Configuration applies.

RAN3 to agree that, in the new Class 1 procedure, the “start” request includes only one UL WUS Configuration and may include a list of NES Cells to which the signaled UL WUS Configuration applies.

Another detail to consider is that, since Cell-A gNB can have many cells and cannot judge itself which of its cell(s) are supposed to act as Cell A, as discussed in [9], NES-Cell gNB shall further indicate in its “start” request to Cell-A gNB which cell(s) of Cell-A gNB should assist in providing the UL WUS configuration.

RAN3 to agree that, in the new Class 1 procedure, NES-Cell gNB indicates to Cell-A gNB which cell(s) of Cell-A gNB should act as Cell A (assisting in providing the UL WUS configuration).

We further believe, it is sufficient and most appropriate, that Cell-A gNB responds with “accept” if, and only if, it can provide the UL WUS configuration in all the cells requested (by NES-Cell gNB) to be Cell A, and “reject” otherwise. The Cell-A gNB cannot judge if broadcasting the UL WUS configuration in a subset of the requested cells will provide good Cell-A coverage for the NES Cell to be in OD-SIB1 mode; thus, the procedure should fail in this situation.

The Cell-A gNB may optionally inform NES-Cell gNB which cells were responsible for the procedure failing. In this way, NES-Cell gNB may learn about the unavailability of the failed cells and, potentially, may be able to find a different set of Cell-A cells to assist the NES Cell.

RAN3 to agree that the new Class 1 procedure fails if Cell-A gNB cannot provide the UL WUS configuration in all the requested cells. The Cell-A gNB may optionally indicate in the failure message which cells were responsible for the procedure failing.

## Design of the new Class 2 procedure to signal the feedback regarding the UL WUS configuration provisioning status

### Further discussion related to Cell-A gNB stopping the broadcasting

RAN3 has already agreed to introduce a new Class 2 XnAP procedure so that Cell-A gNB can provide feedback to NES-Cell gNB that it stops the UL WUS configuration provisioning in its SIBx. This is highlighted in the agreement below:

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

It is of paramount importance that there is no significant time gap between the moment Cell-A gNB stops broadcasting the UL WUS configuration in its SIBx and the moment the NES Cell reverts to periodic SIB1 transmission (if needed); otherwise, the NES Cell might not be accessible. For this reason, Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting. This will give NES-Cell gNB enough time to revert the NES Cell to periodic SIB1 broadcasting if needed.

RAN3 to clarify that Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting.

A final point to consider is the consequence of Cell-A gNB signaling that it stops the broadcasting of the UL WUS configuration in its SIBx. In our view, and in line with Proposal 1 and the past meeting agreements, if Cell-A gNB signals that it stops the broadcasting of the UL WUS configuration, the procedure should terminate.

In other words, the effect of signaling the Class 2 procedure from Cell-A gNB to NES-Cell gNB is the same as signaling the Class 1 procedure from NES-Cell gNB to Cell-A gNB using the option “stop”, i.e., Cell-A gNB removes the UL WUS Configuration and stops the broadcasting.

RAN3 to clarify that the Class 2 procedure (to signal that Cell-A gNB stops the UL WUS configuration provisioning) terminates the UL WUS configuration provisioning.

## Outline of the signaling flow for the two new procedures

We now summarize the signaling steps and the interaction of the two XnAP procedures described above. Figure 2 shows how the Class 1 procedure can be used by a NES-Cell gNB (gNB1) for activating OD-SIB1 transmission and then returning to periodic SIB1 transmission. The steps are as follows:

- In step 1, gNB1 generates an UL WUS configuration for the NES Cell.

- In step 2, gNB1 sends to gNB2 an UL WUS Configuration Provision Request message comprising the UL WUS configuration and an indication of the cell(s) of gNB2 in which gNB1 requests the UL WUS configuration to be provided. In response, gNB1 receives from gNB2 an UL WUS Configuration Provision Response message with an accept indication for the request.

- In step 3, gNB2 starts providing the UL WUS configuration in SIBx of the requested cell(s) as soon as possible, however subject to the SI modification period.

- In step 4, gNB1 starts operating the NES Cell in OD-SIB1 transmission mode.

- After some time has passed, when gNB1 considers that it is beneficial, it reverts the NES Cell to periodic SIB1 transmission in step 5.

- In step 6, gNB1 sends to gNB2 an UL WUS Configuration Provision Request message indicating that the provisioning can be stopped in the cell(s) of gNB2. In response, gNB1 receives from gNB2 an UL WUS Configuration Provision Response message with an accept indication for the request.

- In step 7, gNB2 stops providing the UL WUS configuration in SIBx of the requested cell(s).

Figure 2: An example of how the new Class 1 procedure can be used for activating OD-SIB1 transmission and then returning to periodic SIB1 transmission.

Figure 3 shows how the new Class 2 procedure can be used by Cell-A gNB to inform NES-Cell gNB that the UL WUS configuration provisioning has been stopped, which results in the deactivation of OD-SIB1 transmission and reverting the NES Cell to periodic SIB1 transmission.

Steps 1 to 4 are identical to the ones in Figure 2 and, in this case, it is gNB2 that decides to interrupt the provisioning in step 5. As mentioned in Proposal 10, the signaling of the UL WUS Configuration Provision Status message towards gNB1 (step 5a) should happen before, or simultaneously with, stopping the UL WUS configuration broadcasting (step 5b). After receiving this message, gNB1 reverts the NES Cell to periodic SIB1 transmission in step 6.

Figure 3: An example of how the new Class 2 procedure can be used to deactivate OD-SIB1 transmission and to revert to periodic SIB1 transmission.

RAN3 to agree to capture in a Stage-2 specification the signaling flow for activating OD-SIB1 transmission and reverting to periodic SIB1 transmission as depicted in Figure 2.

RAN3 to agree to capture in a Stage-2 specification the signaling flow for activating OD-SIB1 transmission and reverting to periodic SIB1 transmission as depicted in Figure 3.

A TP reflecting the above proposals is provided in Appendix A.

RAN3 to agree on the TP for BL CR for TS 38.423 provided in Appendix A.

# Conclusion

Based on the discussion in the previous sections we make the following observations:

Observation 1	The NES Cell is expected to be either in OD-SIB1 mode or periodic SIB1 transmission for very long periods of time, i.e., very few interactions between NES Cell and Cell A.

Observation 2	The size of the UL WUS configuration is around 30 octets (240 bits) according to RAN1. This is a negligible amount of data to signal over RAN3 interfaces.

Observation 3	Allowing the new Class 1 procedure to “pause” the broadcasting of the UL WUS configuration for long periods of time results in multiple complications.

Observation 4	There is not sufficient motivation to introduce the option to “pause/resume” the provisioning of the UL WUS configuration from the point of view of interface overhead.

Observation 5	According to RAN2, Cell-A gNB can broadcast the UL WUS configuration of a NES Cell independently of the state of the NES Cell (i.e., both in OD-SIB1 mode or periodic SIB1 mode).

Observation 6	It is unlikely that pausing the provisioning of the UL WUS configuration would result in a meaningful reduction of the energy consumption of Cell-A gNB.

Observation 7	It is simplest and most robust to “update” an UL WUS configuration by first stopping the ongoing provisioning and then starting a new provisioning.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN3 to agree to design simple and robust procedures for enabling UL WUS configuration provisioning.

Proposal 2	RAN3 to agree the UL WUS configuration is semi static and due to the small size of UL WUS Configuration, it is no issue to always send it when needed.

Proposal 3	RAN3 to agree not to introduce a “pause/resume” or similar functionality in the new Class 1 procedure for UL WUS configuration provisioning.

Proposal 4	RAN3 to agree that the choice “start with UL WUS configuration” is renamed “start”, and that the UL WUS configuration information is mandatorily included in the request message whenever this choice is used.

Proposal 5	RAN3 to agree not to introduce an explicit “update” or similar functionality in the new Class 1 procedure for UL WUS configuration provisioning.

Proposal 6	RAN3 to agree that a “start” request can be sent for an ongoing UL WUS configuration provisioning (by referring to an existing provisioning context) and that such “start” request is interpreted by the receiving gNB as a stop the ongoing broadcasting for the ongoing provisioning plus a start request for a new provisioning and considering the information included in the request message.

Proposal 7	RAN3 to agree to the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

Proposal 8	RAN3 to agree that, in the new Class 1 procedure, the “start” request includes only one UL WUS Configuration and may include a list of NES Cells to which the signaled UL WUS Configuration applies.

Proposal 9	RAN3 to agree that, in the new Class 1 procedure, NES-Cell gNB indicates to Cell-A gNB which cell(s) of Cell-A gNB should act as Cell A (assisting in providing the UL WUS configuration).

Proposal 10	RAN3 to agree that the new Class 1 procedure fails if Cell-A gNB cannot provide the UL WUS configuration in all the requested cells. The Cell-A gNB may optionally indicate in the failure message which cells were responsible for the procedure failing.

Proposal 11	RAN3 to clarify that Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting.

Proposal 12	RAN3 to clarify that the Class 2 procedure (to signal that Cell-A gNB stops the UL WUS configuration provisioning) terminates the UL WUS configuration provisioning.

Proposal 13	RAN3 to agree to capture in a Stage-2 specification the signaling flow for activating OD-SIB1 transmission and reverting to periodic SIB1 transmission as depicted in Figure 2.

Proposal 14	RAN3 to agree to capture in a Stage-2 specification the signaling flow for activating OD-SIB1 transmission and reverting to periodic SIB1 transmission as depicted in Figure 3.

Proposal 15	RAN3 to agree on the TP for BL CR for TS 38.423 provided in Appendix A.

# References

RP-250343, Revised WID on enhancements of network energy savings for NR

R3-244708, Summary of offline disc at RAN3#125, Ericsson (Moderator)

R3-245737, Summary of offline disc at RAN3#125-bis, Ericsson (Moderator)

R3-247806, Summary of offline disc at RAN3#126, Ericsson (Moderator)

R3-250801, Summary of offline disc at RAN3#127, Ericsson (Moderator)

R3-252387, Summary of offline disc at RAN3#127-bis, Ericsson (Moderator)

R1-2407057, Study of on-demand SIB1 for UEs in idle/inactive mode for NES, Ericsson

R1-2407103, On-demand SIB1 for NES, Fraunhofer IIS et al.

R3-245498, Xn impact of On-demand SIB1 for UEs in idle/inactive mode, Ericsson

Text Proposal for BL CR for TS 38.423

*************** Start of TP ***************

### 8.4.x	UL WUS Configuration Provision

#### 8.4.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to an NG-RAN node2 and to request the NG-RAN node2 to transmit the provided UL WUS configuration information in one or more of its cells.

The procedure is also used to enable the NG-RAN node1 to request the NG-RAN node2 to stop the transmission of the provided UL WUS configuration information.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: UL WUS Configuration Provision, successful operation

If the “start” is included in the UL WUS CONFIGURATION PROVISION REQUEST  message, the NG-RAN node2 shall, if supported, broadcast UL WUS Configuration (Naming FFS) in SIBx (SIB naming FFS) and reply with the UL WUS CONFIGURATION PROVISION RESPONSE  message. The NG-RAN node2  stores the UL WUS configuration information (Naming FFS).

If the “stop” is included in the UL WUS CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, stop broadcasting UL WUS Configuration (Naming FFS) in SIBx (SIB naming FFS). NG-RAN node2 removes the received UL WUS Configuration.

If the “start” is included in the UL WUS CONFIGURATION PROVISION REQUEST message for an existing Provision ID, the NG-RAN node2 stops the ongoing broadcasting and overwrites the early received UL WUS Configuration (Naming FFS) and the related information.

#### 8.4.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot broadcast the provided UL WUS configuration information in SIBx (Naming FFS) of all the cells indicated in the Assisting Cells List IE in the UL WUS CONFIGURATION PROVISION REQUEST message, or a failure occurs during UL WUS Configuration Provision (Naming FFS), the NG-RAN node2 shall send the UL WUS CONFIGURATION PROVISION FAILURE message. The message shall contain the Cause IE with an appropriate value. The NG-RAN node2 may include the Assisting Cells Failed List IE in the UL WUS CONFIGURATION PROVISION FAILURE message with the list of cells which could not provide the UL WUS configuration information.

#### 8.4.x.4	Abnormal Conditions

Void.

### 8.4.y	UL WUS Configuration Provision Status

#### 8.4.y.1	General

This procedure is initiated by an NG-RAN node2 to inform a neighbouring NG-RAN node1 about a previously admitted UL WUS configuration provision being stopped.

The procedure uses non UE-associated signalling.

#### 8.4.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Provision Status, successful operation

The NG-RAN node2 shall indicate that a previously admitted UL WUS configuration provision in the UL WUS CONFIGURATION PROVISION STATUS message is being stopped.

#### 8.4.y.3	Unsuccessful Operation

Not applicable.

#### 8.4.y.4	Abnormal Conditions

Void.

*************** Next change ***************

#### 9.1.3.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >>UL WUS Configuration | M |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331 [10]. |  |  |
| >>OD-SIB1 Cells List | M |  | Cell List 9.2.3.xxx | List of cells of the NG-RAN node1 for which the request applies. |  |  |
| >>Assisting Cells List | M |  | Cell List 9.2.3.xxx | List of cells of the NG-RAN node2 requested to provide the UL WUS configuration. |  |  |
| >Stop |  |  |  |  |  |  |
| >>OD-SIB1 Cells List | O |  | Cell List 9.2.3.xxx | List of cells of the NG-RAN node1 for which the request applies. |  |  |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.y	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration will be provided in the requested cells of the NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.z	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration cannot be provided in the requested cells of the NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Assisting Cells Failed List | O |  | Cell List 9.2.3.xxx | List of cells of the NG-RAN node2 which cannot provide the UL WUS configuration. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.t	UL WUS CONFIGURATION PROVISION STATUS

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted UL WUS configuration provision is being stopped.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the provision of the UL WUS configuration in the assisting cells is stopped. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.2.3.xxx	Cell List

This IE is used to indicate the NR Cell List.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell List Items |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  |
| >NR CGI | M |  | 9.2.2.7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

*************** End of TP ***************