3GPP TSG-RAN WG3 Meeting #128	R3-253544

Malta, MT, 19 - 23 May 2025

Agenda Item:	14.3

Source:	Huawei

Title:	(TP to BL CR for TS 38.300 and 38.413) Discussion on Suspend/Resume for hard FLSO and the interface management

Document for:	discussion

# 1	Introduction

In this paper, we continue to discuss the NG suspend/resume during the hard feeder link switch over for the NTN regenerative payload.

We also propose TP against 38.300 and 38.413 BL CR to align its with our proposal.

# 2	Discussion

## 2.1. Recap of the discussion history

For the feeder link switch over (FLSO), RAN3 already made the following agreements.

Feeder link switch issue:

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration.

The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.

Thought it was already agreed that the current mechanisms are sufficient for the TNL management during feeder link switch, the previous meetings mainly focused on whether to introduce the NG suspend/resume procedure/indication for the following two different use cases:

- Case 1: hard feeder link switchover

- Case 2: NG interface management when the NTN payload connects/leaves the AMF.

In the RAN3#127bis meeting, it was agreed that no new NGAP suspend/resume procedures are needed in R19 as follows:

- No new NGAP suspend/resume procedures in R19.

Meanwhile the following texts on the hard FLSO are captured into the stage 2 BLCR in [1], with an editor’s note.

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs. Editor’s Note: FFS whether this text is needed.

Also, the stage 3 BLCR in [2] introduced the Downlink NG Transmission Control IE in the RAN Configuration Update message to indicate suspend or resume the downlink NG Transmission, without specifying whether this is for hard FLSO, or for the NG interface management.

If the Downlink NG Transmission Control IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, suspend or resume the downlink NG transmission accordingly.

## 2.2. Case 1 analysis - hard feeder link switchover

The main arguments to introduce the Downlink NG Transmission Control is to inform the CN to tentatively suspend NG transmission before hard FLSO, then send the resume indicator to resume the NG transmissions.

This is to our understanding more like an unnecessary optimization, given that there are multiple ways for the AMF to be aware the the exact moment when the Hard FLSO occurs:

- The AMF can calculate the moment of Hard FLSO based on some information from OAM, such as the satellite ephemeris, ground station position, and the minimum elevation angle of the ground station.

- The AMF can use some information from the underlying connection, such as when the SCTP connection of NGAP is shutdown, the AMF could know that the connection to the ground station has been disconnected.

- The AMF can obtain some information associated with the FLSO from the NTN Control function, such as when to suspend data or when to resume transmission.

Further, it was already agreed the AMF can be configured with the supported TAIs of an on-board gNB as follows.

The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration.

And the reply SA5 LS in [3] clearly described that the AMF is pre-configured with multiple subsequent time windows instances within which the NTN gNB ID and supported TAI list are indicated.  Hence it is much simple for the AMF is pre-configured with the time instance of the Hard FLSO, considering it is static nature.

Therefore, if the AMF knows the feeder link disconnection, it will suspend DL data transmission on its own, without the need for additional suspension indication from gNB to notify the CN. The same is true for the resuming procedure.

Observation 1: The AMF can suspend/resume the data transmission without an additional indication from gNB to notify the CN when the FLSO occurs.

- No need to introduce the suspend/resume indication for hard FLSO towards the AMF.

## 2.3. Case 2 analysis - NG interface management when the NTN payload connects/leaves the AMF

As observed from the proponent proposals, the claimed advantage for the NG suspend/resume procedure may be a little signalling overhead reduction when establishing the NG connection with the same AMF again and not re-establishing the interface context (NG configurations). But it is very doubtful whether the same NG configurations can be actually reused when the satellite gNB accesses the same AMF again. For example, when the gNB resumes NG connection from different location, the supported TA information could be different, so the old stored supported TA is useless; and, if the gNB resumes NG connection via a different GW, the current TNL address is different from the old TNL address, so the old NG-U tunnel configuration is also outdated.

Considering the feeder link switch is generally caused by the changing of satellite coverage, and as analyzed above, if the coverage area changes and GW changes, the NG configurations cannot be reused directly.

Observation 2: The old NG configurations cannot be reused when the gNB accesses the AMF at different location or via different GW.

In addition, the NG suspend/resume operation requires the gNB stores those NG configurations related to multiple AMFs. Considering the earth rotation, it may take a long time for the gNB connects to the same AMF again, which causes much storage consumption on the gNB for retaining multiple AMFs’ NG configurations.

Observation 3: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

Further, with the introduction of the NG removal procedure, there is no need to introduce another different solution since this would introduce high implementation complexities and lead to inter-operation issues.

- No need to introduce the suspend/resume indication for NG interface management when the NTN payload connects/leaves the AMF.

## 2.4. No AMF change during FLSO

The previous meeting also had the following agreement, to specify that the AMF is not changed during the FLSO.

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

We consider this is an important agreement applicable for all RRC state UEs, which should be captured in the stage 2.

- RAN3 to agree the Stage 2 TP to capture that during the feeder link switch, AMF is not changed for the UEs (including idle/inactive/connected UEs).

# 3	Conclusion

This contribution discusses the NG suspend/resume in NR NTN regenerative payload. It is proposed:

Observation 1: The AMF can suspend/resume the data transmission without an additional indication from gNB to notify the CN when the FLSO occurs.

Observation 2: The old NG configurations cannot be reused when the gNB accesses the AMF at different location or via different GW.

Observation 3: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

- No need to introduce the suspend/resume indication for hard FLSO towards the AMF.

- No need to introduce the suspend/resume indication for NG interface management when the NTN payload connects/leaves the AMF.

- RAN3 to agree the Stage 2 TP to capture that during the feeder link switch, AMF is not changed for the UEs (including idle/inactive/connected UEs).

- Agreed the associated TP for 38.300 and 38.413 in Annex.

# 4	References

- R3-252523 Support for Regenerative Payload and MBS broadcast in NR NTN, Ericsson et al.

- R3-252525 Support for Regenerative Payload and MBS broadcast in NR NTN, CATT et al.

- R3-251528 Reply LS to LS on reply to LS on OAM requirements to support regenerative payload (SA5(Huawei))

# Annex A: for TS 38.300 against R3-252523

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN payloads.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

#### 16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

NOTE1:	For regenerative payload, the AMF is not changed for NTN payload served all RRC state UEs during the feeder link switch over.

NOTE2:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, PDU Session Resource Modify Indication procedure) may be used.

The point in time when the hard feeder link switch over is performed at the AMF is pre-configured or determined by the NTN control function.

# Annex B: 2	TP for TS 38.413 against R3-252525

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.7.2	RAN Configuration Update

#### 8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

#### 8.7.2.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

WUS-Assistance-Information,

XrDeviceWith2Rx,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-XrDeviceWith2Rx,

id-SLPositioningRangingServiceInfo,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- RAN Configuration Update Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- RAN CONFIGURATION UPDATE

--

-- **************************************************************

RANConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateIEs} },

...

}

RANConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RANNodeName								CRITICALITY ignore	TYPE RANNodeName									PRESENCE optional	}|

{ ID id-SupportedTAList							CRITICALITY reject	TYPE SupportedTAList								PRESENCE optional	}|

{ ID id-DefaultPagingDRX						CRITICALITY ignore	TYPE PagingDRX										PRESENCE optional	}|

{ ID id-GlobalRANNodeID							CRITICALITY ignore	TYPE GlobalRANNodeID								PRESENCE optional	}|

{ ID id-NGRAN-TNLAssociationToRemoveList		CRITICALITY reject	TYPE NGRAN-TNLAssociationToRemoveList			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX					CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX						PRESENCE optional	}|

{ ID id-Extended-RANNodeName					CRITICALITY ignore	TYPE Extended-RANNodeName							PRESENCE optional	}

,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

DL-NGU-TNLInformationReused ::= ENUMERATED {

true,

...

}

DirectForwardingPathAvailability ::= ENUMERATED {

direct-path-available,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-PDUSessionListMTCommHReq								ProtocolIE-ID ::= 431

id-MaximumDataBurstVolume 								ProtocolIE-ID ::= 432

id-MN-only-MDT-collection								ProtocolIE-ID ::= 433

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-MBS-IntendedServiceAreaList							ProtocolIE-ID ::= x

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>