3GPP TSG-RAN WG3 Meeting #128	R3-253546X

Malta, MT, 19th – 23rd May 2025

Agenda Item:	15.2

Source:	Huawei, Ericsson, CATT

Title:	(TP for TS 36.300) Support of Full eNB as Regenerative Payload - TNL & UP aspetcs

Document for:	Other

# 1	Introduction

This contribution proposes TP to TS 38.300 on the TNL change during the feeder link switch and TNL Functionality for IoT NTN for IoT NTN Regenerative payload.

# 2	Discussion

## 2.1 TNL change during the feeder link switch over

Regarding the TNL change during the feeder link switch over, the previous RAN3-127 meeting agreed that the exiting procedures could be used to indicate the TNL addresses to the AMF, as follows [1]:

- Proposal 2, in case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used.

Accodingly, the following texts are introduced in the section 16.14.4 Switchover in the BLCR [2]

NOTE: In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, PDU Session Resource Modify Indication procedure) may be used.

For the full eNB as regenerative payload, the same agreement can be taken for the TNL address change due to the feeder link switch over. We note also that the multiple SCTP is not supported on LTE, but as SA2 already pointed the TR 23.700-29, S1 can use the UE Retention Information IE.

- RAN3 to agree that in case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used for full eNB as Regenerative Payload, and agree the stage 2 TP.

## 2.2 TNL Functionality for the feeder link switch over

At the previous RAN3-127bis meeting [1], given that the AMF is not changed during the feeder link switch over for the regenerative payload, it was agreed that the transfer of the affected UE(s) could be performed by means of handover or TNL functionality for NR NTN.

Accordingly, the following texts are introduced in the section 16.14.4 Switchover in the BLCR [2]

#### 16.14.4.3 Procedures The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

For the full eNB as regenerative payload, the same agreement can be taken for the transfer of the affected UEs. Indeed multiple SCTP is not supported on LTE, but as SA2 already pointed the TR 23.700-29, S1 can use the UE Retention Information IE.

- RAN3 to agree that for eNB as regenerative payload, the transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and agree the stage 2 TP.

# 3	Conclusion

This paper provides analysis for the eNB regenerative payload and FLSO, the following proposal is made:

- RAN3 to agree that in case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used for full eNB as Regenerative Payload, and agree the stage 2 TP.

- RAN3 to agree that for eNB as regenerative payload, the transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and agree the stage 2 TP.

# 4 Reference

- RAN3 #127bis chairman notes.

- R3-252523 Support for Regenerative Payload and MBS broadcast in NR NTN, Ericsson et al.

# 5	TP for TS 36.300 against R3-250861

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 23.21.5	Switchover

<<<<<<<<<<<<<<<<<<<< Unchanged Texts Omitted >>>>>>>>>>>>>>>>>>>>

#### 23.21.5.3	Procedures

The NTN control function determines the point in time when the feeder link switchover between two eNBs is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UEs' context between the two eNBs at feeder link switchover is performed by means of either S1 based or X2 based handover or TNL functionality, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, E-RAB Modification Indication procedure) may be used.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>