3GPP TSG-RAN WG3 Meeting #R3-128		R3-253554 St Julians, Malta, May 19-23, 2025

Title: 	gNB-DU initiated LTM resource reconfiguration

Source: 	Rakuten, Qualcomm Inc, NTT DOCOMO INC.

Agenda item:	13.2

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss the modification of gNB-DU configured radio resources after the LTM candidate cell configuration has been prepared and the importance of gNB-DU initiated reconfiguration of LTM candidate cell radio resource configuration.

# 2. Discussion

## 2.1 Resources requiring modification post candidate cell preparation

The lead time (duration between LTM candidate cell preparation and execution) during an LTM cell switch could be unpredictably long.

Once the LTM candidate cell configuration has been prepared, it is highly likely that resources like RACH preambles for early UL sync, RACH based LTM cell switch, Configured Grant resources for UL data transmission etc at the candidate cell may have to undergo modification because of UE mobility during the lead time or changes in radio resource availability at the candidate cell.

However, in Rel18, gNB-DU initiated LTM candidate cell re-configuration is not permitted by RAN3. The same has been adopted as a baseline in Rel 19 also. This is illustrated using the figure below

Figure 1: Signaling flow illustrating gNB-DU initiated candidate cell reconfiguration

The gNB-DU is only permitted to cancel an already prepared configuration but not re-configure. The idea behind this it that the gNB-CU will initiate a fresh candidate cell configuration post the cancel procedure.

- This is a key radio resource limitation considering that we are referring to modification of physical resources whose availability may change with time depending on traffic situation. Moreover, a fresh configuration is time consuming and could result in candidate cell configuration being useless.

- There could be changes in resource status and availability at the candidate gNB-DU between LTM candidate cell preparation and execution. For example, the RACH preamble allocated for CFRA LTM cell switch could change based on UE mobility. Once the candidate cell configuration is prepared, it can either executed or cancelled. This, in our view, is too inflexible and limiting genuine cases requiring gNB-DU triggered re-configuration.

- RAN2 has agreed in both Rel 18 and 19 that the RACH preamble could be delivered to the UE along with LTM MAC CE Cell switch command. This is to ensure that an appropriate preamble is used by the UE in case of RACH-based cell switch. Based on the current RAN3 agreement, a simple RACH preamble modification also isn’t possible after the candidate cell configuration has been prepared.

The radio resource reservation changes may result in a modified candidate/target cell configuration at the candidate/target gNB-DU This could be delivered to the UE in two ways.

- Using a candidate gNB-DU initiated F1: UE Context Modification Required procedure via the gNB-CU.

- Candidate gNB-DU indicating to the gNB-CU that a modification is required, for example, using a specific cause value in the LTM Cancel procedure. This could be followed by the gNB-CU triggering a re-configuration of candidate cell configuration.

NOTE: The proposed changes are completely limited to gNB and doesn’t have any UE impacts.

Proposal 1: RAN3 to allow candidate gNB-DU to request modification of candidate cell configured radio resources (ex: CG grant, RACH resources used for pre-UL sync, RACH based LTM etc) to enable CU to trigger re-configuration of LTM candidate cell resource configuration.

# 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: RAN3 to allow candidate gNB-DU to request modification of candidate cell configured radio resources (ex: CG grant, RACH resources used for pre-UL sync, RACH based LTM etc) to enable CU to trigger re-configuration of LTM candidate cell resource configuration.

# 4. Reference

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”