3GPP TSG-RAN3 Meeting #128                                                                   R3-253564

# St.Julian, Malta, May 19th – 23rd, 2025                                    Revision of R3-251757

Agenda Item:	21.3

Source: 	Qualcomm Inc.

Title: 	R19 XR Signaling Enhancements

Document for:	Discussion/Agreement

# Introduction

Following are R19 revised XR WID [1] objectives with RAN3 impacts.

-  Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

-  Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

-  Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

-  QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and burst size to be provided over GTP-U

-  Support of exposure of available data rate [RAN3]

Note: Coordination with SA2, as needed

RAN3#127bis Agreements:

For support of DL PDU Set marking without PDU Set QoS:

From SMF to gNB direction: introduce DL PDU Set Information Marking Support Indication IE in the QoS Flow Level QoS Parameters IE. This indication is also provided to target gNB over XnAP.

From gNB to SMF direction: reuse existing PDU Set based Handling Indicator IE.

For support alternative PDU Set QoS:

Add UL PSDB, DL PSDB, UL PSER and DL PSER in Alternative QoS Parameter Set List IE in NGAP/XnAP/F1AP/E1AP.

Add new codepoints “not fulfilled DL” and “not fulfilled UL” in the Notification information/cause IE in NGAP/XnAP/F1AP.

For QoS Handling when Traffic Characteristics Change Dynamically:

Introduce BSSize and TTNB in NG-U/F1-U/XN-U.

For Support of exposure of available data rate:

Introduce thresholds and direction to trigger NG-RAN monitor available data rate in NGAP/XnAP/F1AP/E1AP.

Introduce “available data rate” in TS 38.425 and TS 38.415.

For Support and specify multi-modality awareness for QoS flows in both DL and UL RAN:

Introduce MMSID in NGAP, XnAP, F1AP and E1AP.

For Specify uplink congestion signalling:

Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

Introduce the same indication in XnAP to inform target gNB.

Continue the discussion on UL rate control.

FFS on specific container/frame to be used for Traffic Characteristics Change Dynamically. FFS on impact to TS 38.420/470.

FFS on non-homogeneous deployment for support of exposure of available data rate.

FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows.

FFS on whether the gNB-CU sends an indication to the gNB-DU to stup the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

Work on stage2 and stage3 details and open issues above…

In this contribution, we propose several signalling enhancements for above XR objectives by taking RAN3#127bis agreements into account.

# UL Rate Control discussion

RAN3#127bis discussed about UL Rate Control mechanism for option 1 and option2 without any conclusion.

Option 1: MAC-CE will provide per QoS flow level uplink rate control information to UE

Option 2: RRC will provide ratio information configuration to UE and MAC-CE will provide per DRB level uplink rate control information.

RAN2#129bis had below agreement

- From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this.

- FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

Based on RAN2 LS to both SA4 and SA2, SA4 briefly discussed and LS was just noted without any further discussion. SA2 sent reply LS S2-2504252 to RAN2 with following content:

- RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2.

- SA2 Feedback:

- SA2 confirms that the CN can indicate to the gNB that a QoS flow is subject to uplink rate control.

- SA2 would like to inform RAN WGs that SA2 has not decided yet on whether the existing or a new mechanism can be reused.

- SA2 would like RAN2 to confirm that the RAN2 LS on UL rate control applies to both GBR and non-GBR.

RAN3#127bis :

FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows.

In order for gNB to provide any UL recommended bit rate control information to UE via MAC-CE, whether it is for option 1 or option 2 based solution, first RAN should be aware what is XR OTT application data rate that can be used for specific XR application. Since all these XR applications are OTT based and without knowing possible XR application codec rates to be used, RAN will not be able to determine what UL recommended data rate values can be sent to UE for UL XR application rate adaptation purpose. During PDU session resource setup procedure, 5GC has to provide for a given specific QoS flow what are applicable data rates that can used. 5GC has to receive this information from XR application server.

Thus we propose:

- For RAN based UL Rate Control feature, RAN3 agrees that 5GC/SMF will provide possible UL recommended data rate values to be used for specific QoS low level.

Next key question is whether to support Option 1 vs Option 2.

RAN3#127bis: Continue the discussion on UL rate control.

For both options, from RAN3 point of view, there will be F1-AP specification impact and there will be RAN2 impacts for  MAC-CE enhancements. If Option2 is used, then we can just reuse existing per DRB level congestion management in DU and from UE perspective, we can reuse same per DRB level MAC-CE for Recommended Bit Rate values in UL. In order to simplify this discussion, probably we can consider Option 2 as baseline. Strictly speaking before RAN2 and RAN3 conclude about which option to be supported, SA4 and SA2 also need to conclude that AS and 5GC will be able to provide per QoS flow level and/or rate control ratio information can be provided from 5GC to RAN. From simplicity point of view, we are ok to support option2 even though RAN2 indicated preference for option 1.

- For UL rate control, support Option 2 solution because it enables to reuse legacy per DRB level MAC-CE for UL rate control and DU can reuse R18 per DRB level rate control information.

# PDU Set Information Marking without PDU Set QoS

From SA2 agreed CR S2-2502672:

If no DL PDU Set QoS parameters are included in the PCC rule, but a DL Protocol Description is included in the PCC rule received by the SMF and the PSA UPF supports PDU Set Handling (i.e. supports to perform PDU Set Information marking), the SMF may, based on the local operator policy, send to the NG-RAN, a DL PDU Set Information Marking Support Indication indicating that the DL PDU Set Information marking is supported.

If the DL PDU Set Information Marking Support Indication has been sent to the 5G-AN (as described in clause 5.37.5.1) and the 5G-AN has provided a PDU Set based handling support Indication indicating that PDU Set handling is supported, the SMF instructs PSA UPF to perform PDU Set marking and provides the PSA UPF the DL Protocol Description used by the service data flow.

5.37.5.3	Non-homogenous support of PDU set based handling in NG-RAN

The SMF, by sending PDU Set QoS parameters (as described in clause 5.7.7.1) or a DL PDU Set Information Marking Support Indication (as described in clause 5.37.5.1) to the NG-RAN, requests the NG-RAN to activate PDU Set handling for a given QoS Flow and the NG-RAN provides the SMF with a PDU Set Based Handling Support Indication if the PDU Set based handling is supported. Based on this indication, SMF may activate the PDU Set identification and marking in the PSA UPF as described in 5.37.5.2.

During mobility procedures that result in the change of NG-RAN, the target NG-RAN provides to the SMF a PDU Set Based Handling Support Indication if it supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the target NG-RAN indication, the SMF may, upon completion of the mobility procedure, initiate the PDU Session modification procedure to provide PDU Set QoS parameters or the DL PDU Set Information Marking Support Indication to NG-RAN and may configure the PSA UPF to activate the PDU Set identification and marking. If the PDU Set Based Handling Support Indication is not received from the target NG-RAN and PDU Set identification and marking is active in the PSA UPF, the SMF may deactivate it.

In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header, it shall start doing so from a complete PDU Set.

NOTE:	Based on the PDU Set Based Handling Support Indication from the target NG-RAN, the SMF can update the QoS profile and, if applicable, Alternative QoS Profiles as defined in clauses 5.7.1.2 and 5.7.1.2a to include the PDU Set QoS parameters respectively.

RAN3#127bis Agreement:

For support of DL PDU Set marking without PDU Set QoS:

From SMF to gNB direction: introduce DL PDU Set Information Marking Support Indication IE in the QoS Flow Level QoS Parameters IE. This indication is also provided to target gNB over XnAP.

From gNB to SMF direction: reuse existing PDU Set based Handling Indicator IE.

But RAN3 has not discussed about support for non-homogeneous deployment scenario and dual connectivity as well.

- RAN3 agree to support Xn-AP and NG-AP signalling enhancements to enable support for DL PDU Set Information marking feature for non-homogeneous deployments and dual connectivity scenarios.

# Reporting of available data rate value for GBR QoS Flow

From SA2 agreed CR [4]

5.45.x	Available bitrate monitoring

The QoS Monitoring for available bitrate allows the measurement and report of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles at the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation.

NOTE 1:	How the NG-RAN measures the available bitrate is NG-RAN implementation specific.

NOTE 2:	The available bitrate can be between the GFBR and the MFBR. The available bitrate can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles.

For the reporting of the available bitrate, periodic reporting is not applied and only threshold-based reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold to the PSA UPF via the GTP-U header of UL packets.

NOTE 3:	The NG-RAN is expected to immediately report the UL and/or DL available bitrate when it matches or exceeds a lower threshold value and report the UL and/or DL available bitrate when it matches or exceeds a higher threshold value with some hysteresis.

The SMF may configure the PSA UPF to perform QoS monitoring for available bitrate as described in clause 5.8.2.18. For the reporting of the available bitrate via the PSA UPF, only event-based reporting applies in the PSA UPF. The Reporting threshold(s) in the PCC rule are not sent to UPF. In UPF, the event for the reporting is the reception of available bitrate information from NG-RAN. According to the QoS Monitoring request for the available bitrate from the SMF, the PSA UPF monitors and exposes the available bitrate information reported from NG-RAN to the target NF as instructed.

During handover, the QoS Monitoring configuration is provided to the Target NG-RAN per QoS Flow in the QoS Monitoring configuration.

5.37.4	Network Exposure of 5GS information

The UL and/or DL available bitrate for a GBR QoS Flow (see clause 5.45.x).

For a GBR QoS Flow, based on the PCC rule from PCF, the SMF requests the NG-RAN to report the UL and/or DL available bitrate. The NG-RAN reports the UL and/or DL available bitrate information to the PSA UPF via the GTP-U header of UL packets.

The SMF further requests the PSA UPF to expose the available bitrate to the AF directly or via local NEF, as described in clause 5.8.2.18.

RAN3#127bis agreed following signalling enhancements:

For Support of exposure of available data rate:

Introduce thresholds and direction to trigger NG-RAN monitor available data rate in NGAP/XnAP/F1AP/E1AP.

Introduce “available data rate” in TS 38.425 and TS 38.415.

FFS on non-homogeneous deployment for support of exposure of available data rate.

In case of non-homogeneous deployment , we need to consider following cases:

Source gNB is supporting but target gNB is not supporting

Target gNB is supporting but source gNB is not supporting.

In order to support these non-homogeneous deployment scenarios, RAN3 need to specify Xn-AP, NG-AP signaling enhancements to be consistent with other XR features like PDU set handling support in non-homogeneous deployments.

- Support Xn and NG-AP signalling enhancements to enable non-homogeneous deployment of XR Available Data Rate Reporting mechanism.

# Enhancements to support unnecessary RLC Re-transmissions avoidance

In RAN3#127bis, Lenovo paper R3-251955 brough the following discussion and is valid point to discuss.

RAN2#129 Agreements on RLC enhancements Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

.RAN3#127bis:

FFS on whether the gNB-CU sends an indication to the gNB-DU to stup the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

Based on above RAN2 agreements, here is the expected example sequence of RLC operation:

- RLC TX sent 0 to 10, deliberately dropped 11 based on PDCP request, sent 12 to 20.

- RLC RX AM started "New timer" for the hole 11, which is typically larger than Treassembly. Treassembly based expiry will trigger RLC AM Status PDU with NAK SN 10.

- While RLC RX waits for PDU retransmission from RLC TX, If the "New timer" expires, RLC RX "assumes" successful reception of this SN and moves forward with the remaining PDUs in the window.

- Key point is, this "New RLC timer at Rx side" should be big enough to give at least one chance for reTx "based on Treassembly based NAK from Rx" or "based on autonomous transmission from Tx" perspective.

For downlink, the PDCP is located in the gNB-CU while the TX RLC entity is located in gNB-DU. To support RAN2’s agreement, F1-U user plane coordination between the gNB-CU-UP (PDCP) and the gNB-DU (RLC) is needed. The gNB-CU-UP PDCP needs to send a discard indication of a PDCP SDU to the gNB-DU so that the gNB-DU can stop the transmission and retransmission of the SDU or the segment of the SDU. When the PDCP discard timer expires for a PDCP SDU, the gNB-CU-CP sends the indication to the gNB-DU to stop the transmission and retransmission of the RLC SDU or the segment of the RLC SDU which corresponds to the specific PDCP SDU. This is valid issue and need to be addressed by RAN3 from DL perspective in split-gNB Architecture. Thus, we propose

- To support avoidance of unnecessary RLC retransmission in DL, the gNB-CU-UP PDCP sends an indication to the gNB-DU RLC entity to stop the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

# Summary

Based on above discussion, we have the following proposals for XR enhancements.

Proposal 1.	For RAN based UL Rate Control feature, RAN3 agrees that 5GC/SMF will provide possible UL recommended data rate values to be used for specific QoS low level.

Proposal 2.	For UL rate control, support Option 2 solution because it enables to reuse legacy per DRB level MAC-CE for UL rate control and DU can reuse R18 per DRB level rate control information.

Proposal 3.	RAN3 agree to support Xn-AP and NG-AP signalling enhancements to enable support for DL PDU Set Information marking feature for non-homogeneous deployments and dual connectivity scenarios.

Proposal 4.	Support Xn and NG-AP signalling enhancements to enable non-homogeneous deployment of XR Available Data Rate Reporting mechanism.

Proposal 5.	To support avoidance of unnecessary RLC retransmission in DL, the gNB-CU-UP PDCP sends an indication to the gNB-DU RLC entity to stop the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

# References

[1] RP-250107: Revised WID on XR (eXtended Reality) for NR Phase 3

[2] TS 23.501: System architecture for the 5G System (5GS); Stage 2

[3] TR 23.700-070: Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2

[4] S2-2502675: KI#9, Support of Available Data Rate