3GPP TSG-RAN WG3 Meeting #128	R3-253575

Malta, MT, 19 – 23 May 2025

Agenda Item:	12.3

Source: 				LG Electronics Inc.

Title:		 Security verification related to NR Femto

Document for:		Discussion and Decision

1.	Introduction

RAN3 sent a LS [1] to SA3 to clarify the security verifications for the NR Femto GW (and AMF, respectively). For this RAN3 LS, SA3 confirmed in [2] that all the verifications listed in the LS need to be performed in NR Femto GW (or AMF if NR Femto GW is not deployed), and TS 33.545 is updated to incorporate relevant verification requirements and mechanisms. Based on the SA3 Reply LS, we will further discuss RAN3 specification impacts on the security verifications for the NR Femto GW (and AMF, respectively), and provide our views on it.

2.	Discussion

In RAN3#127 meeting, the LS [1] was sent to SA3 to ask whether the security verifications need to be specified for the NR Femto GW (and AMF, respectively) as in the LTE HeNB.

LS to SA3 [1]: … RAN3 would like SA3 feedback on whether the following verifications need to be specified for the NR Femto GW (and AMF, respectively), as outlined below: For the NR Femto Gateway: 1: In case of NGAP INITIAL UE MESSAGE message, whether to verify that reported CAG ID is valid for that NR Femto. 2: In case of NGAP SETUP REQUEST message, whether to verify that the identity used by the NR Femto is valid. 3: In case of NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message, whether to verify that the indicated cell identity is valid. For the AMF: 1: In case of an NR Femto directly connected, whether to verify that the CAG ID reported in the NGAP INITIAL UE MESSAGE message is valid for the indicated cell. 2: In case of an NR Femto directly connected, whether to verify that the identity used by the NR Femto is valid when receiving the NGAP SETUP REQUEST message. 3: In case of an NR Femto directly connected, whether to verify that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NGAP PWS FAILURE INDICATION message. …

In the Reply LS [2], SA3 confirmed that all the verifications listed in the LS need to be performed in NR Femto GW (or AMF if NR Femto GW is not deployed). It was also agreed to update TS 33.545 in [3][4].

Reply LS from SA3 [2]: … SA3 kindly asks RAN3 to consider following answers: SA3 has agreed that all the verifications listed in the LS need to be performed in NR Femto GW (or AMF if NR Femto GW is not deployed). SA3 agreed the attached CRs to update TS 33.545 to incorporate relevant verification requirements and mechanisms as well. …

In the current BL CR to TS 38.300 [5], the aspects on the security verifications for the NR Femto GW (and AMF, respectively) is still missing. Based on the Reply LS [2] from SA3, therefore, it is needed to capture the security verifications for the NR Femto GW (and AMF, respectively) into the BL CR to to TS 38.300 [5].

Proposal 1: It is proposed to capture the security verifications for the NR Femto GW (and AMF, respectively) into the BL CR to to TS 38.300.

According to the SA3 CR [3], it is clearly described that the security verifications for the UE-associated procedure are performed in case of the UE-associated messages from the NR Femto in the closed access mode. Therefore, in case of NGAP INITIAL UE MESSAGE message, the NR Femto GW (and AMF, respectively) should verify that the reported CAG ID is valid for that NR Femto in the closed access mode. However, in RAN3#126 meeting, RAN3 decided to avoid mentioning “open”, “closed”, and “hybrid” access mode in NOTE and introducing such definitions for the access mode in NR Femto.

CR (S3-251699) for TS 33.545 [3]: … ## 5.X CAG ID verification The NR Femto GW shall verify that all UE associated messages from Femtos operating in closed access mode can be mapped to a specific CAG ID and that this CAG ID is allowed for the identity of the originating Femto. In the absence of a NR Femto GW, the AMF shall perform this CAG ID verification. One NR Femto can host multiple Femto cells. Hence, the CAG requirements should be considered mapping to individual Femto cell. NOTE 1: The CAG ID being verified may be explicitly present in the message as an information element or may be mapped by other means. NOTE 2: The above requirement implies that the network has to ensure that all UE-associated messages from Femtos operating in closed access mode are subject to the above verification, even if a (rogue) message from a compromised Femto would not indicate to originate from a NR Femto. …

Observation 1: RAN3 agreed to avoid mentioning “open”, “closed”, and “hybrid” access mode in NOTE and introducing such definitions for the access mode in NR Femto.

In the current BL CR to TS 38.300, the closed access mode related descriptions are captured as follows:

- The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

Therefore, we think that “a NR Femto activating an CAG-only cell” can be used instead of “a closed NR Femto” in the corresponding TP.

Proposal 2: It is proposed to use the terminologies “a NR Femto activating an CAG-only cell” instead of “a closed NR Femto” in the corresponding TP.

According to the SA3 CR [4], it was captured in TS 33.545 that the Security Requirements and Principles defined in clause 4.4 of TS 33.320 are reused with some modifications for the NR Femto. Therefore, it is required to refer to TS 33.545 instead of TS 33.320.

Proposal 3: It is proposed to refer to TS 33.545 for the security verifications for the NR Femto GW (and AMF, respectively).

Therefore, we propose to agree the Relay LS to SA3 in Appendix and the corresponding TP in [6].

Proposal 4: It is proposed to agree the Relay LS to SA3 in Appendix and the corresponding TP in [6].

3.	Conclusion

In this contribution, we focused on RAN3 specification impacts on the security verifications for the NR Femto GW (and AMF, respectively) based on Reply LS [2] from SA3, and then provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: It is proposed to capture the security verifications for the NR Femto GW (and AMF, respectively) into the BL CR to to TS 38.300.

Proposal 2: It is proposed to use the terminologies “a NR Femto activating an CAG-only cell” instead of “a closed NR Femto” in the corresponding TP.

Proposal 3: It is proposed to refer to TS 33.545 for the security verifications for the NR Femto GW (and AMF, respectively).

Proposal 4: It is proposed to agree the Relay LS to SA3 in Appendix and the corresponding TP in [6].

4.	Reference

- R3-250822, “LS on security verification related to NR Femtos”, RAN3 (ZTE), February, 2025.

- R3-253021/S3-251668, “Reply LS on security verification related to NR Femtos,” SA3 (ZTE), May, 2025.

- S3-251699, “Update NR Femto Gateway Functionality”, Nokia, April, 2025.

- S3-251787, “New clauses on security requirement and principles”, Huawei, Hisilicon, April, 2025.

- R3-253076, “(BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects”, Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, May, 2025.

- R3-253576, “(TP to TS 38.300) Support of security verification in NR Femto”, LG Electronics, May, 2025.

5.	Appendix: Reply LS for S3-251668 [2]

This appendix provides the draft Reply LS to SA3 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #128 R3-25xxxx Malta, MT, 19th – 23rd May 2025 # Title: [Draft] Reply LS on security verification related to NR Femtos # Response to: R3-253021/S3-251668 # Release: Rel-19 # Work Item: NR_WAB_5GFemto-Core Source: RAN3 To: SA3 Cc: SA2 Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: R3-253576 # 1 Overall description RAN3 thanks SA3 for the Reply LS on security verification related to NR Femtos. RAN3 agreed to capture the security verifications for the NR Femto GW (and AMF, respectively) in TS 38.300. The TP to the 38.300 BL CR (R3-253576) capturing the above RAN3 agreement is provided in the attachment. In addition, RAN3 notices that clause 5.X of TS 33.545 (S3-251699) states the behaviour of the NR Femto operating in closed access mode. However, RAN3 agreed to avoid mentioning “open”, “closed”, and “hybrid” access mode and introducing such definitions for the access mode of the NR Femto in TS 38.300. 2. Action: To SA3: ACTION: RAN3 kindly asks SA3 to take the above into account. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#129 25 – 29 August 2025 Bangalore, India 3GPP TSG RAN3#129bis 13 – 17 October 2025 Prague, CZ

-----------------End of the First Change---------------