3GPP TSG-RAN WG3 Meeting #128	R3-253586

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	11.4

Source:	Nokia, Jio Platforms (JPL)

Title:	(TP for BLCR to TS 38.425) Procedures for Measuring UE Performance

Document for:	Text Proposal

# 1	Introduction

In the last RAN3 meeting we achieved good progress in the topic of split architecture support for Rel-18 use cases.

Specifically, for E1AP we made the following agreements:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

Send the LS to SA5 to seek the clarification on UE throughput UL/DL at PDCP level and UL/DL Packet Loss Rate at PDCP layer.

WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

In addition, for F1AP, the following was agreed related to the packet delay measurement, replacing an earlier agreement that packet delay measured for UE Performance feedback is sent via DDDS in case of split architecture:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

Two open points were captured, namely:

Defer the discussion on packet loss rate in the split architecture in the next meeting.

For Packet delay, whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, UL, DL, and periodicity?

In RAN3 #127 we had also made an agreement related to measuring and reporting of Average Packet Delay for UE Performance Feedback in split architecture:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

In other words, we agreed to re-use to the extent possible the QoS Monitoring framework introduced in Rel-16 for the purpose of reporting of Average Packet Delay from gNB-CU-UP to gNB-CU-CP. In this paper we provide a short overview of the Rel-16 QoS Monitoring Framework. We also provide our views on the needed modifications to this framework to report UE Performance Delay from gNB-CU-UP to gNB-CU-CP.

Furthermore, we provide a TP for BLCR to TS 38.425 in Annex A of this paper.

# 2	Discussion

## 2.1		Average Packet Delay Reporting

### 2.1.1	Overview of Rel-16 QoS Monitoring Framework

The QoS Monitoring Framework is defined in TS 23.501 clause 5.33.3. In this framework, the SMF may trigger QoS monitoring for specific QoS flow(s). This is done as part of e.g. setting up the PDU session context during the Initial Context Setup procedure. The NGAP QoS Flow Level QoS Parameters IE includes a QoS Monitoring Request IE and QoS Monitoring Reporting Frequency IE where the frequency of reporting for the RAN part of delay is a value in the range from 1 to 1800 seconds. NG-RAN reports the RAN part of UL/DL packet delay result to the PSA UPF in the UL data packet or in a dummy UL packet.

In case of split architecture, gNB-CU-CP configures QoS Monitoring in gNB-CU-UP and in gNB-DU. While the configuration towards gNB-CU-UP configures both a monitoring direction (UL, DL, both) and a QoS Monitoring Reporting Frequency in per-DRB granularity, the configuration towards the DU defines only a monitoring direction (UL, DL, both) in per-DRB granularity. This was decided by RAN3 in order to allow a gNB-DU to report delay measurements to gNB-CU-UP based on a configured or implementation specific frequency. (For further details, see LS exchange between RAN3 and SA2, c.f. e.g. discussion initiated at RAN3 #110 R3-207156).

The delay components calculated at gNB-DU are reported through DDDS and Assistance Information to the gNB-CU-UP over F1-U. For F1-U delay measurement (done in the CU-UP), the gNB-DU reports its internal feedback delay in report polling triggered DDDS. Other delay components measured in the gNB-DU are accumulated in the gNB-DU (UL, DL) and provided in Assistance Information. The gNB-DU starts the measurement and reports it to the gNB-CU-UP as indicated in TS 38.425: “The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.” As a result, the reporting type (event-triggered or periodic reporting) and reporting frequency from the gNB-DU are based on OAM configuration or implementation. The gNB-CU-UP calculates the total delay result by combining gNB-DU Delay Result together with its own delay result and reports it to the PSA UPF in an UL data packet or in a dummy UL packet.

The following delay components are handled by the QoS monitoring framework (see TS 38.314 and TS 28.558 for details).

UL average packet delay is the sum of the components D1+ D2.1+ D2.2+ D2.3+ D2.4:

- D1 (UL PDCP packet average delay): This is the average PDCP delay in the UL. It is defined in TS 38.314, clause 4.3.1.1. It is a measurement performed by the UE to measure UL buffering delay in the PDCP layer, i.e. the average time a packet is buffered in the UE PDCP layer before it is transmitted. This measurement is obtained according to the definition in clause 4.3.1.1 of TS 38.314, named "UL PDCP Packet Average Delay per DRB per UE".

- D2.1 (average over-the-air interface packet delay): This is the air interface UL delay. It is defined in TS 38.314, clause 4.2.1.2.2 (and in TS 28.558, clause 6.3.1.1.6). It is measured at the gNB-DU.

- D2.2 (average RLC packet delay): It measures RLC packet delay in the UL. It is defined in TS 38.314, clause 4.2.1.2.3 (and in TS 28.558, clause 6.3.1.1.7). It is measured at the gNB-DU.

- D2.3 (average delay UL on F1-U): This is the average GTP packet delay UL on the F1-U interface. It is measured using the same metric as the average delay DL on F1-U defined in TS 28.558, clause 6.3.1.1.3. It is measured by the gNB-CU-UP.

- D2.4 (average PDCP re-ordering delay in the UL): It measures PDCP re-ordering delay in the UL. It is defined in TS 38.314, clause 4.2.1.2.4 (and in TS 28.558, clause 6.3.1.1.8). It is measured at the gNB-CU-UP.

The average packet delay in the DL is the sum of the components D1+ D2+ D3+ D4:

- D1 (DL delay in over-the-air interface): This is the Average delay in the DL, providing the average time it takes for packet transmission over the air interface. It is measured at the gNB-DU. It is described in TS 28.558, clause 6.3.1.1.1.

- D2 (DL delay on gNB-DU): This is the average RLC SDU delay in the DL within the gNB-DU, for initial transmission of all RLC packets. It is measured at the gNB-DU. It is described in TS 28.558, clause 6.3.1.1.2.

- D3 (DL delay on F1-U): This is the average GTP packet delay DL on the F1-U interface. It is measured at the gNB-CU-UP. It is described in TS 28.558, clause 6.3.1.1.3.

- D4 (DL delay in CU-UP): This is the average PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. It is measured at the gNB-CU-UP. It is described in TS 28.558, clause 6.3.1.1.4.

### 2.1.2	Using QoS Monitoring for UE Performance Delay Reporting

In RAN3 #127, we agreed that the Packet Delay measured for UE Performance Feedback is sent from gNB-CU-UP to the gNB-CU-CP in the case of split architecture. With this agreement, the collection of UE Performance Delay at gNB-CU-UP shares some similarities with the QoS Monitoring framework. Even though QoS Monitoring is triggered by the Core Network, eventually it will be the gNB-CU-CP that needs to configure gNB-CU-UP and gNB-DU to measure and aggregate the overall RAN delay result at the gNB-CU-UP.

Observation 1: Under both QoS Monitoring framework and delay UE Performance support in split architecture the gNB-CU-CP configures gNB-DU to measure RAN delay components and to report the measurements to CU-UP for aggregation.

Finally, in QoS Monitoring framework a gNB-DU reports delay measurements to gNB-CU-UP based on a configured or implementation specific frequency. This is an aspect that needs to be revisited in the context of reporting of UE Performance components from gNB-DU to gNB-CU-UP since it could risk inconsistent aggregation of gNB-DU delay components at gNB-CU-UP if the reporting frequency at the gNB-DU is left to implementation.

Observation 2: The assumption that gNB-DU reports delay measurements to gNB-CU-UP based on a configured or implementation specific frequency needs to be revisited in the context of aggregating delay components from the gNB-DU at the gNB-CU-UP for UE Performance reporting to enable consistent delay component aggregation.

In the subsequent sections, we discuss UE Performance delay support in split architecture using QoS Monitoring framework as a baseline.

### 2.1.3			Configuration of measurement collection and reporting at gNB-DU

UE Performance data collection over Xn requires the measurements to be collected for a specific data collection duration, such that when the “data collection duration” expires, the measurement collection should be stopped. In the current F1AP, the packet delay measurement collection is based on a start-stop trigger using QoS Monitoring Request IE.  For the UE Performance data collection trigger, the target gNB-CU-CP is aware of the data collection duration, e.g. as received from the source gNB in the case of Xn handover scenario. The (target) gNB-CU-CP can start the measurement collection in the (target) gNB-DU as part of the UE Context Setup procedure initiated from the (target) gNB-CU-CP upon receipt of the Handover Request procedure. Once the data collection duration timer expires, the (target) gNB-CU-CP can stop the measurement collection in gNB-DU. This framework is sufficient even when multiple triggers for the Average Packet Delay measurement occur simultaneously, because the gNB-CU-CP will have full control of start and stop of the measurement collection.

Observation 3: The start/stop mechanism over F1 is sufficient and there is no need to introduce Collection Time Duration for the collection of Average Delay components by the gNB-DU.

To support a cleaner solution, we propose to decouple triggering of QoS Monitoring towards the gNB-DU from the triggering of Average Packet Delay measuring and reporting from the gNB-DU to the gNB-CU-UP. Note also that in F1AP, QoS Monitoring is triggered in the gNB-DU through the inclusion of the QoS Monitoring Request IE that has the following structure:

We notice that QoS Monitoring can be requested separately in the UL and DL directions. However, we have defined the Average Packet Delay IE in XnAP as follows, with UL and DL delay components being made mandatory:

Therefore, for the purpose of the UE Performance reporting we think that we only need to consider the codepoint allowing both UL and DL Average Packet Delay components to be configured to the gNB-DU (serving to “start” the measurement) as well as the codepoint to “stop” the measurement.

Proposal 1: Introduce a new IE, Performance Delay Monitoring Request IE, to start the UL and DL delay measurement for the DRB at the gNB-DU and to stop the measurement, encoded as an ENUMERATED (UL and DL, stop,…).

We further discuss whether there is a need to introduce Reporting Periodicity over F1AP. There are two options:

- Option 1: Leverage the existing F1AP framework without Reporting Periodicity.

- Option 2: Introduce Reporting Periodicity over F1AP

As discussed in section 2.1, option 1 is implementation-dependent. This may lack flexibility in terms of signalling-based configuration and could e.g. lead implementations to report delays over F1-U more frequently than required. Through OAM configuration, gNB-CU-UP can be made aware of the configuration value at gNB-DU and hence it will be possible for gNB-CU-UP to identify if it can support the requested periodicity or whether it needs to apply special handling when it receives a request for UE Performance from gNB-CU-CP (e.g. to skip some measurements). RAN3 should therefore discuss whether introduction of signalled reporting periodicity in control of gNB-DU reporting is needed. However, our preference would be to introduce this parameter.

Proposal 2: Introduce reporting periodicity over F1 through a new Performance Delay Monitoring Reporting Periodicity IE that has the same periodicity as the Reporting Periodicity for Data Collection IE in XnAP, namely is encoded as an  ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …).

We think that we need to extend UE Context Setup and UE Context Modification procedures to include an indication to the gNB-DU to control (start, stop) UE Performance monitoring reporting for UL and DL Average Packet Delay together with an indication on the reporting periodicity.

Proposal 3: Enhance F1AP UE CONTEXT SETUP and UE CONTEXT MODIFICATION REQUEST messages to include an indication to the gNB-DU to control (start/stop) UE Performance monitoring reporting for UL and DL Average Packet Delay together with an indication on the reporting periodicity.

Figure 1 illustrates an example of leveraging QoS Monitoring framework for the purpose of Average Delay Reporting from gNB-DU to gNB-CU-UP, allowing the latter to aggregate the RAN delay components (except for the UL Delay Component measured by the UE which is reported directly to the gNB-CU-CP).

Figure 1 Leveraging QoS Monitoring framework for UE performance monitoring in split architecture.

### 2.1.4			Configuration of measurement collection and reporting by the gNB-CU-UP to the gNB-CU-CP

In the last meeting, we agreed to introduce new procedures over E1 interface, namely the Data Collection Reporting Initiation and the Data Collection Reporting procedures, to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request. When it comes to the granularity of UL/DL Average Delay from gNB-CU-UP to gNB-CU-CP we agreed that Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level. Based on these agreements, RAN3 is ready to work on a stage 3 TP for signalling of configuration and reporting of average packet delay measurements over E1.

Proposal 4: Enhance the E1AP DATA COLLECTION UPDATE message to support reporting of average packet delay measurements on a per DRB level.

Since the Data Collection Procedures are non UE-associated, similarly to XnAP, the UE for which UE Performance is requested needs to be indicated from the gNB-CU-CP to the gNB-CU-UP. Therefore, we propose to enhance the E1AP Bearer Context Setup and Bearer Context Modification procedures to include the Data Collection ID providing the context for this data collection.

Proposal 5: Enhance E1AP BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION REQUEST messages to include the Data Collection ID of the Data Collection Reporting Initiation procedure that configured UE Performance measurements to the gNB-CU-UP.

Similarly to the case of F1AP, we propose to introduce an IE in E1AP to control (start, stop) UL and DL delay measurements for UE Performance reporting at gNB-CU-UP. As also for the F1AP case, we think that we only need to consider the codepoint allowing both UL and DL Average Packet Delay components to be configured to the gNB-CU-UP (serving to “start” the measurement) as well as the codepoint to “stop” the measurement.

Proposal 6: Enhance E1AP BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION REQUEST messages with a new IE, Performance Delay Monitoring Request IE, to start the UL and DL delay measurement for the DRB at the gNB-CU-UP and to stop the measurement, encoded as an ENUMERATED (UL and DL, stop, …).

## 2.2		Average Throughput UL/DL Reporting

In the last meeting, we agreed to use Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 to transfer UL/DL UE throughput from gNB-CU-UP to gNB-CU-CP. We also took a WA namely:

WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

We took this working assumption since it seemed to RAN3 that it is feasible to calculate UE Throughput UL/DL at PDCP layer. RAN3 sent an LS about this to SA5 in the previous meeting to ask if the existing DL UE Buffered Throughput per DRB defined in TS 28.552 at PDCP level can be used as a DL UE Throughput measurement. In addition, RAN3 thinks that it is feasible to define a similar metric in the UL related to the received PDCP PDU volume over a certain period and asked SA5 to define such measurement, if feasibility is confirmed. Turning this WA into agreement is pending reply to SA5. For the sake of progress with the RAN3 related signalling procedures we can work on stage 3 TP including an editor’s note with FFS.

Proposal 7: Include UE throughput UL/DL in E1AP stage 3 TP at this meeting, including editor’s note with FFS pending reply from SA5.

## 2.3		Average Packet Loss DL

On this UE Performance metric, RAN3 does not have a clear understanding whether it can be measured at PDCP layer. Therefore, we propose to wait for the reply LS from SA5 on this aspect. After feedback is provided by SA5, RAN3 could consider the way forward.

Proposal 8: RAN3 shall wait for the reply from SA5 regarding the feasibility of measuring Packet Loss Rate in UL and DL at PDCP layer.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: Under both QoS Monitoring framework and delay UE Performance support in split architecture the gNB-CU-CP configures gNB-DU to measure RAN delay components and to report the measurements to CU-UP for aggregation.

Observation 2: The assumption that gNB-DU reports delay measurements to gNB-CU-UP based on a configured or implementation specific frequency needs to be revisited in the context of aggregating delay components from the gNB-DU at the gNB-CU-UP for UE Performance reporting to enable consistent delay component aggregation.

Observation 3: The start/stop mechanism over F1 is sufficient and there is no need to introduce Collection Time Duration for the collection of Average Delay components by the gNB-DU.

Proposal 1: Introduce a new IE, Performance Delay Monitoring Request IE, to start the UL and DL delay measurement for the DRB at the gNB-DU and to stop the measurement, encoded as an ENUMERATED (UL and DL, stop,…).

Proposal 2: Introduce reporting periodicity over F1 through a new Performance Delay Monitoring Reporting Periodicity IE that has the same periodicity as the Reporting Periodicity for Data Collection IE in XnAP, namely is encoded as an  ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …).

Proposal 3: Enhance F1AP UE CONTEXT SETUP and UE CONTEXT MODIFICATION REQUEST messages to include an indication to the gNB-DU to control (start/stop) UE Performance monitoring reporting for UL and DL Average Packet Delay together with an indication on the reporting periodicity.

Proposal 4: Enhance the E1AP DATA COLLECTION UPDATE message to support reporting of average packet delay measurements on a per DRB level.

Proposal 5: Enhance E1AP BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION REQUEST messages to include the Data Collection ID of the Data Collection Reporting Initiation procedure that configured UE Performance measurements to the gNB-CU-UP.

Proposal 6: Enhance E1AP BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION REQUEST messages with a new IE, Performance Delay Monitoring Request IE, to start the UL and DL delay measurement for the DRB at the gNB-CU-UP and to stop the measurement, encoded as an ENUMERATED (UL and DL, stop,…).

Proposal 7: Include UE throughput UL/DL in E1AP stage 3 TP at this meeting, including editor’s note with FFS pending reply from SA5.

Proposal 8: RAN3 shall wait for the reply from SA5 regarding the feasibility of measuring Packet Loss Rate in UL and DL at PDCP layer.

Also, in the annex of this paper we provide a TP to TS 38.425 to reflect the agreement:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

# Annex  – TP to BLCR to TS 38.425

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[3]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[4]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[5]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[6]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 23.501: "System Architecture for the 5G System".

[9]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture description".

[x]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

## 5.2	NR user plane protocol layer services

NOTE 1:	In this section, NR user plane protocol layer services are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

The following functions are provided by the NR user plane protocol:

-	Provision of NR user plane specific sequence number information for user data transferred from the node hosting NR PDCP to the corresponding node for a specific data radio bearer.

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for user data associated with a specific data radio bearer.

-	Information of NR PDCP PDUs that were not delivered to the UE or not transmitted to the lower layers.

-	Information of NR PDCP PDUs transmitted to the lower layers for user data associated with a specific data radio bearer.

-	Information of downlink NR PDCP PDUs to be discarded for user data associated with a specific data radio bearer;

-	Information of the currently desired buffer size at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer.

-	Information of the currently desired data rate in bytes at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer;

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for retransmission user data associate with a specific data radio bearer;

-	Information of NR PDCP PDUs transmitted to the lower layers for retransmission user data associated with a specific data radio bearer.

-	Information of the specific events at the corresponding node.

-	Information on Radio Link Quality from the corresponding node for user data associated with a specific data radio bearer.

-	Information for QoS monitoring or UE performance monitoring from the corresponding node for user data associated with a specific data radio bearer.

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

### 5.4.3	Transfer of Assistance Information

#### 5.4.3.1	Successful operation

NOTE 1:	In this section, PDCP duplication and delay measurement related information are not applicable to E-UTRA PDCP.

The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures.

An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only.

The Transfer of Assistance Information procedure may be invoked if

-	the corresponding node decides to send the Radio Quality Assistance Information and/or the PDCP duplication activation suggestion to the node hosting the NR PDCP entity for the concerned data radio bearer or,

-	the corresponding node decides to send the Radio Quality Assistance Information to the node hosting the NR PDCP entity for the concerned RLC entity.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring or UE performance monitoring and to send the monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The ASSISTANCE INFORMATION DATA frame may include one or more Radio Quality Assistance Information. The information shall consist of the information indicated in the Assistance Information Type.

The ASSISTANCE INFORMATION DATA shall be sent, if supported, when the corresponding node receives a DL USER DATA PDU including the Assistance Information Report Polling Flag set to 1.

The ASSISTANCE INFORMATION DATA frame may include the PDCP Duplication Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or not activate DL PDCP duplication. The node hosting the NR PDCP entity may take this information into account to take a decision on whether to activate or not activate PDCP duplication.

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN.

The ASSISTANCE INFORMATION DATA frame may include UL Congestion Information and/or DL Congestion information measured by the corresponding node. The node hosting the NR PDCP entity shall, if supported, take this information into account to perform ECN marking in the NG-RAN node, or to further send it to the UPF for ECN marking or information exposure as specified in TS 23.501 [8].

Figure 5.4.3.1-1: Successful Transfer of Assistance Information Data

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### 5.5.3.46	UL Delay Indicator

Description: This parameter indicates the presence of the UL Delay DU Result. This information element is not applicable to E-UTRA PDCP.

Value range: {0= UL Delay DU Result not present, 1= UL Delay DU Result present}.

Field length: 1 bit.

#### 5.5.3.47	DL Delay Indicator

Description: This parameter indicates the presence of the DL Delay DU Result. This information element is not applicable to E-UTRA PDCP.

Value range: {0= DL Delay DU Result not present, 1= DL Delay DU Result present}.

Field length: 1 bit.

#### 5.5.3.48	UL Delay DU Result

Description: This field indicates UL delay measured at the corresponding node in milliseconds for the concerned DRB over Uu interface. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the total UL delay for the concerned DRB and it may report the calculated total delay to the UPF for the purpose of QoS monitoring as specified in [8]. The node hosting PDCP may also use this information for the purpose of UE performance monitoring as specified in TS 38.300 [x]. This information element is not applicable to E-UTRA PDCP.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.49	DL Delay DU Result

Description: This field indicates DL delay measured at the corresponding node in milliseconds for the concerned DRB over Uu interface. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the total DL delay for the concerned DRB and it may report the calculated total delay to the UPF for the purpose of QoS monitoring as specified in [8]. The node hosting PDCP may also use this information for the purpose of UE performance monitoring as specified in TS 38.300 [x]. This information element is not applicable to E-UTRA PDCP.

Value range: {0..232-1}.

Field length: 4 octets.

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>