3GPP TSG-RAN WG3 Meeting #128	R3-253591

St Julian’s, Malta, 19 – 23 May, 2025

Title:	[Draft] Reply LS to LS on Continuous MDT

Response to:	S5-252114 - R3-253024

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	Nokia [to be RAN3]

To:	SA5

CC :

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA5 for their reply LS on Continuous MDT.

Concerning: Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

Answer from RAN3: RAN3 has discussed and would like to provide the following OAM requirements on configuration details:

- OAM configuration enables the gNB to select the optimal immediate MDT configuration for available UEs, e.g. by taking into account UE capabilities and combine it with the corresponding OAM-defined logged MDT configuration for Continuous MDT.

- OAM provisions immediate MDT configurations for continuous MDT which includes own TR (for the immediate MDT configuration) and additionally includes the TR of the corresponding logged MDT configuration to which it is linked.

- Trace records for immediate MDT include both the immediate MDT TR/TRSR (in order to identify the immediate MDT configuration) as well as the logged MDT TR/TRSR (for correlation).

- The OAM configuration enables the gNB to determine that an MDT Configuration (Logged or Immediate MDT) is for Continuous MDT.

- Continuous MDT uses the same Area Scope parameter definition as legacy MDT, and in case of absence of Area Scope parameter the Continuous MDT session is applicable within the entire PLMN.

- OAM determines a Configuration Area for Continuous MDT to be an area within which NG-RAN nodes will select UEs for a given Continuous MDT job. The Continuous MDT solution should enable that the Configuration Area is possibly smaller than the area represented by the Area Scope.

- The OAM system indicates in the MDT configuration which measurements that are mandatory and which measurements can be performed optionally by the UE.

Concerning: Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

Answer from RAN3: RAN3 would like to point out that continuous MDT problem is only relevant for MDT Configurations where there exists at least one UE measurement that is common across RRC Idle/RRC Inactive and RRC Connected states (e.g., RSRP). MDT configurations corresponding to the same measurement having possibly different reporting or logging parameters are equivalent with respect to Continuous MDT. In order to limit the Continuous MDT impacts to the UE and enable a wider applicability of the feature, in Continuous MDT a UE is should not be mandated to provide all the measurements indicated in the Trace Reference to a gNB.

On this aspect, RAN3 would like to provide the following OAM requirement:

- The OAM system indicates in the MDT configuration which measurements are mandatory and which measurements can be performed optionally by the UE.

Concerning: Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

Answer from RAN3: RAN3 has discussed the issue raised by SA5 related to overlapping TR/TRSRs. RAN3 believes that this issue would be best solved by reusing the Cell Traffic Trace functionality and a proper ID for the correlation of the management-based MDT reports at TCE. A gNB sends Cell Traffic Trace to an AMF when it selects a UE for MDT, informing AMF of the allocated TR and TRSR. This means that the AMF knows the pair of TR and TRSR allocated by the gNB for a given MDT session to a given UE. In legacy MDT framework, the AMF forwards to the TCE the TR and TRSR pair, along with (optionally) the IMEI-TAC of the UE, depending on the anonymization level, which are subsequently combined by TCE together with the MDT records received from different gNBs. For Continuous MDT, AMF could create a UE identifier, not revealing UE identity, from a permanent UE identifier and send it to TCE for the Trace Correlation.

2. Actions:

To SA5 : 	RAN3 would like to ask SA5 to take the above clarifications into account and provide feedback to RAN3 if needed.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #129			  		25 - 29 	August 2025								Bengaluru, India

TSG-RAN WG3 Meeting #129-bis			  	13 - 17 	October 2025							Prague, CZ