3GPP TSG RAN WG3 Meeting #128	R3-253608

St Julian’s, Malta, 19 - 23 May, 2025

Agenda Item:	14.3

Source:	Jio Platforms Limited (JPL)

Title:	Support of Enhanced Multi-Hop Layer-2 UE-to-Network Relay

Document for:	Discussion

### 1. Introduction

Release 17 introduced a single-hop Layer-2 UE-to-Network relay mechanism in NR (specified in 3GPP TS 38.401, Clauses 8.19.1–8.19.4). While single-hop relay effectively extends coverage for UEs that are immediately out of range of a gNB but within range of an in-coverage Relay UE, many real-world scenarios (e.g., deep rural deployments, disaster areas, search-and-rescue missions, or vehicular convoys) require multiple hops of sidelink relay to reach the network.

This TDoc proposes a new clause, Clause 8.19.6 (Enhanced Multi-Hop L2 U2N Relay), which builds upon single-hop procedures to support two or more relay hops in a backward-compatible manner. We provide a comprehensive technical overview and outline of the new procedures, focusing on:

- Multi-hop chain formation and coordinated configuration.

- Topology awareness at the gNB-CU for robust, scalable relay.

- Streamlined RRC procedures and minimal interface changes, especially on F1.

- Backward compatibility with existing single-hop relay implementations.

### 2. Background and Rationale

Single-hop relay (Rel-17) enables coverage extension by allowing an in-coverage Relay UE to forward traffic from a Remote UE over PC5. However:

- Range limitation: A single hop covers only the distance from the out-of-coverage UE to one in-coverage Relay UE. Some environments require multiple intermediate hops to reach coverage.

- Reliability: A single-hop path might be disrupted if the Relay UE moves or fails; multi-hop can provide alternative paths or at least reconfigurable options.

- Evolving use cases: Industrial IoT, public safety, and remote coverage expansions increasingly demand multi-hop for robust connectivity.

Hence, multi-hop U2N relay is identified as a key Rel-19 feature to extend coverage and enhance reliability. The requirement is to:

- Extend the existing single-hop L2 relay architecture to support multi-hop.

- Maintain backward compatibility (do not break single-hop).

- Minimize impact on existing interfaces (notably F1, Xn).

- Provide a foundation for future intelligent routing (though not mandated in the initial specification).

### 3. Detailed Architectural Overview

#### 3.1. Basic Concept of Multi-Hop U2N Relay

- Layer-2 approach: As with single-hop, multi-hop relay remains an L2 forwarding concept. Each Relay UE terminates MAC/RLC for both the downstream PC5 link (toward the Remote UE) and the upstream link (which may be another PC5 link to an upstream Relay UE or a Uu link to the gNB).

- End-to-end anchor: The gNB-CU is the anchor for the Remote UE’s RRC and user-plane termination. The multi-hop chain simply acts as a bridging mechanism at L2.

#### 3.2. Roles of Remote UE, Relay UE(s), and gNB-CU

- Remote UE:

  - Out of direct coverage.

  - Discovers potential Relay UEs via sidelink discovery signals.

  - Initiates RRC connection setup (forwarded over PC5 hops) toward the gNB.

- Relay UE:

  - In coverage (directly or indirectly via another Relay UE).

  - Receives and forwards RRC signaling and data traffic.

  - Maintains two sets of RLC channels: one with the upstream side (another Relay UE or gNB) and one with the downstream side (another Relay UE or the Remote UE).

- gNB-CU:

  - Maintains an RRC context for each Remote UE, as well as control contexts for each Relay UE.

  - Allocates radio resources (via the gNB-DU) and configures the sidelink parameters.

  - Maintains a “topology map” of multi-hop chains to coordinate multi-hop resource assignment and adaptation.

#### 3.3. Example Multi-Hop Topology

(Remote UE) <--PC5--> [Relay UE1] <--PC5--> [Relay UE2] <--Uu--> (gNB)

- Relay UE2 is fully in coverage of the gNB (Uu interface).

- Relay UE1 is out of direct coverage from gNB but in sidelink range of Relay UE2.

- The Remote UE relies on Relay UE1 → Relay UE2 → gNB path.

- From the gNB’s perspective, it sees:

  - “UE2” as an in-coverage Relay UE with a normal RRC connection over Uu.

  - “UE1” as a remote device connected via UE2.

  - The final “Remote UE” as a remote device connected via UE1 (and transitively via UE2).

### 4. Enhanced Procedures for Multi-Hop Relay

This section outlines the proposed enhancements to be added as Clause 8.19.6 in TS 38.401.

#### 4.1. Bulk Configuration of Relay Chains

Current Single-Hop:

- When a Remote UE initiates RRC setup (via the Relay UE), the gNB-CU configures the Relay UE with PC5/Uu relay channels in several sequential steps (SRB1 first, followed by additional configurations for SRB2 and DRBs).

Multi-Hop Challenge:

- Cascading these steps over each hop can incur significant delay for the final Remote UE. Each hop might need a back-and-forth exchange before proceeding to the next hop.

Proposal:

- Use a “chain-level” bulk configuration procedure. When the gNB-CU learns (from the initial RRC Setup Request or from Relay UEs’ measurement reports / discovery info) that a Remote UE requires multiple hops, it prepares a unified configuration container.

- This container includes all the required sidelink RLC configurations (PC5 link parameters, logical channel details, IDs, QoS, etc.) for each hop in the chain, plus any relevant Uu modifications for the in-coverage Relay UE.

- The gNB-CU sends this container as part of an RRC Reconfiguration message to the final in-coverage Relay UE, which then distributes the relevant sub-parts to the next downstream Relay UE, and so on, until it reaches the Remote UE.

Benefits:

- Reduced latency: Fewer RRC round trips.

- Scalability: Allows 2, 3, or more hops without dramatically increasing setup time.

- Implementation: The gNB-DU largely sees normal “UE Context Setup” for the in-coverage Relay UE, while the multi-hop specifics are contained in a pass-through container on RRC signaling.

#### 4.2. Topology Awareness at the gNB-CU

To orchestrate bulk configuration, the gNB-CU must track the chain structure:

- Multi-Hop Context: For each Remote UE, the gNB-CU’s control-plane maintains which Relay UEs are in the path. This includes an ordered list: [Remote UE] → Relay UE1 → Relay UE2 → ... → Relay UEn → gNB.

- Discovery and Maintenance:

  - Discovery: Relay UEs periodically broadcast or scan for discovery messages (PC5). This is consistent with the existing ProSe/discovery framework. Once a potential route is found, the information is sent upstream to the gNB-CU so it can build or update the chain context.

  - Maintenance: If any Relay UE fails, moves away, or becomes overloaded, the gNB-CU can adapt the chain or instruct the Remote UE to discover alternative routes.

- Chain Identifier or Tag: The gNB-CU may assign an internal identifier (e.g., ChainID) to each multi-hop path for ease of reference. This ID can be used in RRC messages to indicate chain membership.

Advantages:

- Coordinated resource allocation: The gNB-CU can schedule resources (indirectly via the DU or via sidelink coordination) more effectively when it knows how many Remote UEs rely on each Relay UE.

- Adaptation: Supports advanced features like route switching or partial chain reconfiguration if a better path becomes available.

- Controlled Hops: The network can impose a max hop limit (e.g., 3 or 4) for performance, ensuring acceptable QoS/latency.

#### 4.3. Streamlined PC5 and Uu RLC Channel Setup

Multi-hop means each hop requires a pair of RLC channels for inbound and outbound traffic. For the final hop in coverage, that “pair” is the PC5 link plus the standard Uu link (Relay UE ↔ gNB). For intermediate hops, it’s PC5 ↔ PC5. The core enhancements:

- Single RRC Reconfiguration per Relay UE:

  - Instead of multiple incremental RRC configurations for each SRB and DRB, the gNB-CU can send a single comprehensive configuration that includes all the necessary logical channels for a given Remote UE’s SRBs and DRBs.

  - Example: The message contains IEs describing SRB1, SRB2, and the required DRB(s). The Relay UE applies these channel configs to both its sidelink (downstream) and its upstream (which might be sidelink or Uu).

- Bearer Parallel Setup:

  - As soon as the signaling bearer (SRB1) is established across all hops, the gNB-CU can push the data bearer configurations.

  - Security procedures (e.g., Security Mode Command for the Remote UE) can be passed through the chain. Meanwhile, the physical RLC channels for DRBs can be pre-established so that once security is completed, the data bearers become active immediately.

- Consistent Identity:

  - The Remote UE is assigned a unique ID or combination of IDs (e.g., a C-RNTI or T-UEID in some networks). All Relay UEs in the chain use that same ID in RLC or MAC headers for forwarding.

  - This prevents complicated ID translations at each hop.

- Retention of RLC Context:

  - Relay UEs may keep partially configured RLC contexts (semi-active) for a Remote UE if the link is expected to re-establish quickly (e.g., if the Remote UE goes briefly out of range).

  - This is an implementation detail but can significantly improve reliability.

#### 4.4. Minimal Impact on gNB-DU/gNB-CU F1 Interface

A crucial design principle is limiting changes to F1 (between gNB-CU and gNB-DU):

- Relay Abstraction: The DU schedules a Relay UE like any normal UE (with potential sidelink grants if the DU is controlling sidelink resources, or the Relay UE uses autonomous resource selection). The DU does not need to know that “UE2 is serving two more hops.”

- No New F1 Procedures: Existing F1AP messages such as UE Context Setup Request, UE Context Modification, etc., are reused. Possibly new IEs are optional.

- Backward Compatibility:

  - An older DU that doesn’t understand the new IEs can ignore them.

  - The multi-hop logic is primarily in the gNB-CU and Relay UEs.

  - Thus, deployments can upgrade the gNB-CU first, enabling multi-hop even with older DUs, as long as the Relay UEs also support multi-hop.

#### 4.5. Intelligent Route Optimization (Optional)

- Out-of-scope for core normative text: The specification does not define or require AI/ML-based routing algorithms.

- Future Potential: Vendors/operators can deploy advanced route-selection logic in the gNB-CU or an external orchestrator. The multi-hop architecture is flexible enough to allow dynamic re-routing or load balancing without changing the fundamental specification.

### 5. Backward Compatibility and Coexistence with Single-Hop

#### 5.1. Preservation of Existing Procedures in Clauses 8.19.1–8.19.5

- Single-hop L2 relay remains intact.

- All new multi-hop procedures (Clause 8.19.6) apply only when the network (gNB-CU) and UEs (Relay UE, Remote UE) negotiate or advertise multi-hop capability.

#### 5.2. Core Network Transparency

- From a 5GC perspective, the Remote UE is anchored at the gNB (via N2, N3). The number of relay hops at L2 is irrelevant to the core.

- No new NAS procedures or core network identifiers are introduced for multi-hop.

#### 5.3. RRC and NAS Continuity

- Standard RRC flows are used for the Remote UE’s attach, security, registration, and mobility procedures. They are simply forwarded over multiple sidelink hops.

- If the Remote UE moves into direct coverage or finds a shorter chain, the normal RRC handover or cell reselection logic can be triggered. The gNB-CU updates the path accordingly.

#### 5.4. F1/Xn Interface Compatibility

- F1: As explained, minimal or no changes. Possibly an optional IE in UE Context Setup Request indicating “This UE acts as a relay for X remote UEs,” but the DU can ignore it if not supported.

- Xn: For inter-gNB mobility, the source gNB can inform the target gNB that the incoming UE is served via multi-hop relay. However, if the target gNB supports single-hop or direct coverage, it can reassign the path. The core “Xn Handover Request/ Acknowledge” messages remain fundamentally the same, with optional extension for multi-hop context if needed.

### 6. Detailed Procedure Flow Example

This section provides a step-by-step example of how multi-hop relay might be established and subsequently managed. The example covers:

- Discovery

- Access Initiation

- Chain Configuration

- Bearer Setup

- Mobility/Handover

#### 6.1. Multi-Hop Discovery and Access Initiation

- Out-of-coverage Remote UE periodically transmits or monitors sidelink discovery signals.

- Relay UE1 (which is also out of direct gNB coverage but has a connection to Relay UE2) responds to or detects the Remote UE’s discovery messages. The Remote UE now sees “Relay UE1” as a possible relay candidate.

- Relay UE1 itself has discovered/connected to Relay UE2 which is in gNB coverage. Relay UE2 has a normal Uu connection to the gNB.

- The Remote UE attempts an RRC Setup Request (or an initial access request message) via PC5 to Relay UE1. Relay UE1, upon receiving it, forwards a “relay request” to Relay UE2. Relay UE2 then sends an indication (via RRC or an internal message to the gNB-CU) that there is a Remote UE two hops away requesting service.

- The gNB-CU becomes aware of the needed chain: “Remote UE → Relay UE1 → Relay UE2 → gNB.”

#### 6.2. Relay Chain Configuration Exchange

- Once the gNB-CU decides to admit this multi-hop connection, it prepares a Multi-Hop Configuration Container. This container includes:

  - Sidelink RLC config for Remote UE ↔ Relay UE1.

  - Sidelink RLC config for Relay UE1 ↔ Relay UE2.

  - Uu RLC adjustments (if needed) for Relay UE2.

  - Bearer mapping for SRB1, SRB2, and future DRBs.

  - IDs (e.g., a chain ID or the Remote UE’s C-RNTI, additional T-UEIDs, etc.).

- The gNB-CU places this container in an RRC Reconfiguration message addressed to Relay UE2 over Uu.

- Relay UE2 extracts the relevant portion that configures “Relay UE2 ↔ Relay UE1” sidelink, then encapsulates the portion for “Relay UE1 ↔ Remote UE” in a message to Relay UE1 over PC5.

- Relay UE1, upon receiving its portion, further distributes the final config for “Relay UE1 ↔ Remote UE” PC5 channels to the Remote UE.

- In this manner, the entire chain’s radio bearers can be configured in a single coordinated flow from the gNB-CU. This drastically reduces the multiple multi-step negotiations that would otherwise occur.

#### 6.3. Bearer Setup, Security, and RLC Configuration across Multiple Hops

- SRB1 is established end-to-end (over multiple PC5 segments + one Uu segment).

- The gNB-CU initiates Security Mode Command for the Remote UE (same as single-hop). The messages are forwarded hop by hop.

- After security activation, the gNB-CU can quickly configure SRB2 and DRBs for user-plane traffic. Because the RLC channel definitions for these were pre-included in the configuration container, activation is swift.

- The Remote UE can now exchange user-plane traffic over the newly established DRBs. Data flows: Remote UE ↔ Relay UE1 (PC5) ↔ Relay UE2 (PC5) ↔ gNB (Uu).

- RLC/PDCP layers in each Relay UE handle segmentation, re-transmissions, and MAC scheduling for their respective hops.

#### 6.4. Mobility and Handover with Multi-Hop Relay

- If Relay UE2 (in coverage) moves to another cell served by a different gNB but remains in coverage, a standard RRC handover can occur. The gNB-CU triggers an Xn-based or N2-based handover procedure, transferring the context of Relay UE2 (including the multi-hop chain info) to the target gNB.

- The target gNB sets up equivalent resources for Relay UE2 (still treating Relay UE2 as a single UE). The chain context can be re-established, or a multi-hop re-configuration can be triggered if needed.

- If Relay UE1 moves closer to the new gNB or to some other Relay UE in coverage, the system can pivot the chain so that the Remote UE sees a new route. The gNB-CU updates the multi-hop chain references accordingly.

- If the Remote UE itself moves into direct coverage, the gNB-CU can remove the relay chain and hand over the Remote UE to a direct RRC connection.

### 7. Observations and Proposals

Below is a consolidated table capturing the observations and associated proposals:

| Observation | Proposal |
| --- | --- |
| 1. Single-hop relay limitation Rel-17 covers only one hop, inadequate for deep coverage. | 1. Introduce multi-hop L2 U2N relay Extend coverage and throughput with 2+ relay hops. |
| 2. Non-disruptive architecture Need multi-hop without breaking single-hop. | 2. Enhanced multi-hop design A chain of Relay UEs (each using PC5) to link a truly remote UE to the gNB, with minimal changes to single-hop procedures. |
| 3. Sequential hop-by-hop setup latency Naive approach adds extensive overhead. | 3. Bulk chain configuration The gNB-CU configures multiple relay hops in a single or minimized set of RRC procedures, reducing round-trip delays. |
| 4. Lack of chain-level awareness CU must manage complex multi-hop relationships. | 4. Topology management Maintain a record of each hop in the chain to enable coordinated allocation, quick reconfiguration, and future route optimization. |
| 5. RLC channel setup overhead Multi-hop environment duplicates the need for many PC5↔Uu pairs. | 5. Streamlined multi-hop RLC - Single RRC reconfiguration per Relay UE. - Pipelined bearer setup. - Consistent ID usage. - Potential RLC context retention for robustness. |
| 6. Potential impact on F1 We must keep the design minimal to support older gNB-DUs. | 6. Minimal F1 changes - No new mandatory F1 procedures. - Relay abstraction: DU sees each Relay UE as a normal UE. - Optional IEs for multi-hop if DU can interpret them. |
| 7. Intelligent routing AI/ML-based path selection can enhance performance. | 7. Future AI/ML approach Keep the normative spec simple and allow vendor-specific or operator-specific route optimization on top. |
| 8. Backward compatibility Essential to preserve existing single-hop and legacy UEs. | 8. Full backward compatibility - Single-hop procedures remain in 8.19.1–8.19.5. - Multi-hop is optional, triggered only if network/UEs support it. - No changes required in 5GC or mandatory changes in gNB-DU. |

### 8. Conclusion and Proposed Actions

This TDoc proposes adding a new clause, Clause 8.19.6, to 3GPP TS 38.401 to define enhanced multi-hop L2 UE-to-Network relay for Release 19. The proposed approach:

Proposal 1: Extends single-hop relay to multiple hops in a modular and backward-compatible manner.

Proposal 2: Introduces bulk configuration of relay chains, ensuring low-latency setup.

Proposal 3: Defines topology awareness at the gNB-CU, enabling coordinated management of multi-hop paths.

Proposal 4: Minimizes changes to the F1 interface and preserves existing RRC/NAS flows for seamless integration.

Proposal 5: Leaves future route optimization or AI/ML solutions as optional enhancements, not mandatory for baseline multi-hop operation.

Proposed Actions:

- Discuss and Approve the detailed architecture in RAN WG#X.

- Incorporate the proposed Clause 8.19.6 into TS 38.401.

- Coordinate with relevant WGs (e.g., RAN2 for RRC, RAN3 for interface aspects) for any supporting CRs or new IEs.

- Ensure that the final text aligns with existing single-hop relay procedures and ensures consistent referencing to TS 38.331 for RRC messages.

## Appendix A: Potential RRC Signaling Extensions (Illustrative)

While the exact RRC IE definitions would be specified in 3GPP TS 38.331, the following outlines possible structures:

- MultiHopRelayConfig: A container at the gNB-CU indicating:

  - numHops (e.g., 2, 3, etc.)

  - chainList[] containing identities of intermediate Relay UEs and the Remote UE.

  - BearerConfigList[] describing SRB1, SRB2, DRB details (QoS, LCID, PDCP config).

- RelayInfoToDownstream: A sub-container each Relay UE passes to the next hop, containing the relevant PC5 configuration.

- Optional ChainID or RelayPathID for referencing the entire path in future reconfiguration or release procedures.

Note: These are conceptual placeholders. Actual naming and structure will follow standard 3GPP RRC IE naming conventions and syntax.

## Appendix B: Example Message Sequence Diagram

This expanded diagram highlights how the bulk configuration is distributed downstream, enabling the entire chain to be set up in a coordinated manner.