3GPP TSG-RAN WG3 Meeting #128	R3-253609

# Malta, MT, 19 - 23 May, 2025

Agenda Item:	14.3

Source:    TCL.

Title:         Further Discussion on Support of NTN Regenerative Architecture

Document for:	Discussion and Decision

# Introduction

As part of Release 19, a new work item is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks to support NTN architecture with 5G system functions on board of satellite (i.e., regenerative payloads) [1]. The support of NTN architecture with 5G system functions of regenerative payloads provides new architecture option(s) besides the transparent payload as specified in Rel-17 and Rel-18, which makes the deployment of non-terrestrial network more flexible. On this regard, during the RAN3 127, there was a discussion on NG Interface management, inactive UE support and feeder link switch issues, and the following agreements has been made.

Agreements: Support of RRC_INACTIVE: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Whether to do any extra enhancement(s) for Inactive to be continued. Support of NG Suspend/Resume: Whether to support NG Suspend/Resume is to be continued, companies are encouraged to further consider the pros and cons of the NG Suspend/Resume.

During the RAN3 127 meeting the following issues were discussed:

And during the last meeting [6]

Whether to support NG suspend/Resume? When the on-board gNB performs feeder link switchover to the new NTN Gateway, gNB could notifies the AMF about the new DL NG-U TNL Information through Path Switch Request or PDU Session Resource Modify Indication message? TNL address updates due to feeder link switch [Agreement] Proposal 2, in case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used. NG Connection interruption due to hard feeder link switch over Proposal 3bis, Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any).

This report discusses NG interface NG Connection interruption due to hard feeder link switch over management and feeder link switch and their impacts on UE NG related procedures.

# Discussion

3GPP is currently discussing the regenerative payload architecture of NTN satellite access in both SA and RAN WGs As part of Release 19, it is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks to support non-terrestrial network architecture with 5G system functions on board of satellite (i.e., regenerative payloads). In such an architecture the gNB can be deployed on-board of a satellite terminating the Uu radio interface toward the UE (via the service link) and terminating the NG network interface toward the 5GC (via the feeder link). Additionally, different gNBs hosted by two regenerative satellite payloads may often be interconnected to each other’s by the mean of an inter-satellite link (ISL) interface for exchanging of Xn protocol messages. Under such architecture, unlike terrestrial networks (TN) where NG interfaces is presumed to be always available, in regenerative NTN access, NG logical interfaces may often be disconnected, switched, suspended, or updated due to satellite movement or the switching of the  feeder link between gNB and AMF [2, 3, 4]. For example, if a UE is accessing from an NTN access with gNB on-board, there will always be a single NG logical link for such a UE towards its selected AMF/UPF (Figure 1). If a feeder link switching occurred (Figure 2), UE’s NG link related procedures such as initial UE message, initial UE context setup message exchange between UE serving gNB and selected AMF will be impacted.

Figure 1: Example of Regenerative NTN access with Feeder Link Switching

An NG disconnection caused by a feeder link change may disrupt UE NG-related procedures, including initial UE message exchange, initial UE context setup, paging message delivery, and downlink signaling/data transmissions. Such disruptions can affect UE RRC resumption, RRC reestablishment, and both UE-triggered and network-triggered state transitions between RRC IDLE and RRC CONNECTED modes. To maintain service continuity while preserving AMF stability during feeder link switches, two mitigation approaches can be considered. First, NG-related UE procedures could be contained within NG setup/removal signaling. Second, for irrecoverable link failures (after timeout or retry exhaustion), the 5GC system could either: (a) leverage AMF-controlled suspension/resumption of downlink transmissions based on a new RAN configuration indicator, or (b) enable satellite gNBs to dynamically reroute procedures via alternative Xn/NG links without AMF reselection. The proposed RAN configuration update indicator would carry two parameters: like NG link status and recoverability information (including time/retry thresholds for recovery attempts). Based on the above discussion, we propose:

Figure 2: Procedures of connection procedures during NG disconnection:

- Proposal 1: Support 5GC’s capability to suspend or resume downlink signaling/data transmissions to the gNB based on a new indicator in the RAN configuration update.

- Proposal 2:  If proposal 1 is agreed, include parameters such as NG link status and recoverability information (e.g., gNB time/retry thresholds for NG recovery attempts) in the solution.

- Proposal 3: Discuss containing UEs NG related procedure such as initial UE message, initial UE context setup as a part of NG setup/removal signalling to guarantee that AMF will not be changed for the UEs during the feeder link switch.

# Conclusion

In this contribution, we have discussed NG interface management and feeder link switch and their impacts on UE NG related procedures. Based on the discussion we proposed to discuss the following proposals

- Proposal 1: Support 5GC’s capability to suspend or resume downlink signaling/data transmissions to the gNB based on a new indicator in the RAN configuration update.

- Proposal 2:  If agreed, include parameters such as NG link status and recoverability information (e.g., gNB time/retry thresholds for NG recovery attempts) in the solution.

- Proposal 3: Discuss containing UEs NG related procedure such as initial UE message, initial UE context setup as a part of NG setup/removal signalling to guarantee that AMF will not be changed for the UEs during the feeder link switch.

# References

- RP-240775, “Revised WID: Rel-19 WI Non-Terrestrial Networks (NTN) for NR Phase 3”, THALES, CATT, RAN#103, Maastricht, The Netherlands, March 18-22, 2024.

- R3-243017,” LS on Support of Regenerative-based Satellite Access” SA2 (VIVO), Fukuoka, Japan, 20 - 24 May, 2024.

- R2-2405156” Regenerative payload for NR NTN” Samsung. Fukuoka, Japan, May 20th-24th, 2024

- R2-2404799” Considerations on regenerative payload in NTN” Fukuoka, Japan, May 20th-24th, 2024.

- R3-247844” Summary of Discussion on CB: NRNTN2_Regenerative”, CATT (Moderator), Orlando, US, November 2024.

- R3-252348 ” Summary of Offline Discussion on NRNTN”, Xiaomi, Wuhan, China, April, 2025