3GPP TSG-RAN WG3 Meeting #128	R3-253625

Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1500 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the definition of UE Throughput - option2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT,ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-Core |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: | As part of the Rel18 WI on AI/ML for NG-RAN, Average UE Throughput, among other metrics, was agreed as a metric for UE performance monitoring. However, the definition of Average UE Throughput has the following issue: - The reference of “9.2.3.4 Bit Rate” as the IE type of UE throughputs incurred a mismatch in the definition of the Average UE Throughput in TS 28.558(no matter RLC level or PDCP level) and the definition in TS 38.423. |  |  |  |  |  |  |  |  |  |
| Summary of change: | Introduce a new IE type for the Average UE Throughput Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has a limited impact on previous version of the specification because it only impacts the AI/ML for NG-RAN. This CR is NBC. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | Maintaining an incorrect metric because it fails to accurately represent the Average UE Throughput. |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 9.2.3.179, 9.2.3.xxx(new), 9.3.5 |  |  |  |  |  |  |  |  |  |
|  | Y |  | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

#### 9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Average UE Throughput 9.2.3.xxx |  |
| Average UE Throughput UL | O |  | Average UE Throughput 9.2.3.xxx |  |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.xxx	Average UE Throughput

This IE specifies the Average UE Throughput UL/DL, which indicates that the number of bits delivered by NG-RAN in UL or to NG-RAN in DL, divided by the effective transmission time.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average UE Throughput | M |  | INTEGER (0..4,000,000,000,000,…) | The unit is: bit/s |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<Unmodified Text Skipped>>

AverageUEThroughput	::= INTEGER (0..4000000000000,...)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

UEPerformance ::= SEQUENCE {

dL-UE-AverageThroughput						AverageUEThroughput					OPTIONAL,

uL-UE-AverageThroughput						AverageUEThroughput					OPTIONAL,

uE-AveragePacketDelay						AveragePacketDelay		OPTIONAL,

uE-AveragePacketLossDL						PacketLossRate			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,

...

}

UEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CHANGES END