3GPP TSG-RAN WG3 Meeting #128	R3-253664

Malta, MT, 19th–23th May, 2025

Title: 	Discussion on the AIoT supportable NGAP Interface Management

Source: 	Tejas Networks

Agenda item:	16.2

Document Type:	Discussion

# 1. Introduction

This TDoc discusses the enhancements required for supporting NGAP Interface Management Procedures for AIoT.

# 2. Discussion

2.1 Signalling of AIoT RAN Information to AIOTF

AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list.

AIOTF may be aware of the location of reader via OAM configuration.

Shown below are the agreed proposals from the previous meetings. 
It is clear from these agreed statements that the AIoT RAN information known to the AIOTF contains supported AIoT area list, served reader ID list and may also contain reader locations. When a gNB become active it can signal the latest AIoT RAN information to the AIoTF by the NG SETUP REQUEST message. When there is a change in the reader list (when a reader is down/added) and AIoT area list supported by the gNB, the updated supported lists can be sent to AIOTF via RAN CONFIGURATION UPDATE message.

It was agreed in the last meeting to introduce a new AIoT area. We assume one reader should only serve in a single AIoT area. Mapping of these readers with their respective areas should also be sent in the AIoT RAN Information transfer IE, from the gNB to AMF/AIOTF..

9.2.6.1 NG SETUP REQUEST

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| ** Excluding IEs that remain unaffected ** |  |  |  |  |  |  |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| AIoT RAN Information | O |  | AIoT RAN Information(9.3.x.x) | AIoT area, reader information and reader mapping information | YES | Reject |

9.3.X.X.AIoT RAN Information

| - IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Serving Reader List | O | 1 |  |  |
| >Serving Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Reader ID | M |  |  |  |
| >>Reader Location | O |  | 9.3.x.y |  |
| AIoT Area and reader mapping list | O | 1 | AIoT areas supported by gNB and readers mapped with these areas. |  |
| >AIoT Area item |  | 1..<maxnoofAIoTareas> |  |  |
| >>AIoT area Code | M |  | FFS |  |
| >>PLMN List |  | 1 |  |  |
| >>>PLMN Item |  | 1..<maxnoofPLMNs> |  |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 |  |
| >>>>Reader list |  | 1 |  |  |
| >>>>>Reader items |  | 1..<maxnoofreaders |  |  |
| >>>>>>Reader ID | M |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTareas | Maximum no. of AIoTAreas supported by a gNB. Value is 256. |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |

#### 9.3.x.y Reader Location

This IE indicates the location information of the A-IoT Reader provided by the NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reader Location | M |  |  |  |
| >Civic Address |  |  |  |  |
| >>Civic Address | M |  | OCTET STRING | Includes the CivicAddress as defined in TS 29.572. |
| >NG-RAN Access Point Position |  |  |  |  |
| >>NG-RAN Access Point Position | M |  | OCTET STRING | Includes the NG-RAN Access Point Position IE as defined in TS 38.455. |
| >NG-RAN High Accuracy Access Point Position |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position | M |  | OCTET STRING | Includes the NG-RAN High Accuracy Access Point Position IE as defined in TS 38.455. |
| >Geographical Coordinates |  |  |  |  |
| >>Geographical Coordinates | M |  | (OCTECT STRING) | Includes the Geographical Coordinates IE as defined in TS 38.455 |

Proposal 1: Support Signalling of AIoT information from gNB to AIoTF in the NG SETUP REQUEST message which enable the gNB to provide latest AIoT RAN information to the AIOTF.

Proposal 2: Support signaling of updated AIoT information from gNB to AIoTF via RAN CONFIGURATION UPDATE message.

Introducing an AIoT indicator in the NGAP Setup Request message. The detail of this indicator can be further discussed.

AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

2.2 AIoT Indicator

As per the last meeting it was decided to introduce AIoT indicator in the NG SETUP REQUEST message. Following are the agreed proposals from the previous chair meetings RAN3#127 and RAN3#127 bis.

The main objective of introducing this IE is to disable/ignore some of the mandatory IEs related to NG RAN communication which are already existing in the NG SETUP message if the node only supports AIoT functionality. We propose to introduce an AIoT Support indicator if the NG-RAN node only support AIoT feature. When this IE is set, the mandatory IEs (Supported TA list, Default paging DRX) related to NR will be disabled/ignored at the receiving NGAP entity.

#### 9.2.6.1	NG SETUP REQUEST

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| AIoT support Indicator | O |  | Enumerated(true) | This IE indicates the NG-RAN node only supports AIoT | YES | reject |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. If AIoT support Indicator is true, this IE is ignored | YES | reject |
| **Excluding IEs that remain unaffected. ** |  |  |  |  |  |  |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 | If AIoT support indicator is true, this IE is ignored | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |

Proposal 3: Introduce AIoT support Indicator IE to ignore mandatory IEs related to the NR at the receiving NGAP entity, when the NG-RAN node only supports AIoT functionality.

#### 2.3 Enhancement of NGAP procedures

Enhancing the existing NGAP interface management procedures

In the last meeting (RAN3#127) chair note it was decided to enhance existing NGAP interface management instead of creating a new AIoT specific AP.

#### 2.3.1 AIoTF ID in NG SETUP RESPONSE message

#### The first modification is introduction of AIoT Support Indicator IE and AIoT RAN information IE in the NG SETUP REQUEST message in the RAN CONFIGURATION UPDATE message which was mentioned in the clause 2.1 and 2.2.
The NG-RAN needs to select an AIoTF to respond, so the AIoTF ID must be included in the NG SETUP RESPONSE message during direct connectivity. Some of the mandatory IEs in NG SETUP RESPONSE message are disabled if AIOTF ID is present in the NG SETUP RESPONSE message.

#### Proposal 4:Propose to Introduce AIOTF ID in the NG SETUP RESPONSE originating from the AIOTF in the Direct connectivity case.

#### 2.3.2 Changes to NG RESET message

#### In the NG RESET message  we propose to introduce AIoT – Associated Logical NG connection List to reset one or more device connections.

#### 9.3.3.z AIoT associated Logical NG- connection List

This IE contains a list of AIoT-associated logical NG-connections

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIoT-associated Logical NG-connection Item |  | 1..<maxnoofNGAIoTConnectionsToReset> |  |  |
| >RAN NGAP device ID | O |  |  |  |

#### Proposal 5: Propose to introduce AIOT- Associated Logical connection List to reset one or more AIoT device connections in the NG RESET message.

3.Conclusions

Proposal 1: Support Signalling of AIoT information from gNB to AIoTF in the NG SETUP REQUEST message which enable the gNB to provide latest AIoT RAN information to the AIOTF.

Proposal 2: Support signalling of updated AIoT information from gNB to AIoTF via RAN CONFIGURATION UPDATE message.

Proposal 3: Introduce AIoT support Indicator IE to ignore mandatory IEs related to the NR at the receiving NGAP entity, when the NG-RAN node only supports AIoT functionality.

#### Proposal 4:Propose to Introduce AIOTF ID in the NG SETUP RESPONSE originating from the AIOTF in the Direct connectivity case.

Proposal 5: Propose to introduce AIOT- Associated Logical connection List to reset one or more AIoT device connections in the NG RESET message.

4.Annex

TP to TS 38.413

#### 9.2.6.1	NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node ® AMF, NG-RAN node ® AIOTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| AIoT Support Indicator | O |  | Enumerated(true) | This IE indicates the NG-RAN node only supports AIoT | YES | reject |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. If AIoT support Indicator is true, this IE is ignored | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 | If AIoT support Indicator is true, this IE is ignored | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| AIoT RAN Information | O |  | AIoT RAN Information(9.3.x.x) | AIoT area, reader information and reader mapping information | YES | Reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |

#### 9.2.6.2	NG SETUP RESPONSE

This message is sent by the AMF or the AIOTF to transfer application layer information for an NG-C interface instance.

Direction: AMF ® NG-RAN node, AIOTF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 | This IE is ignored if the Extended AMF Name IE or AIOTF ID is present | YES | reject |
| AIOTF ID | O |  |  | This IE is present if NGRAN connecting to AIOTF directly. | YES | reject |
| Served GUAMI List |  | 1 |  | This IE is ignored if AIOTF ID is present | YES | reject |
| >Served GUAMI Item |  | 1..<maxnoofServedGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |
| >>Backup AMF Name | O |  | AMF Name 9.3.3.21 |  | - |  |
| >>GUAMI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Relative AMF Capacity | M |  | 9.3.1.32 | This IE is ignored if AIOTF ID is present | YES | ignore |
| PLMN Support List |  | 1 |  | This IE is ignored if AIOTF ID is present | YES | reject |
| >PLMN Support Item |  | 1..<maxnoofPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>Slice Support List | M |  | 9.3.1.17 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support | O |  | 9.3.3.44 | If NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>Extended Slice Support List | O |  | 9.3.1.191 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >>Onboarding Support | O |  | ENUMERATED (true, ...) | Indication of onboarding support. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |
| Mobile IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for mobile IAB. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedGUAMIs | Maximum no. of GUAMIs served by an AMF. Value is 256. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |

#### 9.2.6.3	NG SETUP FAILURE

This message is sent by the AMF or the AIOTF to indicate NG setup failure.

Direction: AMF ® NG-RAN node, AIOTF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to Wait | O |  | 9.3.1.56 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node ® AMF, NG-RAN node ® AIOTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| AIoT RAN Information | O |  | AIoT RAN Information(9.3.x.x) | AIoT area, reader information and reader mapping information | YES | Reject |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

#### 9.2.6.5	RAN CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the AMF or the AIOTF to acknowledge the NG-RAN node transfer of updated information for an NG-C interface instance.

Direction: AMF ® NG-RAN node, AIOTF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.6	RAN CONFIGURATION UPDATE FAILURE

This message is sent by the AMF or the AIOTF to indicate RAN configuration update failure.

Direction: AMF ® NG-RAN node, AIOTF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to Wait | O |  | 9.3.1.56 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.6.11	NG RESET

This message is sent by both the NG-RAN node and the AMF to request that the NG interface, or parts of the NG interface, be reset.

This message is also used by both NG-RAN and the AIOTF when there is reset required in the AIoT part.

Direction: NG-RAN node ® AMF and AMF ® NG-RAN node, NG-RAN node ® AIOTF and AIOTF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >NG interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all, …) |  | - |  |
| >Part of NG interface |  |  |  |  |  |  |
| >>UE-associated Logical NG-connection List | M |  | 9.3.3.25 |  | - |  |
| >>AIoT associated Logical NG- connection List | O |  | 9.3.3.z |  | - |  |

#### 9.2.6.12	NG RESET ACKNOWLEDGE

This message is sent by both the NG-RAN node and the AMF or both the NG-RAN node and the AIOTF as a response to an NG RESET message.

Direction: NG-RAN node ® AMF and AMF ® NG-RAN node , NG-RAN node ® AIoTF and AIoTF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| UE-associated Logical NG-connection List | O |  | 9.3.3.25 |  | YES | ignore |
| AIoT associated Logical NG-connection List | O |  | 9.3.3.z |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.13	ERROR INDICATION

This message is sent by both the NG-RAN node and the AMF , or both the NG-RAN node and the AIOTF, to indicate that some error has been detected in the node.

Direction: NG-RAN node ® AMF and AMF ® NG-RAN node, NG-RAN node ® AIOTF and AIOTF ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | O |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | O |  | 9.3.3.2 |  | YES | ignore |
| Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | ignore |

9.3.X.X.AIoT RAN Information

This IE provides AIoT RAN information from NG RAN node to the AIOTF

| - IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Serving Reader List | O | 1 |  |  |
| >Serving Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Reader ID | M |  |  |  |
| >>Reader Location | O |  | 9.3.x.y |  |
| AIoT Area and reader mapping list | O | 1 | AIoT areas supported by gNB and readers mapped with these areas. |  |
| >AIoT Area item |  | 1..<maxnoofAIoTareas> |  |  |
| >>AIoT area Code | M |  | FFS |  |
| >>PLMN List |  | 1 |  |  |
| >>>PLMN Item |  | 1..<maxnoofPLMNs> |  |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 |  |
| >>>>Reader list |  | 1 |  |  |
| >>>>>Reader items |  | 1..<maxnoofreaders |  |  |
| >>>>>>Reader ID | M |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTareas | Maximum no. of AIoTAreas supported by a gNB. Value is 256. |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |

#### 9.3.x.y Reader Location

This IE Indicates the location of readers.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reader Location | M |  |  |  |
| >Civic Address |  |  |  |  |
| >>Civic Address | M |  | OCTET STRING | Includes the CivicAddress as defined in TS 29.572. |
| >NG-RAN Access Point Position |  |  |  |  |
| >>NG-RAN Access Point Position | M |  | OCTET STRING | Includes the NG-RAN Access Point Position IE as defined in TS 38.455. |
| >NG-RAN High Accuracy Access Point Position |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position | M |  | OCTET STRING | Includes the NG-RAN High Accuracy Access Point Position IE as defined in TS 38.455. |
| >Geographical Coordinates |  |  |  |  |
| >>Geographical Coordinates | M |  | (OCTECT STRING) | Includes the Geographical Coordinates IE as defined in TS 38.455 |

#### 9.3.3.z AIoT associated Logical NG- connection List

This IE contains a list of AIoT-associated logical NG-connections

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIoT-associated Logical NG-connection Item |  | 1..<maxnoofNGAIoTConnectionsToReset> |  |  |
| >RAN NGAP device ID | O |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNGAIoTConnectionsToReset | Maximum no. of AIoT-associated logical NG-connections allowed to reset in one message. Value is FFS. |