3GPP TSG-RAN WG3 Meeting #128	R3-253665

Malta, MT, 19th–23th May, 2025

Title: 	Enhancement of Inventory/Command procedure

Source: 	Tejas Networks Limited

Agenda item:        16.2

Document Type:	Discussion

# 1.Introduction

In this Tdoc, we propose the introduction of two new Information Elements (IEs) within the command procedures. These IEs are essential for accurately determining when to release the per session per device contexts in the NG-RAN.

# 2. Discussion

2.1 Maximum Response Wait Time IE

Suppose the AIOTF has sent a command request targeting to a specific device, there can be a possibility that NG RAN may not get any response as the device is unreachable. The reason for unreachability of the device can be different. So we propose to introduce a ‘ Maximum response wait time” IE in the Command Assistance Information IE  of the Command Request Transfer IE which is in the Command Request message. ‘Maximum Response Wait Time’ IE is the time the NG RAN should  wait for a response from a requested device. If the NG-RAN didn’t receive a response from the device within this time period, the NG-RAN will indicate to the core network about the inability to reach the device.

#### 9.3.3.yy3	Command Assistance Information

This IE includes the assistance information for the A-IoT Command Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Estimate of Expected D2R Message Size | O |  | FFS |  |
| Maximum Response Wait Time | O |  | Time | This IE provides the maximum wait time for the NG-RAN from the start of Command Request to provide a Command Response. |

Proposal 1: Propose to introduce Maximum Response Wait time IE in the Command Assistance Information IE, which will help to indicate the AIOTF about the unavailability of the requested device.

2.2 Last Command Indication IE

It was agreed in RAN2 that a Reader can serve only one procedure at a time.It is necessary to release the per session per deivce contexts  after a successful completion of a session(correlation ID) .So that readers can proceed with the next procedure. The AIOTF is aware of the number of command requests to be sent as well as the IDs of the recipient devices during the procedure. AIOTF is also aware of the last command request for a particular session(correlation ID). . So AIOTF can include a flag in its last command request to the requisite device indicating it’s the  final command for this particular correlation ID. So we propose to introduce an IE “Last Command Indication” in the Command Request Transfer IE.

#### 9.3.x.5	Command Request Transfer

This IE provides the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  | YES | reject |
| A-IoT NAS PDU | M |  | OCTET STRING |  | YES | reject |
| Command Assistance Information | O |  | 9.3.3.yy3 |  | YES | reject |
| Last Command Indication | O |  | ENUMERATED(true,..) | This IE when set to true indicates that there are no more command Requests for the corresponding A-IoT Correlation Identifier. | YES | reject |

Proposal 2: Propose to introduce “Last Command Indication” IE in the last Command Request of the ongoing session (Correlation ID) to indicate the end of the procedure.

## 2.3 Device Identification

A part of the last meeting’s chair note is shown below:

Editor’s Note: this IE enables identification of a single A-IoT device, a group of A-IoT devices, all A-IoT devices, detailed coding FFS

- The Inventory Request Transfer IE, also includes the following:

  - Device Identification for Paging (to enable paging for single device, a group of devices, all devices)

  - Requested Service Area Information (A-IoT Area list and/or Reader list)

  - Inventory Assistance Information (Approximates Number of Target A-IoT Devices).

In the last meet it was decided to include an IE which can be used for device Identification for paging. The main purpose of including this IE is to filter the required AIoT devices from the pool of AIoT devices for a specific procedure.

In release 19, RAN2 supports paging to carry only one ID (can be a device ID, filtered ID) or no ID. But RAN2 is working on making the paging message extensible for future release and it is better to have provision in NGAP message to support multiple IDs being signalled from AIoTF. Therefore the  AIoT device Identification Request Information Element (IE) shall be structured to support three distinct cases: (1) Device Identity List:, (2) Group Identity List: , and (3) a NULL option to represent inventory of All Devices. This can be achieved by defining the IE as CHOICE type with three branches. In release 19, the maximum IDs in the device ID list can be 1.

#### AIoT Device Identification Requested IE

This IE specifies the identity associated with the A-IoT device(s) that are to be paged.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE AIoT Device Identification Requested | M |  |  |  |
| >Device Identity list |  |  |  |  |
| >>Device Identity Item |  | 1<..maxnoofdeviceIDs> |  |  |
| >>>Device Identity | M |  | Device Identification |  |
| >Group Identity list |  |  |  |  |
| >>Group Identity item |  | 1<..maxnoofgroupIDs> |  |  |
| >>>Group Identity | M |  | Device Identification |  |
| >All Devices |  |  | NULL |  |
| Range bound | Explanation |  |  |  |
| maxnoofdeviceIDs | In this release, Maximum no. of device IDs allowed is 1 |  |  |  |
| maxnoofgroupIDs | In this release, Maximum no. of group IDs allowed is 1 |  |  |  |

#### Device Identification IE

This IE represents the A-IoT device identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Device Identity | M |  | OCTET STRING |  |

Proposal 3: Propose to introduce AIoT Device Identification Requested IE as a Choice Type IE with three branches (Device Identity list, Group Identity list and All devices)

3.Conclusions

Proposal 1: Propose to introduce Maximum Response Wait time IE in the Command Assistance Information IE, which will help to indicate the AIOTF about the unavailability of the requested device.

Proposal 2: Propose to introduce “Last Command Indication” IE in the last Command Request of the ongoing session (Correlation ID) to indicate the end of the procedure.

Proposal 3: Propose to introduce AIoT Device Identification Requested IE as a Choice Type IE with three branches (Device Identity list, Group Identity list and All devices)