3GPP TSG-RAN WG3 Meeting #128	             R3-253667

St Julian, Malta, 19-23 May 2025

Agenda Item:	20.2

Source:	CMCC

Title:	Support Direct AI/ML Positioning

Document for:	Discussions

# Introduction

In the RAN 3 127 Bis meeting, the following agreement for Case 3b has been made [1]:

Proposal 1, RAN3 agrees the following enhancements to support sample-based measurement configuration: - introduce new Channel Response Window Size IE and Channel Response Number IE in TRP Measurement Quantities Item.

# Discussions on Direct AI/ML Positioning

### Model Training

Agreement (RAN1#119)  [2]

Agreement - selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Agreement (RAN1#120)  [3]

Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24} Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. Conclusion For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

Based on the agreements made by RAN1 127 meetings that are quoted and highlighted above, for Case 3b, the measurements are obtained by the NG-RAN Node from TRPs and reported from the NG-RAN Node to the LMF. The measurements might be:

- Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

- Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

For sample-based measurement (if supported), from RAN1 perspective, LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa:

- Nt is the window size over which to collect samples.

- Nt' is the number of samples to report

- k is the resolution of the window, indicating the interval between samples.

For the sample-based measurement results, each measurement type corresponds to different results. RAN1 is discussing three types of measurements for each selected estimated channel response:

- DP (Delay Profile) contains the information of: (a) delay.

- PDP (Power Delay Profile) contains the information of: (a) delay, (b) power.

- CIR (Channel Impulse Response) contains the information of: (a) delay, (b) power and (c) phase.

For path-based measurement, it refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). The measurement parameters include: number of additional paths, k.

Additionally, from RAN1 perspective, the measurement report from the gNB to the LMF includes the following:

- (Mandatory) Channel measurement

- (Mandatory) Timestamp of the channel measurement

- (Optional) Quality of the channel measurement

RAN1 120Bis meeting made the following agreement:

Agreement For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved.

For sample-based measurement, a new “positioning measurement type” of “Channel Response” in the NRPPa Positioning Measurement Request is introduced.

For sample-based measurement result, a new “Channel Response” IE is introduced, which includes a list of channel response measurements,  Timing Reporting Granularity Factor, and Channel Response Window Size.

For each channel response measurement, the timing information is mandatory to be included, the power information and quality information are optional to be included.

For path-based measurements, the parameter value of k (number of paths) needs to be signaled from the LMF to the NG-RAN Node via NRPPa Positioning Measurement Request.

For path-based measurement, the timing information associated with the measurement needs to be reported with the measurement, while the quality is optionally.

If CU-DU split architecture is considered, Positioning Measurement procedure defined in F1AP would have the same impact as NRPPa impacts as proposed above.

### Model Inference

For LMF-side model inference, the UE or gNB can generate the necessary input data while the termination point for this input data lies within the LMF where the inference process is performed [4] [5].

If the necessary input data is generated from NG-RAN Node for LMF-side model inference, Measurement procedure defined in NRPPa is reused to support the channel measurement request and report.

If CU-DU split architecture is considered, Positioning Measurement procedure defined in F1AP is reused to support the channel measurement request and report for Case 3b to support direct model inference in LMF.

### Model Monitoring

In Case 3b, the LMF should be the entity responsible for deriving the monitoring metric, as outlined in [2]. The LMF gather measurements and estimated positions as labels, it is logical for the LMF to make the monitoring decisions. However, it is likely that NG-RAN node could provide further measurements to the LMF for the model monitoring purpose.

For Case 3b, LMF is the entity that is responsible for model monitoring.

NG-RAN node might provide further measurements to the LMF for the model monitoring purpose. RAN3 should wait for the RAN1 progress.

# Conclusions

In this submission, we discussed the direct AI/ML positioning cases and potential RAN 3 impact. We presented the following observations and proposals:

- For sample-based measurement, a new “positioning measurement type” of “Channel Response” in the NRPPa Positioning Measurement Request is introduced.

- For sample-based measurement result, a new “Channel Response” IE is introduced, which includes a list of channel response measurements,  Timing Reporting Granularity Factor, and Channel Response Window Size.

- For each channel response measurement, the timing information is mandatory to be included, the power information and quality information are optional to be included.

- For path-based measurements, the parameter value of k (number of paths) needs to be signaled from the LMF to the NG-RAN Node via NRPPa Positioning Measurement Request.

- For path-based measurement, the timing information associated with the measurement needs to be reported with the measurement, while the quality is optionally.

- If CU-DU split architecture is considered, Positioning Measurement procedure defined in F1AP would have the same impact as NRPPa impacts as proposed above.

- If the necessary input data is generated from NG-RAN Node for LMF-side model inference, Measurement procedure defined in NRPPa is reused to support the channel measurement request and report.

- If CU-DU split architecture is considered, Positioning Measurement procedure defined in F1AP is reused to support the channel measurement request and report for Case 3b to support direct model inference in LMF.

- For Case 3b, LMF is the entity that is responsible for model monitoring.

- NG-RAN node might provide further measurements to the LMF for the model monitoring purpose. RAN3 should wait for the RAN1 progress.

# References

Draft_MeetingReport_RAN3#127-bis_v20250419x

Draft_Minutes_report_RAN1#119_v030

Draft_Minutes_report_RAN1#120_v010

3GPP TS 38.455 V18.3.0 (2024-09), “NR Positioning Protocol A (NRPPa).”

3GPP TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.”