3GPP TSG-RAN WG3 Meeting #128	R3-253674

St. Julien's, Malta, 19 - 23 May, 2025

Title:	Discussion on Command Procedure

Agenda Item:	16.2

Source:	China Unicom

Document for:	Discussion

- Introduction

During RAN3#127bis meeting, the agreement about A-IoT Command procedure can be found as the following:

About Command Procedures: - Introduce a follow-on command indication in Inventory Request Transfer IE, in case of Command after inventory. - For command related NGAP messages, the Correlation Identifier IE is the same one as in the related Inventory procedure. - WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure. - WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages. - Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages. - The Command Request Transfer IE, also includes the following: - An A-IoT NAS PDU - Command Assistance Information: (Estimate of Expected D2R Message Size) - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…) ...... It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

In this contribution, we discuss open issues of A-IoT command operation.

- Discussion

The Command Type (e.g., Read/Write/Disable) information is useful for uplink (UL) resource scheduling.

- Read: requires substantial UL resources to accommodate data retrieval from devices.

- Write/Disable: only need minimal resources for acknowledgment (ACK/NACK).

By including the Command Type in the Command Assistance Information, the gNB can dynamically allocate resources based on the specific operation type. This avoids over-provisioning or under-provisioning resources, thereby improving spectral efficiency and reducing latency. Without explicit Command Type indication, the gNB would need to adopt a conservative inefficient approach, leading to inefficient resource utilization.

The inclusion of Command Type can be made optional, allowing the AIOTF to provide it only when the AF explicitly specifies the operation type. If omitted, the gNB can default to legacy scheduling mechanisms, ensuring backward compatibility.

Proposal 1: The Command Assistance Information includes the Command Type (read, write, disable).

Ambient IoT scenario often involve managing hundreds of devices simultaneously. In case of multiple devices (group of devices or all devices) operation (inventory and commond), it should be supported that parallel transactions between the gNB and AIOTF for the command transmissions for different devices.

Without a mechanism to distinguish transactions for different devices, the gNB cannot efficiently handle parallel Command Requests. For example, if multiple devices are targeted with "Read" commands, the gNB must uniquely associate responses with their respective requests to avoid misdelivery.

Proposal 2: Introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

For introducing Device Association or a transaction ID, both options are feasible.

- Device Association: Leverages existing mechanisms (e.g., RAN Device NGAP ID), it requires minimal additional standardization impact.

- Transaction ID: A lightweight integer field added to Command Request/Response messages, it requires negligible signaling overhead.

- Conclusion

In this paper, the following proposals are made:

Proposal 1: The Command Assistance Information includes the Command Type (read, write, disable).

Proposal 2: Introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

- Reference

[1] RAN3 #127-bis Meeting Report, Wuhan, China