3GPP TSG-RAN WG3 Meeting #128	R3-253680

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	AI/ML based Mobility Optimization for NR-DC

Source:	Jio Platforms (JPL), Deutsche Telekom, FiberCop

Agenda item:	11.4

Document Type:	Discussion and agreement

# Introduction

At RAN3#127-bis, what was captured in the meeting minutes follows:

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked.

The following list of exit conditions to stop the collection of UE performance at SN side:

- the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE;

- the NR-DC with the S-NG-RAN node for the UE is released.

UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture?

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In this TDoc submission, further discussions are proposed based on above agreements, to add and open points at the end.

# Mobility Optimization for NR-DC

## AI/ML-based MN Mobility Optimization Support

The evolution of dual connectivity (DC) mechanisms in 5G New Radio (NR) networks offers a promising avenue to deliver improved mobility robustness, throughput gains, and seamless user experience, especially in heterogeneous and high-mobility environments. As specified in 3GPP TR 38.743, the application of Artificial Intelligence and Machine Learning (AI/ML) methods to mobility optimization in NR-DC has emerged as a key enhancement area. This technical contribution builds upon the foundational statement in TR 38.743 (Clause 5.1.1), which emphasizes the feasibility of optimizing NR-DC mobility through AI/ML, with inference anchored at the Master Node (MN).

Observation 1:  Following 3GPP TR 38.743 (Clause 5.1.1) NR-DC mobility can be optimized through AI/ML with inference anchored at the Master Node (MN) in order to support UE performance metrics.

Hence MN can serve as the centralized inference point in a distributed RAN architecture, leveraging its coordination role to collect, process, and act upon real-time and historical mobility-related data from both the Secondary Node (SN) and the User Equipment (UE). The MN, being the anchor for RRC signaling and bearer control in NR-DC, is strategically positioned to host the AI/ML inference. This function can analyze mobility patterns, radio link metrics (i.e. RSRP, RSRQ, SINR), UE mobility history, handover success rates, dual connectivity activation/deactivation patterns, and QoS satisfaction metrics to predict and optimize SN addition and release decisions.

Observation 2:  The Master Node (MN), due to its role as the RRC and bearer control anchor in NR Dual Connectivity (NR-DC), is optimally positioned to host AI/ML-based inference for mobility optimization.

By localizing inference at the MN, latency in mobility-related decision-making is minimized, and decisions are harmonized across cells and interfaces that the MN manages. The inference outcome may directly influence the triggering conditions and thresholds for initiating dual connectivity (e.g., SN addition based on predicted throughput gain or mobility robustness improvement), as well as the selection of optimal SN candidates. This predictive capability is especially beneficial in scenarios involving UEs with high velocity, irregular mobility patterns, or in environments with dynamic interference conditions (e.g., urban macro deployments or cross-frequency DC).

Observation 3:  The MN infers SN addition or change based on the anticipated performance benefit to the UE, aligning the inference-driven actions to the DC activation with actual traffic distribution and service needs.

Proposal 1: It is proposed that the MN leverage AI/ML inference to estimate whether the SN can effectively serve a portion of the UE’s traffic. This prediction should quantify potential UE performance gains, guiding informed SN addition decisions in NR-DC.

We further propose to focus on enhancing baseline NR-DC operation through AI/ML without altering the standardized RRC procedure flows. This allows for non-intrusive integration of intelligence into existing network functions, aligning with implementation realities and operator deployment strategies.

Proposal 2:  Focus on enhancing baseline NR-DC operation through AI/ML without altering the standardized RRC procedure flows.

## SN Predicted Performance Role in NR-DC configuration

A key application of AI/ML in NR Dual Connectivity (NR-DC) is to predict how NR-DC potential configurations impact UE performance, enabling the Master Node (MN) to selectively initiate DC only when meaningful performance improvements are expected. While some suggest that simply signaling the assumed PSCell to the Secondary Node (SN) is sufficient—drawing parallels to the predictive UE trajectory support introduced for single connectivity in Rel-18—this approach lacks the depth required for effective mobility optimization in a dual connectivity context.

Observation 4:  relying solely on PSCell signaling is insufficient for intelligent DC activation. AI/ML-based predictions should guide both PSCell selection and traffic distribution to fully realize NR-DC performance gains.

Unlike single connectivity, where traffic terminates at a single NG-RAN node, NR-DC involves flexible traffic distribution between the MN and SN. Thus, the effectiveness of DC depends not only on the PSCell selection but also on the extent to which the SN contributes to serving UE traffic. If the MN’s AI/ML-based decision assumes a certain traffic load to be handled by the SN but this traffic is not actually offloaded, the performance gains anticipated by the MN may not materialize and may even degrade overall UE experience.

Observation 5: for NR-DC to be effective, both PSCell selection and traffic offloading to the SN must align with AI/ML predictions. Misalignment between expected and actual traffic handling can lead to suboptimal performance and reduced UE experience.

Therefore, limiting assistance information to just the PSCell assumption provides insufficient context for the SN to configure an optimal DC setup. To enable effective resource allocation and traffic steering, the SN must be informed not only of the predicted PSCell but also of the expected traffic volume it is meant to handle, based on the MN’s inference. This added intelligence would allow the SN to align with the AI/ML-driven intent behind the DC configuration and realize the intended UE performance benefits.

Proposal 3: SN be provided with both the predicted PSCell and the expected traffic volume to enable more informed DC configuration. This additional information will ensure that the SN can effectively allocate resources and align with AI/ML-driven performance optimization.

Additionally the potential for inter-node data sharing, when permitted (e.g., limited measurement forwarding from SN to MN via Xn or F1 interfaces), improves the feature richness and accuracy of AI/ML inference. Privacy-preserving training methods and lightweight inference engines are proposed to ensure compatibility with real-time processing requirements and RAN performance constraints.

Observation 6:  AI/ML inference in MN can be enhaced only when inter-node data sharing is possible.

Actual performance improvements depend on the SN handling a meaningful share of UE traffic. Without conveying the expected traffic volume alongside PSCell information, the SN may be unable to apply an effective DC configuration. Therefore, it is essential that assistance information include both the anticipated PSCell and the traffic load forecasted by the MN.

Proposal 4: MN provides the SN with both the predicted PSCell and the estimated UE traffic volume to be served. This will enable the SN to configure dual connectivity more effectively and fulfill the AI/ML-optimized performance objectives.

# Conclusions

In summary, this contribution aligns with the current scope defined in TR 38.743 but extends the discussion by proposing a concrete AI/ML framework for MN-based mobility optimization in NR-DC. The objective is to provide enhanced service continuity, reduced mobility-related failures, and improved spectral utilization through proactive and intelligent dual connectivity decisions. The proposed enhancements maintain backward compatibility with existing NR-DC mechanisms while opening the door for future standardization and experimentation within the AI/ML-based RAN optimization landscape

Based on this TDoc discussion, following observations and proposals are proposed:

Observation 1:  Following 3GPP TR 38.743 (Clause 5.1.1) NR-DC mobility can be optimized through AI/ML with inference anchored at the Master Node (MN) in order to support UE performance metrics.

Observation 2:  The Master Node (MN), due to its role as the RRC and bearer control anchor in NR Dual Connectivity (NR-DC), is optimally positioned to host AI/ML-based inference for mobility optimization.

Observation 3:  The MN infers SN addition or change based on the anticipated performance benefit to the UE, aligning the inference-driven actions to the DC activation with actual traffic distribution and service needs.

Proposal 1: It is proposed that the MN leverage AI/ML inference to estimate whether the SN can effectively serve a portion of the UE’s traffic. This prediction should quantify potential UE performance gains, guiding informed SN addition decisions in NR-DC.

Proposal 2:  Focus on enhancing baseline NR-DC operation through AI/ML without altering the standardized RRC procedure flows.

Observation 4:  relying solely on PSCell signaling is insufficient for intelligent DC activation. AI/ML-based predictions should guide both PSCell selection and traffic distribution to fully realize NR-DC performance gains.

Observation 5: for NR-DC to be effective, both PSCell selection and traffic offloading to the SN must align with AI/ML predictions. Misalignment between expected and actual traffic handling can lead to suboptimal performance and reduced UE experience.

Proposal 3: SN be provided with both the predicted PSCell and the expected traffic volume to enable more informed DC configuration. This additional information will ensure that the SN can effectively allocate resources and align with AI/ML-driven performance optimization.

Observation 6:  AI/ML inference in MN can be enhaced only when inter-node data sharing is possible.

Proposal 4: MN provides the SN with both the predicted PSCell and the estimated UE traffic volume to be served. This will enable the SN to configure dual connectivity more effectively and fulfill the AI/ML-optimized performance objectives.