3GPP TSG-RAN WG3 Meeting #128	R3-253733

Malta, MT, 19th May– 23th  May 2025

Agenda Item:       10.2

Source:     ZTE Corporation

Title: 	(TP for SON BLCR for 38.401, 38.473, 38.413, 37.340, and 38.423)Discussion on MRO

Document for:	Discussion and Approval

1	Introduction

This contribution provides  a discussion on left issues on MRO for LTM, CHO with SCG(s), and subsequent CPAC.

2	MRO for LTM

2.1 NW solution for LTM failure/near failure caused by wrong beam

RAN3 received an LS (R3-251514) on SON for LTM from RAN2 at the last meeting. In this LS, RAN2 requested RAN3 to define a network based solution to address near failure caused by BFR shortly after a successful LTM (Case 1), and asked whether this solution could also apply to LTM failure due to wrong beam (Case 2). We provide solution and TPs in [1].

2.2 RACH-less LTM

In the last meeting, the issue on TA acquisition was discussed. we provide solution and TPs in [2]

2.3 SHR handling

The existing T310, T312, and T304 timers can be reused as SHR triggers in LTM near-failure scenarios. Following current MRO principles, upon receiving the SHR, the CU would first analyze the cause of the near-failure and then forward the SHR to the source DU if triggered by T310 or T312 (Configured at source DU), or to the target DU if triggered by T304 (Configured at target DU). We suggest to capture this SHR handling approach  in TS 38.401 stage 2.

Proposal 1: To capture the SHR handling approach in TS 38.401 stage 2. (The corresponding TP is provided in Annex A.)

2.4 Ping-Pong Avoidance

In the previous RAN3 meetings, the issue on ping-pong optimization for subsequent LTM was discussed.

The progress from the RAN3#126 meeting:

CU can adjust the candidate Cell list for LTM based on UHI.

DU should be able to resolve LTM ping-pong based on information over F1.

CU sends UHI to target DU (always or only when ping pong is detected)? CU sends an indication (new IE in existing message, new message, …) to DU(s) that LTM ping-pong has been detected?

Include HO type to UHI entries?

CU filters out L3 HOs from the UHI sent to DU?

Other additional information in UHI?

The progress from the RAN3#126 bis meeting:

For intra-CU case, CU detects the potential LTM ping-pong issue based on local information, DU decides and solves the LTM ping-pong issue.

RAN3 to discuss which option to be agreed for UHI and ping-pong issue:

Option 1: CU sends the UHI to DU upon successful LTM cell switch/UHI is updated, e.g. via the CU-DU CELL SWITCH NOTIFICATION message.

Option 2: CU sends the UHI to DU when LTM ping-pong is detected, e.g. via the ACCESS AND MOBILITY INDICATION message.

Option 3: CU sends an indication to DU that LTM ping-pong has occurred, e.g. via the CU-DU CELL SWITCH NOTIFICATION message.

Option 4: CU filters the LTM related UHI, and sends to DU via UE Context Setup Request, UE Context Modification Request or CU-DU CELL SWITCH NOTIFICATION.

Option 5: No need to send UHI from CU to DU.

According to current TS38.300, see below:

TS38.300 The source NG-RAN node collects and stores the UE History Information for as long as the UE stays in one of its cells. The resulting information is then used in subsequent handover preparations by means of the Handover Preparation procedures over the NG and XN interfaces, which provide the target NG-RAN node with a list of previously visited cells and associated (per-cell) information elements. The Handover Preparation procedures also trigger the target NG-RAN node to start collection and storage of UE history Information and thus to propagate the collected information.

Observation 1 : In legacy L3 HO, the UHI is always sent to the target node during each handover:

- To enable real-time optimization of future handovers at the target node , i.e., the target cell selection, PingPong avoidance.

- To trigger the target node to begin collecting UHI and propagate it to the next node.

Since the target cell is always aware of the UHI in legacy handovers, following the same principle, it is essential to provide UHI to the target DU during each LTM cell switch. This ensures that the target DU has access to relevant historical mobility data, enabling it to make more better real-time decisions for future LTM cell switch, not only to prevent ping pong but also to optimize target cell selection.

Furthermore, as local AI is expected to play a significant role in 5GA and 6G, UHI will be essential for improving AI/ML based selection of the optimal target cell. Restricting the provision of UHI to the target DU would limit its ability of AI-driven optimizations.

Observation 2 : As local AI is expected to play a significant role in 5GA and 6G, always sending the UHI to the target DU in the LTM cell switch can enhance the performance of local AI/ML-based selection of the optimal target cell.

Although ping-pong optimization involves both the source and target cells, since the UHI is always sent to the target DU during each LTM cell switch, all nodes involved in the LTM cell switch sequence (except for the initially accessed node) will receive the UHI. Therefore, always sending the UHI to the target DU does not prevent the source cell from performing ping-pong optimization. This approach is consistent with the legacy L3 handover, where the UHI is also delivered to the target node.

Observation 3:  Always sending the UHI to the target DU does not prevent the source cell from performing LTM ping-pong optimization. All nodes involved in the LTM cell switch sequence (except the initially accessed node) will receive the UHI.

During the RAN3 discussion, some companies expressed concerns that always sending the UHI to the target DU may increase signaling load over the F1 interface. We believe this issue can be solved through implementation. For example, during a prolonged connection period with frequent LTM cell switches, the CU may, upon detecting F1 congestion, then forward only the  partial UHI records from the most recent period to the target DU, rather than the full UHI.

Observation 4: The potential F1 signaling overhead during frequent LTM cell switches can be avoided through implementation, For example, the CU may, upon detecting F1 congestion, then forward only the  partial UHI records from the most recent period to the target DU, rather than the full UHI.

Based on above observation, we provide the following proposal.

Proposal 2: Following the legacy L3 HO principle, the CU should always send the UHI to the target DU during each LTM cell switch.

Based on the inter gNB-DU LTM procedure in following figure, we propose including UHI in Step 20. When the CU sends the CU-DU cell switch notification to the target DU, it may carry the UHI to enable the target DU to perform real-time optimization for subsequent LTM cell switch.

Proposal3 : the gNB-CU sends the UHI to the target gNB-DU via the CU-DU CELL SWITCH NOTIFICATION.

| Step | Source cell | Target Cell | Handover type |
| --- | --- | --- | --- |
| 1 | cell 1 | cell 2 | L3 HO |
| 2 | cell 2 | cell 3 | L3 HO |
| 3 | cell 3 | cell 4 | LTM |

Consider a UE moving through the following sequence of cells (Cell 1 is the initial accessing cell):

Original UHI  is (Cell1, cell2, cell3, cell4) without filtering, can be interpreted as following cell handover sequence:

(Cell 1 → Cell 2)

(Cell 2 → Cell 3)

(Cell 3 → Cell 4)

Now, if L3 HO events are filtered out and only stayed cell caused LTM cell switch in are retained, the filtered UHI (Cell1, cell4) would be interpreted as following:

(Cell 1 → Cell 4)

Clearly, the filtered UHI creates a gap, leading to incorrect pairing of the source and target cells during the handover process.

Proposal 4: Filtering out L3 HOs from the UHI creates gaps in the UHI, leading to misinterpretation of source-target cell pairs. It is proposed to include a handover type (indicating the type of cell handover in) in UHI entries while keeping the UHI is not filtered.

Based on above discussion, we provide stage 2  TP for TS38.401, and stage3 TPs for TS38.413 and TS38.473, as provided In annex B, C, D.

Proposal 5: For LTM Ping-pong avoidance, RAN3 to agree  the Stage 2 TP for TS38.401, and the Stage 3 TPs for TS38.413 and TS38.473, as provided in the annex B, C, D.

3	MRO for CHO with SCG(s)

3.1 Leftover issue

After RAN3#127bis meeting, there are some open issues left, which need to be further discussed, as below:

MRO for CHO with Candidate SCG(s): No additional information is needed in handover report message. Check stage 3 TP on the Target cell CGI related in handover report message. One SCGFailureInfomation is included in SCG Failure Indication message, no need aggregation. Respective MN in stage2 is at least a candidate MN. Information needed in the SCG Failure Indication message: The Xn UE AP ID in the receiving node. Additional information reported from the UE in SCGFailureInformation: The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered. No support of near failure case in R19. Whether Handover Report message is needed for case 7a? In addition to SCGFailureInformation, information needed in the SCG Failure Indication message? - CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?) - Suitable PCell ID

### 2.1.1 Whether Handover Report message is needed for case 7a?

During the RAN3 # 126 the following was agreed:

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

The view of last serving MN may need to send message to the candidate MN(s) which may need optimization applies also for case 7a.

There may be a need to forward the RLF report also in case not relevant to too late CHO execution. The case 7a when CHO execution conditions fulfilled first then with some delay the CPA/CPC execution conditions are also fulfilled consequently the CHO execution is unsuccessful while CPA/CPC execution is successful, which may be caused by delayed fulfilment of CPA/CPC execution conditions which may require optimization on CPA/CPC related part. The RLF report received by source (serving) MN may also need to be forwarded to candidate target MN.

For too early HO case of 7a, since the RLF happened in the target Node, the RLF report will at first send to the candidate target MN, subsequently the target MN may provide Handover report to the source Node. In this case, the RLF report has already received in the candidate target MN then based on the information in the RLF report, the candidate target MN can do optimization of delay of CPA/CPC execution. There is no extra need for source Node to send a Handover report for candidate target MN.

For wrong cell HO case of 7a, since the RLF happened in the target Node, the RLF report will at first send to the candidate target MN, subsequently the target MN may provide Handover report to the source Node. In this case, the RLF report has already received in the candidate target MN then based on the information in the RLF report, the candidate target MN can do optimization of delay of CPA/CPC execution. There is no extra need for source Node to send a Handover report for candidate target MN.

Based on above analysis,  Handover Report message from source MN to candidate MN is not needed for case 7a

Proposal 6: Handover Report message from source MN to candidate MN is not needed for case 7a.

### 2.1.2 Additional information for SCG failure indication?

In addition to SCGFailureInformation, the following information needed in the SCG Failure Indication message

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

- Only in user Case 1b 2b 3b of tool late CPC execution,  the source MN received SCGFailureInformation from the UE and forwards it to the candidate MN. Since the SCGFailureInformation includes failedPSCellId 和 MeasResultFreqList and candidate MN has the knowledge of CPAC configuration of the UE, the candidate MN can deduce the CPC failure type and suitable PSCell ID, there is no need to provide duplicated information from source MN.

Proposal 7：No further information in addition to SCGFailureInformation in the SCG Failure Indication message.e.g CPC failure type Suitable PSCell ID.

## 2.2 Stage 2 updates for CHO with candidate SCGs

- Some stage2 description for CHO with candidate SCGs in TS37.340 is not accurate and need further updates.

### 10.18.3 Conditional PSCell addition or change failure One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows: - Too Late CPC Execution: UE receives CPC configuration/S-CPAC configuration/CHO with Candidate SCG(s) configuration, while a SCG failure occurs before CPC execution; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE. - Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE. - CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. For Too late CPC execution and CPC/CPA execution to wrong PSCell, there are two sub-cases for wrong candidate PSCell list selection: - if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC/S-CPAC/CHO with Candidate SCG(s) or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN; - else, it is wrong candidate PSCell list selection at the node initiating the CPC/S-CPAC/CHO with Candidate SCG(s) or at the MN initiating the CPA. In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

- In current running TP for TS37.340, the definition of the node initiating for CHO with Candidate SCG(s) is not accurate. Either in case of too late CPC execution or CPC/CPA execution to wrong PSCell, it is candidate MN/target MN prepare the CPAC configuration for example event trigger, candidate PSCell list. While the candidate MN is not the Node who trigger the who procedure. It is noted in case of CHO with candidate SCG(s), the source MN provides the maximum number of conditional reconfigurations that the candidate MN can prepare for the UE in the Handover Request message.With out the trigger from source MN, the candidate MN can not provide candidate PSCell list to the candidate SN node. Therefore Node provide the candidate target PSCells is different from the Node triggering CHO with candidate SCG(s) procedure.

Proposal 8: For TP for TS 37340, to correct the MN node of wrong candidate PSCell list selection for CHO with candidate SCG(s) as candidate or target MN provided CHO with Candidate SCG(s).

An example of updates is shown as below:

For Too late CPC execution and CPC/CPA execution to wrong PSCell, there are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC/S-CPAC, or by the candidate or target MN provided CHO with Candidate SCG(s) configurations or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC/S-CPAC or at the candidate or target MN provided CHO with Candidate SCG(s) configuration or at the MN initiating the CPA.

Another part need updates in TP for 37340 is detection mechanism:

Detection mechanism The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated. The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA or S-CPAC procedure: - Too Late CPC Execution: if CPC/S-CPAC/CHO with Candidate SCG(s) is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence. - Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found. - CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

- For CHO with candidate SCG user case 7b/8b, the current description is enough. It is the target MN received SCGFailureInformation  from the UE and performs the initial analysis.

- However the description is not accurate for CHO with candidate SCG user case 1b/2b/3b, the SCGFailureInformation  is not received from the UE , the information received from the source MN before CHO and/or CPAC execution. Instead of source MN, it is candidate MN node receiving SCGFailureInformation perform analysis for CPAC error detection. In order to mitigate misunderstanding, it is necessary to add a separate paragraph to describe the MN detection failure CHO with candidate SCG cases.

Proposal 9: For TP for TS 37340, to adding description for candidate or target MN detection the failure when the SCGFailureInformation  message received from the source MN.

An example of update is as following:

Detection mechanism The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated. The MN performs the analysis when SCGFailureInformation is received from the source MN in case of Too Late CPC Execution with candidate SCG(s). The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA or S-CPAC procedure or the NG-RAN node as candidate MN prepare CHO with Candidate SCG(s): - Too Late CPC Execution: if CPC/S-CPAC/CHO with Candidate SCG(s) is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

## 2.3 Stage 3 updates

- Semantic description update for XnAP HANDOVER REPORT message:

- It was agreed to reuse HANDOVER REPORT to in case of too late CHO execution, reusing this message from the last serving MN to the relevant candidate MN.

- The message was used for the case of HO too early, HO to wrong cell, Inter-system ping-pong which means the NG-RAN node 1 is the target Node in previous handover.

- However the direction of the message is opposite in case of Too Late CHO with candidate SCG. In this case, NG-RAN node 2 is the target Node in previous handover.

Take Handover cause IE as an example, the source Node is NG-RAN node 1 instead of NG-RAN node 2.

| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2. If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it indicates the handover cause employed for handover from NG-RAN node 1. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 10: To update semantic description of HANDOVER REPORT message if Handover Report Type is set to “Too Late CHO with candidate SCG”.

4	MRO for SCPAC

After the RAN3#127bis meeting, there is an open issue on enhanced UHI in the case of S-CPAC , as below:

MRO for S-CPAC The description on S-CPAC information forwarding is technically correct. Cleanup of the text is needed. Whether to enhance UHI in case of S-CPAC? The motivation and benefits need to be clarified.

The issue was raised of whether to support forwarding the SCG UHI to the target node during the S-CPAC procedure to avoid ping-pong immediately. However, we believe this type of real-time optimization introduces significant complexity of SCPAC reconfiguration at network side and  is not aligned with the principles of MRO, which focuses on long-term optimization. Additionally, it would require modifications to the UE's SCPAC configuration whenever a ping-pong is detected. From our perspective,  the real-time reconfiguration of SCPAC to avoid ping-pong is not necessary.

Proposal 11:  The real-time reconfiguration of SCPAC to avoid ping-pong is not necessary.

- 5 Conclusion:

The following observations and proposals are provided for MRO for LTM:

Proposal 1: To capture the SHR handling approach in TS 38.401 stage 2. (The corresponding TP is provided in Annex A.)

Observation 1 : In legacy L3 HO, the UHI is always sent to the target node during each handover:

- To enable real-time optimization of future handovers at the target node , i.e., the target cell selection, PingPong avoidance.

- To trigger the target node to begin collecting UHI and propagate it to the next node.

Observation 2 : As local AI is expected to play a significant role in 5GA and 6G, always sending the UHI to the target DU in the LTM cell switch can enhance the performance of local AI/ML-based selection of the optimal target cell.

Observation 3:  Always sending the UHI to the target DU does not prevent the source cell from performing LTM ping-pong optimization. All nodes involved in the LTM cell switch sequence (except the initially accessed node) will receive the UHI.

Observation 4: The potential F1 signaling overhead during frequent LTM cell switches can be avoided through implementation, For example, the CU may, upon detecting F1 congestion, then forward only the  partial UHI records from the most recent period to the target DU, rather than the full UHI.

Proposal 2: Following the legacy L3 HO principle, the CU should always send the UHI to the target DU during each LTM cell switch.

Proposal3 : the gNB-CU sends the UHI to the target gNB-DU via the CU-DU CELL SWITCH NOTIFICATION.

Proposal 4: Filtering out L3 HOs from the UHI creates gaps in the UHI, leading to misinterpretation of source-target cell pairs. It is proposed to include a handover type (indicating the type of cell handover in) in UHI entries while keeping the UHI is not filtered.

Proposal 5: For LTM Ping-pong avoidance, RAN3 to agree  the Stage 2 TP for TS38.401, and the Stage 3 TPs for TS38.413 and TS38.473, as provided in the annex B, C, D.

The following proposals are provided for MRO for CHO with SCG(s):

Proposal 6: Handover Report message from source MN to candidate MN is not needed for case 7a.

Proposal 7：No further information in addition to SCGFailureInformation in the SCG Failure Indication message.e.g CPC failure type Suitable PSCell ID.

Proposal 8: For TP for TS 37340, to correct the MN node of wrong candidate PSCell list selection for CHO with candidate SCG(s) as candidate or target MN provided CHO with Candidate SCG(s).

Proposal 9: For TP for TS 37340, to adding description for candidate or target MN detection the failure when the SCGFailureInformation  message received from the source MN.

Proposal 10: To update semantic description of HANDOVER REPORT message if Handover Report Type is set to “Too Late CHO with candidate SCG”.

The following proposals are provided for MRO for SCPAC:

Proposal 11:  The real-time reconfiguration of SCPAC to avoid ping-pong is not necessary.

- Reference

- R3-253734 (TP for SON BLCR for 38.300, 38.401, and 38.473) LTM failure and near failure caused by wrong beam, ZTE

- R3-253735 (TP for SON BLCR 38.300 and38.473)MRO for LTM- invalid TA, ZTE

Annex A: 	TP for BLCR for TS38.401 for LTM SHR handing

<< Start of Change >>

7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

The gNB-CU receives a SHR report associated to an LTM mobility event from the UE or via the Access and Mobility message over Xn or via Downlink RAN configuration transfer procedure over NG. The gNB-CU may forward the SHR report to the source gNB-DU  in case of the SHR triggered by the T310 and T312 timer,or to the target gNB-DU in case of the SHR triggered by the T304 timer.

<< End of Change >>

Annex B: 	TP for BLCR for TS38.401 for LTM PingPong Avoidance

<< Start of Change >>

7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

In case of LTM cell switch, the gNB-CU collects and stores the UE History Information for as long as the UE stays in one of its cells. The gNB-CU may send UE history information to the target DU for the subsequent LTM cell switch, when notifying the target DU about the current initiation of the LTM cell switch command to the UE.

<< End of Change >>

Annex C: TP for BLCR for TS38.473 for LTM PingPong Avoidance

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

8.3.10	CU-DU Cell Switch Notification

8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the TA Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Tag ID Pointer IE is also included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

If the UE History Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, store the collected information and use it for future LTM cell switch preparation.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| UE History Information | O |  | OCTET STRING | UE History Information, as same definition in TS 38.413 [3]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

Annex D: TP for BLCR for TS38.413 for LTM PingPong Avoidance

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| HO Type | O |  | ENUMERATED ( LTM, …) | The handover type for UE accessing the indicated stayed cell. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8. |

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>

Annex E: TP for BLCR for TS37.340 for CHO with SCG(s)

10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration/S-CPAC configuration/CHO with Candidate SCG(s) configuration, while a SCG failure occurs before CPC execution; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE.

For Too late CPC execution and CPC/CPA execution to wrong PSCell, there are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC/S-CPAC, or by the candidate or target MN provided CHO with Candidate SCG(s) configurations or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC/S-CPAC or at the candidate or target MN provided CHO with Candidate SCG(s) configuration or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

Detection mechanism

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated.

The MN performs the analysis when SCGFailureInformation is received from the source MN in case of Too Late CPC Execution with candidate SCG(s).

The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA or S-CPAC procedure or the NG-RAN node as candidate MN prepare CHO with Candidate SCG(s):

-	Too Late CPC Execution: if CPC/S-CPAC/CHO with Candidate SCG(s) is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

-	Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found.

-	CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

Annex F: TP for BL CLR for TS38.423 for CHO with SCG(s)

9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …, Too Late CHO with candidate SCG) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2. If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it indicates the handover cause employed for handover from NG-RAN node 1. | YES | ignore |
| Source cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2). If the Handover Report Type is set to “Too Late CHO with candidate SCG”,it indicates NG-RAN CGI of source cell for handover procedure (in NG-RAN node 1) | YES | ignore |
| Target cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to “Inter-system ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cell. If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it is the suitable cell in the NG-RAN node 2. | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | Global Cell Identity 9.2.2.73 | CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the failure | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message or in the SN STATUS TRANSFER message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | UE RLF Report 9.2.2.59 | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| CHO Configuration | O |  | 9.2.2.76 |  | YES | ignore |
| Target cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the target NG-RAN node. | YES | ignore |
| Time Since Failure | O |  | INTEGER (0.. 172800, ...) | Corresponds to the TimeSinceFailure IE received from the UE in RLF Report as defined in TS 36.331 [14]. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.3.xx	SCG Failure Indication

This message is sent by NG-RAN node1 to NG-RAN node2 to report a SCG failure event that occurred while CHO with candidate SCG was configured.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the candidate target NG-RAN node | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message as defined in TS 38.331 [10] | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- SCG FAILURE INDICATION

--

-- **************************************************************

SCGFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SCGFailureIndication-IEs}},

...

}

SCGFailureIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TargetNG-RANnodeUEXnAPID	CRITICALITY ignore      	TYPE NG-RANnodeUEXnAPID				PRESENCE mandatory}|

{ ID id-SCGFailureReportContainer   CRITICALITY ignore		TYPE SCGFailureReportContainer		PRESENCE mandatory},

...

}