# 3GPP TSG-RAN WG3 Meeting #128	                                                          R3-253735

# Malta, MT, 19th-23nd May 2025

Agenda item:	10.2

Source:	ZTE Corporation

Title:	(TP for SON BLCR 38.300, 38.401 and 38.473)MRO for LTM outdated TA and TA acquisition type

Document for:	Discussion

1 Introduction

In  this contribution, we only focus on the following MRO issue:

1: How to optimize the “Invalid TA”issue.

2: Network based or UE based solution for TA acquisition type

The related meeting notes except as below:

RAN3#127bis： MRO for LTM: LTM failure due to outdated TA MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM. RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE. For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell. Whether UE based solution or network based solution is used for LTM failure due to outdated TA? Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case? Network based or UE based solution for TA acquisition type For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3. FFS for intra-DU case. RAN3#127： MRO for LTM: For RACH-less optimization: -RAN3 to optimize the “Invalid TA” in outdate case? MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM? Scenario needs to be further clarified (e.g. outdated TA, re-establishment in target cell, etc…). Network based or UE based solution for LTM candidate cell list? Network based solution is used for LTM candidate cell list. Network based or UE based solution for TA acquisition type? Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context? The agreements achieved in RAN3 #126 [1]:

The corresponding RAN 2 meeting minute:

The agreements achieved in RAN2 #128 [3]: - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues. - Reuse the existing ra-InformationCommon for the RA-based LTM failure. - If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF. The agreements achieved in RAN2 #127bis [4]: - The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure. - If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD. - Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). - Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation. - Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell. - UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

2 Discussion

2.1 Network based or UE based solution for TA acquisition type

RAN3 has agreed that MRO will cover the scenario that RACH-less LTM fails including outdated TA and UE performs RACH based LTM, following issue is FFS.

Network based or UE based solution for TA acquisition type For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3. FFS for intra-DU case.

- According to current specification, for inter-DU case, after DU trigger LTM cell switch for the UE, it will send  DU-CU CELL SWITCH NOTIFICATION message to the CU, and the TA Information List of the candidate cells is included. But for intra-DU case, only target cell ID  is included in the DU-CU CELL SWITCH NOTIFICATION message.

13. The gNB-DU sends the Cell Switch Command to the UE. 14. The gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID.

- Therefore, for intra-DU case, CU does not aware TA acquisition type of the UE.

- Observation 1: For intra-DU case, CU does not aware TA acquisition type of the UE.

- However, if the UE context has not been released in the source DU, the source DU can aware TA acquisition type of the UE.

- For intra-DU LTM cell switch case, since the source DU and the target DU are the same, if the source cell trigger a RACH-less LTM but the UE perform RACH based access with a different TA value to the same target cell, the DU can find this issue immediately and the UE context is still kept at that time, so even if the CU does not aware TA acquisition type of the UE, the source DU can know. Therefore, for intra-DU LTM cell switch case, TA acquisition type issue can be handled by the Network without any signaling impact in RAN3.

- Observation 2: For intra-DU LTM cell switch case, the source DU can aware TA acquisition type of the UE since the UE context has not been released  when the source DU find the TA issue.

- Proposal 1:  TA acquisition type issue can be handled by the Network without any signaling impact in RAN3.

2.2 Who does the optimization for outdated TA issue

According to current specification, for a UE configured with intra-CU LTM, if LTM cell switch is determined by the source DU, the source DU will send CELL SWITCH COMMAND MAC CE to UE and subsequently send DU-CU CELL SWITCH NOTIFICATION to the CU. If the source DU consider the UE  has a valid TA value for the target cell, it would include the TA value in the  CELL SWITCH COMMAND MAC CE. Then the UE will perform RACH-less LTM to the target cell. However, if the provided TA value is invalid (e,g, it is out date), the RACH-less LTM cell switch may fail since the target cell can not decode the UE's uplink messages which arrives outside the target cell's reception window caused by the incorrect TA. Therefore, the key issue is that how to determine the validity of the TA value which is  based on the judgement of source gNB DU. If the source DU keeps a wrong maintenance of the TA validity time, there might exist that the source gNB DU thinks the TA is still valid but actually not.

Therefore, the source DU should know whether the provided the TA value is valid or not and do the optimization for outdated TA issue.

Proposal 2: the source gNB DU should do the optimization for outdated TA issue.

2.3 Network based or UE based solution for outdated TA issue

- 2.3.1 UE-based solution

For a UE-based solution, when the UE detects RACH-less LTM cell switch failure, and followed by a successfully RACH-based access to the same target cell, it can store the difference between TA value for successful access and TA value used in failed RACH-less LTM in the RLF report.

However, RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE, and based on above analysis in the section 2.1, the source CU does not aware TA acquisition type of the UE for intra-DU case, and it is hard to say whether source DU still keeps UE context upon receiving RLF report, but the UE can know TA acquisition type used in the failed RACH-less LTM cell switch, therefore, one simple way is that the UE only store the difference between TA value for successful access and TA value in cell switch command in the RLF report. On the other word, if the TA value used in failed RACH-less LTM is calculated by UE, the UE will not store the TA related information in the RLF report.

In addition, since the UE context may have been released when receiving RLF report from the UE, the source DU do not know the configured validity time of TA at that time for the UE, so that it is hard for the source DU to do optimization for outdated TA issue. In other word, the source DU can not know how to adjust the validity time of TA value. Therefore, the UE may need to further report the time difference between receiving PDCCH order and performing RACH-less LTM in the RLF report. Based on this time difference information, the source DU can know the validity time of TA should be less than this reported time difference.

Observation 3: since the UE context may have been released when receiving RLF report from the UE, the source DU do not know the configured validity time of TA at that time for the UE. So that it is hard for source DU to know how to adjust the validity time of TA value.

Proposal 3: for UE based solution, the UE store the following TA information in the RLF report in case the TA value used in failed RACH-less LTM is acquired from the source DU.

- TA value used for successful access;

- TA value received in cell switch command;

- the time difference between receiving PDCCH order and performing RACH-less LTM.

For the node to identify this outdated TA issue, there are two options: gNB CU or source gNB DU. Based on previous agreement, the gNB CU need to do the analysis for too early LTM, too late LTM and wrong target cell selection issue upon receiving RLF report, even if let the gNB DU does the analysis for the outdated TA issue, the workload for gNB CU won’t decrease. To reduce the workload for gNB DU, we prefer gNB CU do the analysis for outdated TA issue, too.

Proposal 4: for UE based solution, gNB-CU do the analysis for outdated TA issue.

- 2.3.2 Network based solution

- For inter-DU LTM cell switch case:

- Upon receiving a RACH-less LTM cell switch command from the source cell, the UE shall immediately execute RACH-less LTM to the target cell. As we analyzed in the section 2.1, for inter-DU LTM cell switch case, the source DU will send TA Information List of candidate cells which includes the TA value sent to the UE to the source CU in the CELL SWITCH NOTIFICATION, then source CU forward this info to the target DU.

From the target cell's perspective, upon receiving a CELL SWITCH NOTIFICATION from the source DU via the source CU, it should anticipate the UE's imminent arrival and expect a RACH-less access if TA value associated to the target cell is included. If the target cell observes the UE performing a RACH-based access, it may suspect a RACH-less LTM failure. However, since RACH-less LTM failure may caused by wrong beam or by outdated TA, the network need to do further analysis. For the node to identify this outdated TA issue, there also are two options: gNB-CU or target gNB-DU.

Option 1: the gNB-CU do the analysis

- If the target gNB-DU receiving a CELL SWITCH NOTIFICATION from the source DU via the source CU and TA value associated to the target cell is included, but the UE performing a RACH-based access, then the target cell can send the RACH-based related info (e.g, the TA value used for successful RACH-based accessing ) to the gNB-CU in the ACCESS SUCCESS message. Since gNB-CU also know the TA value provided to the UE in the LTM cell switch command, it will compare the two TA values, if it find the TA value used in RACH-based accessing is much different from the one provided by the source gNB-DU, which means the TA value provided to UE is wrong, then without waiting for the subsequently RLF report from the UE, the gNB-CU can forward this RACH-based related info ( e.g, the TA value used for successful RACH-based accessing) to the source DU in the ACCESS AND MOBILITY INDICATION message, since this message is sent immediately after LTM switch, we can suppose the source DU still keep the UE context information, then the source DU can  do optimization.

Option 2: the target gNB-DU to do the analysis

- If the target gNB-DU find the TA value used in RACH-based accessing is much different from the one it received in the  CELL SWITCH NOTIFICATION, which means the TA value provided to UE is wrong, then without waiting for the subsequently RLF report from the UE, the target cell can send the RACH-based related info ( e.g, the TA value used for successful RACH-based accessing) to the gNB-CU in the ACCESS SUCCESS message, then the gNB-CU can forward this info to the source gNB-DU to do optimization.

Both options are workable. However, compared with gNB-DU, the processing ability of gNB-CU is more powerful. We prefer gNB-CU to do the analysis for outdated TA issue.

- Proposal5: for inter-DU LTM cell switch case, if the target gNB-DU receiving a CELL SWITCH NOTIFICATION from the source DU via the source CU and TA value associated to the target cell is included, but the UE performs a RACH-based access, the target gNB-DU shall send the TA information ( e.g, the TA value used for successful RACH-based accessing) to the gNB-CU in the ACCESS SUCCESS message.

- Proposal6:  for NW based solution, gNB-CU do the analysis for outdated TA issue and forward the TA info to the source DU if it considers TA is outdated.

- For intra-DU LTM cell switch case:

- For intra-DU LTM cell switch case, since the source DU and the target DU are the same, and the TA Information List of candidate cells will not be sent to the CU, if the source cell trigger a RACH-less LTM but the UE perform RACH based access with a different TA value to the same target cell, the DU can find this issue immediately, and the DU can decide how to do optimization by implementation without any signaling impact in RAN3.

- Proposal7: for intra-DU LTM cell switch case, outdated TA issue can be identified and optimized by DU implementation without any signaling impact in RAN3.

2.3.3 Comparison

|  | - UE based solution | - NW based solution |
| --- | --- | --- |
| - Impacts on spec | - Enhance RLF report | - Enhance ACCESS SUCCESS message |
| - Impacts on DU | - None | - Target DU may need to add TA value used for successful accessing in the ACCESS SUCCESS message. |
| - Impacts on UE | - Store TA information in the RLF report and sent to the NW. Besides TA value used for successful accessing, more information such as TA value received in cell switch command and time difference between receiving PDCCH order and performing RACH-less LTM are needed in the RLF report since the UE context may have been released by source gNB-DU when receiving RLF report. | - None |
| - Overhead on F1AP and Xn signalling | - For one failure event due to outdated TA, one F1AP message (ACCESS AND MOBILITY INDICATION) is needed. In case RLF report is received by other CU, Xn message is needed. | - For inter-DU LTM failure event due to outdated TA, one F1 message (ACCESS AND MOBILITY INDICATION) is needed. - For intra-DU LTM failure event due to outdated TA, no additional message. |
| - Pros | - Target DU can do nothing. | - Having no impacts on UE. - Providing faster response for the issue, since the source DU can acquire the assistant info to determine whether UL early sync should be optimized upon the RACH-less LTM cell switch is executed, without waiting for the UE RLF report which may be received long time later. - 3)For intra-DU LTM failure event due to outdated TA, no additional message is needed. |

Observation 4: Compared with UE based solution, the network based solution seems having following three advantages:

- Having no impacts on UE;

- Providing faster response for the TA issue, since the source DU can acquire the assistant info to determine whether UL early sync should be optimized upon the RACH-less LTM cell switch is executed, without waiting for the UE RLF report which may be received long time later;

- For intra-DU LTM failure event due to outdated TA, no additional message is needed.

Proposal 8: RAN3 to adopt network based solution for TA invalid issue.

3 Conclusion

- Observation 1: For intra-DU case, CU does not aware TA acquisition type of the UE.

- Observation 2: if the UE context has not been released in the source DU, the source DU can aware TA acquisition type of the UE.

Observation 3: since the UE context may have been released when receiving RLF report from the UE, the source DU do not know the configured validity time of TA at that time for the UE. So that it is hard for source DU to know how to adjust the validity time of TA value.

Observation 4: Compared with UE based solution, the network based solution seems having following three advantages:

- Having no impacts on UE;

- Providing faster response for the TA issue, since the source DU can acquire the assistant info to determine whether UL early sync should be optimized upon the RACH-less LTM cell switch is executed, without waiting for the UE RLF report which may be received long time later;

- For intra-DU LTM failure event due to outdated TA, no additional message is needed.

- Proposal 1:  TA acquisition type issue can be handled by the Network without any signaling impact in RAN3.

Proposal 2: the source gNB DU should do the optimization for outdated TA issue.

Proposal 3: for UE based solution, the UE store the following TA information in the RLF report in case the TA value used in failed RACH-less LTM is acquired from the source DU.

- TA value used for successful access;

- TA value received in cell switch command;

- the time difference between receiving PDCCH order and performing RACH-less LTM.

Proposal 4: for UE based solution, gNB-CU do the analysis for outdated TA issue.

- Proposal 5: for inter-DU LTM cell switch case, if the target gNB-DU receiving a CELL SWITCH NOTIFICATION from the source DU via the source CU and TA value associated to the target cell is included, but the UE performs a RACH-based access, the target gNB-DU shall send the TA information ( e.g, the TA value used for successful RACH-based accessing) to the gNB-CU in the ACCESS SUCCESS message.

- Proposal 6:  for NW based solution, gNB-CU do the analysis for outdated TA issue and forward the TA info to the source DU if it considers TA is outdated.

- Proposal 7: for intra-DU LTM cell switch case, outdated TA issue can be identified and optimized by DU implementation without any signaling impact in RAN3.

Proposal 8: RAN3 to adopt network based solution for TA invalid issue.

Annex A: TP to TS38.300

<< Start of Change >>

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

-	LTM cell switch failure due to outdated TA: an handover failure occurs during the RACH-less based LTM cell switch procedure which is triggered by network via sending the Cell switch Command with TA value to UE, the UE successfully attempts to re-connect to the same target cell with RACH-based access with new TA value.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover, handover to wrong cell and LTM cell switch failure due to outdated TA are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM or CHO with candidate SCG(s) is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

-	LTM cell switch failure due to outdated TA:  there is a recent RACH-less LTM cell switch for the UE triggered by network via sending the Cell switch Command with TA value to UE prior to the connection failure e.g. the UE successfully attempts to connect to the same target cell with RACH-based access with a new TA value provided from network in RAR message.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of Too Late Handover when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may inform the failure event to the candidate NG-RAN node(s) controlling the candidate PSCell where the mobility configuration caused the failure.

Editor’s note: FFS on the definition of failure event.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

<< End of Change >>

Annex B: TP for BL CR for TS38.401

<< Start of Change >>

7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

If the target gNB-DU receiving a CELL SWITCH NOTIFICATION from the source gNB-DU via the gNB-CU and TA value associated to the target cell is included, and the UE performing a RACH-based access successfully, the target gNB-DU may send the RACH-based related info (e.g, the TA value used for successful RACH-based accessing) to the gNB-CU in the ACCESS SUCCESS message. After the gNB-CU receives the TA value from the target gNB-DU, it performs initial analysis and may forward the LTM report to the source gNB-DU  in case of LTM cell switch failure due to outdated TA.

<< End of Change >>

Annex C: TP to TS38.473

<< Start of Change >>

8.3.8	Access Success

8.3.8.1	General

The purpose of the Access Success procedure is to enable the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover, conditional PSCell addition,conditional PSCell change,LTM, or subsequent CPAC. The procedure uses UE-associated signalling.

8.3.8.2	Successful Operation

Figure 8.3.8.2-1: Access Success procedure. Successful operation.

The gNB-DU initiates the procedure by sending a ACCESS SUCCESS message.

Upon reception of the ACCESS SUCCESS message, the gNB-CU shall consider that the UE successfully accessed the cell indicated by the included NR CGI IE in this gNB-DU and consider all the other CHO or conditional PSCell addition or conditional PSCell change preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU as cancelled. In case of subsequent mobility, the other preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU are kept.

If the TA Value IE is included in the ACCESS SUCCESS  message, the gNB-CU shall consider that the UE performs RACH-based access the cell indicated by the included NR CGI IE in this gNB-DU successfully, and may forward it to the source gNB-DU in the ACCESS AND MOBILITY INDICATION message to transfer the TA value(s).

Interaction with other procedure:

The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate gNB-DUs for this UE, if any.

<< Next Change >>

8.11.1	Access and Mobility Indication

8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Report Information List IE the gNB-DU shall take it into account for optimization of LTM cell switch related parameters.

8.11.1.3	Abnormal Conditions

Not applicable.

9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover, conditional PSCell addition, conditional PSCell change, LTM, or subsequent CPAC.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |
| TA Value | O |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | YES | ignore |

<<Next Change >>

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| LTM Report Information List |  |  |  |  | YES | ignore |
| >LTM Report Information Item |  | 1..<maxnoofLTMReports> |  |  |  |  |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |  |  |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |  |  |
| >>TA Value | O |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

<<Next Change >>

9.4	Message and Information Element Abstract Syntax (with ASN.1)

[snip]

9.4.4	PDU Definitions

<<skip unchanged part >>

LTMReportInformationList,

<<skip unchanged part >>

id-LTMReportInformationList,

<<skip unchanged part >>

-- **************************************************************

--

#### -- ACCESS SUCCESS ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

#### -- Access Success

--

-- **************************************************************

AccessSuccess ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ AccessSuccessIEs}},

...

}

AccessSuccessIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-NRCGI								CRITICALITY reject	TYPE NRCGI									PRESENCE mandatory	}|

{ ID id-TAValue		            CRITICALITY ignore	TYPE TAValue		PRESENCE optional	},

...

}

-- **************************************************************

--

#### --  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReportInformationList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional }|

{ ID id-LTMReportInformationList	    CRITICALITY ignore	TYPE LTMReportInformationList		PRESENCE optional },

...

}

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

[snip]

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-rLFReportFailureType

[snip]

LTMReportInformationList::= SEQUENCE (SIZE(1.. maxnoofLTMReports)) OF LTMReportInformationItem

LTMReportInformationItem ::= SEQUENCE {

gNB-CU-UE-F1AP-ID		    GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID		    GNB-DU-UE-F1AP-ID,

nRCGI						NRCGI,

tAValue                     INTEGER (0..4095)  OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { LTMReportInformationItem-ExtIEs} }	OPTIONAL,

...

}

LTMReportInformationItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

[snip]

RLFReportInformationList	::= SEQUENCE (SIZE(1.. maxnoofRLFReports)) OF RLFReportInformationItem

RLFReportInformationItem	::= SEQUENCE {

nRUERLFReportContainer		NRUERLFReportContainer,

uEAssitantIdentifier		GNB-DU-UE-F1AP-ID		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { RLFReportInformationItem-ExtIEs} }	OPTIONAL,

...

}

RLFReportInformationItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-C-RNTI				CRITICALITY ignore EXTENSION C-RNTI					PRESENCE optional },

{ID id-rLFReportFailureType	CRITICALITY ignore EXTENSION RLFReportFailureType		PRESENCE optional },

...

}

RLFReportFailureType ::= ENUMERATED{ toolateLTM, tooearlyLTM, lTMtowrongcell,...}

[snip]

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules

[snip]

maxnoofLTMReports						INTEGER ::= 64

[snip]

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-rLFReportFailureType								ProtocolIE-ID ::= 999 --to be assigned by MCC