3GPP TSG-RAN WG3 Meeting #128	R3-253744

Malta, MT, 19-23, May, 2025

Agenda Item:	8.1

Source:	Huawei

Title:	Response to R3-253163 Discussion on SRS Frequency Hopping

Document for:	Discussions & Approval

# 1	Introduction

This document is a response paper to R3-253163 Discussion on SRS Frequency Hopping.

The basis of the document is the R3-253163, the response uses the tracing mode change.

RAN2 replied the LS [1] (R2-2503185) to RAN1 on non-Redcap UE UL SRS frequency hopping for positioning, and also informed RAN3 to consider and determine whether there is any impacts to support non-Redcap UE SRS frequency hopping for positioning.

In this contribution, we share our analysis on the potential impacts and provide the corresponding CRs [2][3][4].

# 2	Discussion

In RAN1#120 meeting, RAN1 reached the following agreement for enabling non-RedCap UE performing UL SRS frequency hopping for positioning:

  - Extend Rel-18’s UL frequency hopping UL SRS for positioning transmission to non-RedCap UEs in a single carrier

  - UE capability for non-RedCap UEs for UL SRS frequency hopping for positioning transmission

Based on the agreement, RAN2 replied the LS including following agreements in the reply LS [1]:

Agreements: No new RRC parameter is needed for support of SRS frequency hopping for non-RedCap UE. This does not exclude a related capability, to be discussed in RAN1. The current Rel-18 restriction on maximum bandwidth per hop as 20MHz should be restricted to RedCap UE only, not to non-RedCap UE. Inform RAN1 that we expect this spec impact.

In Rel-18, the gNB could configure SRS frequency hopping only for RedCap UEs. Based on the recent agreements, it is now confirmed that SRS frequency hopping is extended to both RedCap and non-RedCap UEs in Rel-19.

- In Rel-19, SRS frequency hopping is supported for all UEs, including both RedCap and non-RedCap UEs.

In Rel-18 NRPPa signaling, a new explicit indicator was not introduced for the request of Tx hopping configuration because when the existing Bandwidth IE in the Requested SRS Transmission Characteristics IE exceeds the bandwidth capability of the RedCap UE, the serving gNB implicitly understands that Tx hopping configuration is needed. Now in Rel-19, SRS frequency hopping is supported for all UEs, including both RedCap and non-RedCap UEs. Therefore, requesting SRS frequency hopping based solely on the Bandwidth IE is no longer sufficient, as the bandwidth of non-RedCap UEs typically exceeds 20 MHz. Thus, a new explicit indication is needed to request Tx hopping configuration in Rel-19 scenarios.

Resp. We are still looking for the sense of the change, usually LMF should request some general characteristics of the SRS (e.g. overall bandwidth), and let gNB decides whether the bandwidth is achieved by hopping or not. We also do not undersand why the text above highlighted cannot apply for non-RedCap UE. Why for a non-RedCap UE the gNB cannot implicitly understands that Tx hopping configuration is needed??

Examples:

- A non-RedCap UE currently working on a smaller BWP of 20MHz for the sake of power saving, the gNB, upon reception of the request from LMF on a 100MHz SRS configuration, does not need to reconfigure or switch the BWP to a 100MHz one.

- A UE may be at the cell edge, where the UL is coverage limited. The gNB is well aware of the situation and configuring the SRS frequency hopping helpful for improving SNR.

In either example, we believe that the choice of frequency hopping is better decided by gNB itself rather than LMF.

- Introduce the SRS Frequency Hopping Request Indication IE in the Requested SRS Transmission Characteristics IE to enable LMF request Tx hopping configuration from gNB.

- Endorse the corresponding Tel-19 CRs to NRPPa[2] and F1AP[3]. And agree the reply LS[5] to RAN2 to inform RAN3’s decision.

Resp. Neither the coverpage, neither the discussion tdoc respond to the important question why NRPPa and LMF behaviour is different from Rel-18 RedCAP?

Without respond to this question and further explanation we do not believe any impact is needed over NRPPa, and the gNB is able like in Rel-18.

# 3	Conclusion

There is no evidence of any impact is needed over NRPPa, and the gNB is able, like in Rel-18, to support frequency hopping for both RedCap and non-RedCap UEs.  A draft LS should be send to RAN2 accordingly.

# 4	Reference

[1] R2-2503185, Reply LS on non-RedCap UE UL SRS frequency hopping for positioning, ZTE Corporation

[2] R3-253164, Correction on NRPPa for additional indication for the request of SRS frequency hopping, ZTE Corporation

[3] R3-253165, Correction on F1AP for additional indication for the request of SRS frequency hopping, ZTE Corporation

[4] R3-253166, Correction on F1AP for additional indication for the request of SRS frequency hopping, ZTE Corporation

[5] R3-253167, Reply LS on non-RedCap UE UL SRS frequency hopping for positioning, ZTE Corporation