# 3GPP TSG-RAN WG3 Meeting #128	R3-253928

# Malta, MT, 19 - 23 May, 2025

Agenda Item:	21.3

Source:	Ericsson, China Telecom, Huawei, Samsung, Nokia, Nokia Shanghai Bell, CATT

Title:	[TP to XR BL CR for 38.413] Update on XR NGAP issues

Document for:	Other

# 1	Introduction

This TP adds the following changes:

- Capture that the NG-RAN node ignores the PSDB and PSER in AQP by adding semantics description in the 9.3.1.151 Alternative QoS Parameters Set List.

- define available bitrate and threshold as a 32-bit integer with unit kbps.

- Use value “8” for maxnoofThresholds for available bitrate reporting

- Remove FFS on MMSID octet string size.

- Change “data rate” to “bitrate”

This annex below provides the TP to BLCR for TS38.413.

# TP to BL CR for TS 38.413

-----------------Start of the Changes-------------------

### 8.2.1	PDU Session Resource Setup

#### 8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU sessions and the corresponding QoS flows, and to setup corresponding DRBs for a given UE. The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

For each PDU session in the PDU SESSION RESOURCE SETUP REQUEST message, if the Alternative QoS Parameters Set List IE is included in the GBR QoS Flow Information IE in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may accept the setup of the QoS flow when notification control has been enabled if the requested QoS parameters or at least one of the alternative QoS parameters sets can be fulfilled at the time of setup. In case the NG-RAN node accepts the setup fulfilling one of the alternative QoS parameters it shall indicate the alternative QoS parameters set which it currently fulfils in the Current QoS Parameters Set Index IE within the PDU Session Resource Setup Response Transfer IE of the PDU SESSION RESOURCE SETUP RESPONSE message. If the NG-RAN node receives a PDU SESSION RESOURCE SETUP REQUEST message containing the Delay Critical IE in the Dynamic 5QI Descriptor IE of the QoS Flow Level QoS Parameters IE of the PDU Session Resource Setup Request Transfer IE set to the value "delay critical" but the Maximum Data Burst Volume IE is not present in the Alternative QoS Parameters Set List IE, the NG-RAN node shall use the Maximum Data Burst Volume IE included in the Dynamic 5QI Descriptor IE.

For each QoS flow which has been successfully established, the NG-RAN node shall, if supported, store the Redundant QoS Flow Indicator IE if included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and consider it for the redundant transmission as specified in TS 23.501 [9].

For each QoS flow which has been successfully established, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [9]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store this information and, if supported, use it for RAN part delay reporting.

For each GBR QoS flow which has been successfully established, if the Monitoring Request on Available Bitrate IE was included in the GBR QoS Flow Information IE in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, store this information and perform available bitrate monitoring, as specified in TS 23.501 [9].

For each QoS flow requested to be setup, if the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, store the received PDU Set QoS Parameters in the UE context and use it as specified in TS 23.501 [9]. If the ECN Marking or Congestion Information Reporting Request IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it accordingly for the specific QoS flow. If the ECN Marking or Congestion Information Reporting Status IE is included in the PDU Session Resource Setup Response Transfer IE, the SMF shall, if supported, use it to deduce if ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active.

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

If the PDU Set QoS Parameters IE or the DL PDU Set Information Marking Support Indication IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE SETUP RESPONSE message the PDU Set based Handling Indicator IE in the PDU Session Resource Setup Response Transfer IE. If the PDU Set based Handling Indicator IE is included in the PDU Session Resource Setup Response Transfer IE in the PDU SESSION RESOURCE SETUP RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.501 [9].

If the MBS Support Indicator IE is included in the PDU Session Resource Setup Response Transfer IE in the PDU SESSION RESOURCE SETUP RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.247 [44].

For each QoS flow requested to be setup, if the MMSID IE is included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [9] and TS 38.300 [8].

-----------------Next Changes-------------------

### 8.2.3	PDU Session Resource Modify

#### 8.2.3.1	General

The purpose of the PDU Session Resource Modify procedure is to enable configuration modifications of already established PDU session(s) for a given UE. It is also to enable the setup, modification and release of the QoS flow for already established PDU session(s). The procedure uses UE-associated signalling.

#### 8.2.3.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

For each QoS flow which has been successfully added or modified, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [9]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall store this information and, if supported, use it for RAN part delay reporting.

For each GBR QoS flow which has been successfully added or modified, if the Monitoring Request on Available Bitrate IE was included in the GBR QoS Flow Information IE in the PDU Session Resource Modify Request Transfer IE of the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, store this information and perform available bitrate monitoring, as specified in TS 23.501 [9].

The NG-RAN node shall report to the AMF, in the PDU SESSION RESOURCE MODIFY RESPONSE message, the result for each PDU session requested to be modified listed in the PDU SESSION RESOURCE MODIFY REQUEST message:

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

If the PDU Set QoS Parameters IE or the DL PDU Set Information Marking Support Indication IE is included in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE MODIFY RESPONSE message the PDU Set based Handling Indicator IE in the PDU Session Resource Modify Response Transfer IE. If the PDU Set based Handling Indicator IE is included in the PDU Session Resource Modify Response Transfer IE in the PDU SESSION RESOURCE MODIFY RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.501 [9].

If the MBS Support Indicator IE is included in the PDU Session Resource Modify Response Transfer IE in the PDU SESSION RESOURCE MODIFY RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.247 [44].

For each QoS flow to be added or modified, if the MMSID IE is included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [9] and TS 38.300 [8].

-----------------Next Changes-------------------

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

Upon receiving the PDU Session Resource Setup List IE contained in the HANDOVER REQUEST message and the HANDOVER REQUEST message does not contain the No PDU Session Indication IE, the target NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. The target NG-RAN node shall report to the AMF in the HANDOVER REQUEST ACKNOWLEDGE message the result for each PDU session resource requested to be setup. In particular, for each PDU session resource successfully setup, it shall include the Handover Request Acknowledge Transfer IE containing the following information:

-	The list of QoS flows which have been successfully established in the QoS Flow Setup Response List IE.

-	The Data Forwarding Accepted IE if the data forwarding for the QoS flow is accepted.

-	The list of QoS flows which have failed to be established, if any, in the QoS Flow Failed to Setup List IE.

-	The UP transport layer information to be used for the PDU session.

-	The security result associated to the PDU session.

-	The redundant UP transport layer information to be used for the redundant transmission for the PDU session.

-	The PDU Set based Handling Indicator if the HANDOVER REQUEST message includes the PDU Set QoS Parameters IE or the DL PDU Set Information Marking Support Indication IE.

-	The ECN Marking or Congestion Information Reporting Status if the HANDOVER REQUEST message includes the ECN Marking or Congestion Information Reporting Request IE.

-----------------Next Changes-------------------

#### 9.3.1.12	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.18 |  | - |  |
| Allocation and Retention Priority | M |  | 9.3.1.19 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.10 | This IE shall be present for GBR QoS flows and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [9]. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1.. 1800, …) | Indicates the reporting frequency for RAN part delay for QoS monitoring. Units: second | YES | ignore |
| PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| DL PDU Set Information Marking Support Indication | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Monitoring Request on Available Bitrate |  | 0..1 |  |  | YES | ignore |
| >Monitoring Request | M |  | ENUMERATED (ul, dl, both, stop, …) | Indicates to monitor and report UL, or DL, or both UL/DL available bitrate for the associated QoS flow as specified in TS 23.501 [9], or stop the corresponding QoS monitoring. | - | ignore |
| >DL Available Bitrate Report Thresholds | C-ifReportDL |  | Available Bitrate Report Threshold List 9.3.1.x |  | - | ignore |
| >UL Available Bitrate Report Thresholds | C-ifReportUL |  | Available Bitrate Report Threshold List 9.3.1.x |  | - | ignore |
| MMSID | O |  | OCTET STRING (SIZE(1)) | Multi-modal service ID from the application, used to indicate QoS flows are related to a multi-modal service, as specified in TS 23.501 [9] and TS 38.300 [8]. | YES | ignore |
| Indication of Bitrate Adaptation | O |  | ENUMERATED (uplink, …) | Indicates that the QoS Flow allows rate adaptation in the indicated direction. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportDL | This IE shall be present if the Monitoring Request IE is set to the value “dl” or “both”. |
| ifReportUL | This IE shall be present if the Monitoring Request IE is set to the value “ul” or “both”. |

-----------------Next Changes-------------------

#### 9.3.1.151	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  |  |  |
| >Alternative QoS Parameters Set Index | M |  | 9.3.1.152 |  | - |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Packet Delay Budget | O |  | 9.3.1.80 |  | - |  |
| >Packet Error Rate | O |  | 9.3.1.81 |  | - |  |
| >Maximum Data Burst Volume | O |  | 9.3.1.83 | Maximum Data Burst Volume is specified in TS 23.501 [9]. This IE is included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. | YES | ignore |
| >PDU Set Delay Budget Downlink | O |  | 9.3.1.135 | This IE is included if the PDU Set Packet Delay Budget IE in the DL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >PDU Set Delay Budget Uplink | O |  | 9.3.1.135 | This IE is included if the PDU Set Packet Delay Budget IE in the UL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >PDU Set Error Rate Downlink | O |  | 9.3.1.81 | This IE is included if the PDU Set Error Rate IE in the DL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >PDU Set Error Rate Uplink | O |  | 9.3.1.81 | This IE is included if the PDU Set Error Rate IE in the UL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |

-----------------Next Changes-------------------

#### 9.3.1.x 	Available Bitrate Report Threshold List

This IE contains a list of available bitrate report thresholds. It is used for available bitrate report for UL and DL as specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available Bitrate Report Threshold Item |  | 1..<maxnoofThresholds> |  |  |
| >Reporting Threshold | M |  | INTEGER (0..4.000.000.000) | This IE indicates the Reporting threshold as specified in TS 23.501 [9]. The unit is Kbps. |

| Range bound | Explanation |
| --- | --- |
| maxnoofThresholds | Maximum no. of thresholds allowed to be provided by the SMF. Value is 8. |

-----------------Next Changes-------------------

///////////////////////////////////////////////////////////////////////skip unrelated////////////////////////////////////////////////////////////////////////

-----------------Next Changes-------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-PduSetDelayBudgetDownlink,

id-PduSetDelayBudgetUplink,

id-PduSetErrorRateDownlink,

id-PduSetErrorRateUplink,

id-DLPDUSetInformationMarkingSupportIndication,

id-MonitoringRequestonAvailableDataRate,

id-MMSID,

id-Indication-of-bitrate-adaptation,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

maxnoofRSPPQoSFlows,

maxnoofThresholds

FROM NGAP-Constants

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			AlternativeQoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate												OPTIONAL,

guaranteedFlowBitRateUL				BitRate												OPTIONAL,

packetDelayBudget					PacketDelayBudget									OPTIONAL,

packetErrorRate						PacketErrorRate										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaximumDataBurstVolume	CRITICALITY ignore	EXTENSION MaximumDataBurstVolume		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetDownlink 	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetErrorRateDownlink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		}|

{ ID id-PduSetErrorRateUplink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		},

...

}

AssistanceInformationQoE-Meas ::= INTEGER (1..16, ...)

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

AreaScopeOfQMC ::= CHOICE {

cellBased				CellBasedQMC,

tABased					TABasedQMC,

tAIBased					TAIBasedQMC,

pLMNAreaBased			PLMNAreaBasedQMC,

choice-Extensions		ProtocolIE-SingleContainer { { AreaScopeOfQMC-ExtIEs} }

}

AreaScopeOfQMC-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

AvailableBitrateReportThresholdList	::= SEQUENCE (SIZE(1..maxnoofThresholds)) OF AvailableBitrateReportThresholdItem

AvailableBitrateReportThresholdItem	::= SEQUENCE {

reportingThreshold				ReportingThreshold,

iE-Extensions					ProtocolExtensionContainer { { AvailableBitrateReportThresholdItem-ExtIEs} }	OPTIONAL,

...

}

AvailableBitrateReportThresholdItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AvailableRANVisibleQoEMetrics ::= SEQUENCE {

applicationLayerBufferLevelList		ENUMERATED {true, ...}		OPTIONAL,

playoutDelayForMediaStartup			ENUMERATED {true, ...}		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { AvailableRANVisibleQoEMetrics-ExtIEs} }	OPTIONAL,

...

}

AvailableRANVisibleQoEMetrics-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

DL-CP-SecurityInformation ::= SEQUENCE {

dl-NAS-MAC				DL-NAS-MAC,

iE-Extensions			ProtocolExtensionContainer { { DL-CP-SecurityInformation-ExtIEs} }	OPTIONAL,

...

}

DL-CP-SecurityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DL-Signalling ::= ENUMERATED {true,	...}

DL-NAS-MAC ::= BIT STRING (SIZE (16))

DLForwarding ::= ENUMERATED {

dl-forwarding-proposed,

...

}

DL-NGU-TNLInformationReused ::= ENUMERATED {

true,

...

}

DLPDUSetInformationMarkingSupportIndication		::= ENUMERATED {true,	...}

DirectForwardingPathAvailability ::= ENUMERATED {

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

-- G

GBR-QosInformation ::= SEQUENCE {

maximumFlowBitRateDL		BitRate,

maximumFlowBitRateUL		BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			NotificationControl									OPTIONAL,

maximumPacketLossRateDL		PacketLossRate										OPTIONAL,

maximumPacketLossRateUL		PacketLossRate										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {GBR-QosInformation-ExtIEs} }	OPTIONAL,

...

}

GBR-QosInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	}|

{ ID id-MonitoringRequestonAvailableBitrate			CRITICALITY ignore	EXTENSION MonitoringRequestonAvailableBitrate		PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

IndexToRFSP ::= INTEGER (1..256, ...)

Indication-of-bitrate-adaptation ::= ENUMERATED {uplink, ...}

InfoOnRecommendedCellsAndRANNodesForPaging ::= SEQUENCE {

recommendedCellsForPaging		RecommendedCellsForPaging,

recommendRANNodesForPaging		RecommendedRANNodesForPaging,

iE-Extensions		ProtocolExtensionContainer { {InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs} }	OPTIONAL,

...

}

InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

MDT-Location-Info-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Location-Information::= BIT STRING (SIZE (8))

MMSID ::= OCTET STRING (SIZE (1))

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

MobileIAB-Supported ::= ENUMERATED {

true,

...

}

MonitoringRequestonAvailableBitrate ::= SEQUENCE {

monitoringRequest						MonitoringRequest,

dlAvailableBitrateReportThresholds		AvailableBitrateReportThresholdList			OPTIONAL,

-- The above IE shall be present if the Monitoring Request IE is set to the value “dl” or “both”

ulAvailableBitrateRateReportThresholds		AvailableBitrateReportThresholdList			OPTIONAL,

-- The above IE shall be present if the Monitoring Request IE is set to the value “ul” or “both”

iE-Extensions							ProtocolExtensionContainer { { MonitoringRequestonAvailableBitrate-ExtIEs} } OPTIONAL,

...

}

MonitoringRequestonAvailableBitrate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MonitoringRequest ::= ENUMERATED {ul, dl, both, stop,...}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

NotAllowedTACs ::= SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC

NotificationCause ::= ENUMERATED {

fulfilled,

not-fulfilled,

... ,

not-fulfilled-DL,

not-fulfilled-UL

}

NotificationControl ::= ENUMERATED {

notification-requested,

...

}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

QosFlowLevelQosParameters ::= SEQUENCE {

qosCharacteristics					QosCharacteristics,

allocationAndRetentionPriority		AllocationAndRetentionPriority,

gBR-QosInformation					GBR-QosInformation									OPTIONAL,

reflectiveQosAttribute				ReflectiveQosAttribute								OPTIONAL,

additionalQosFlowInformation		AdditionalQosFlowInformation						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowLevelQosParameters-ExtIEs} }	OPTIONAL,

...

}

QosFlowLevelQosParameters-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-QosMonitoringRequest				CRITICALITY ignore	EXTENSION QosMonitoringRequest					PRESENCE optional}|

{ID id-QosMonitoringReportingFrequency	CRITICALITY ignore	EXTENSION QosMonitoringReportingFrequency	PRESENCE optional}|

{ID id-PDUsetQoSParameters				CRITICALITY	ignore	EXTENSION PDUsetQoSParameters					PRESENCE optional}|

{ID id-DLPDUSetInformationMarkingSupportIndication			CRITICALITY ignore	EXTENSION DLPDUSetInformationMarkingSupportIndication		PRESENCE optional }|

{ ID id-MMSID							CRITICALITY ignore	EXTENSION MMSID									PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

ReportArea ::= ENUMERATED {

cell,

...

}

ReportingThreshold	::= INTEGER (0..4000000000)

RepetitionPeriod ::= INTEGER (0..131071)

-----------------Next Changes-------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofThresholds							INTEGER ::= 8

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-PduSetDelayBudgetDownlink							ProtocolIE-ID ::= a1

id-PduSetDelayBudgetUplink								ProtocolIE-ID ::= a2

id-PduSetErrorRateDownlink								ProtocolIE-ID ::= a3

id-PduSetErrorRateUplink								ProtocolIE-ID ::= a4

id-DLPDUSetInformationMarkingSupportIndication			ProtocolIE-ID ::= a5

id-MonitoringRequestonAvailableBitrate					ProtocolIE-ID ::= b1

id-MMSID												ProtocolIE-ID ::= c1

id-Indication-of-bitrate-adaptation						ProtocolIE-ID ::= d1

END

-- ASN1STOP

-----------------End of the Changes-------------------