3GPP TSG-RAN WG3 Meeting #128	R3-253965

# St Julian, Malta, 19th – 23rd May, 2025

Agenda Item:	19.2

Source:	Ericsson, Samsung, ZTE, Nokia

Title:	(TP to BLCR for 38.300) Introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD)

Document for:	Discussion, Approval

# 1	Introduction

These TPs mirror proposals agreed at RAN3-128.

# 2	TP to TS38.300

/*********************Start of changes***********************/

## 17.2	Cross-Link Interference Management

When different TDD DL/UL patterns are used between neighbouring cells, UL transmission in one cell may interfere with DL reception in another cell: this is referred to as Cross Link Interference (CLI).

To mitigate CLI, gNBs can exchange and coordinate their intended TDD DL-UL configurations over Xn and F1 interfaces; and the victim UEs can be configured to perform CLI measurements. There are two types of CLI measurements:

-	SRS-RSRP measurement in which the UE measures SRS-RSRP over SRS resources of aggressor UE(s);

-	CLI-RSSI measurement in which the UE measures the total received power observed over RSSI resources.

Layer 3 filtering applies to CLI measurement results and both event triggered and periodic reporting are supported.

In case of Sub-band full duplex (SBFD) operation, gNB-to-gNB CLI may also be present. Additionaly, DL transmission in one cell may interfere with UL reception in another cell: this is referred to as gNB-to-gNB CLI. A victim gNB reports gNB-to-gNB CLI related information of its serving cells to neighbour gNBs. The neighbour gNB should evaluate the received information and it may take CLI mitigation actions when necessary.

/*********************End of changes***********************/