3GPP TSG-RAN WG3 Meeting #128	R3-253857

Malta, MT, 19 – 23 May, 2025

Title: 	(TP to BLCR for TS 38.473) Discussion on on-demand SIB1 for UEs in idle or inactive mode

Source: 	Huawei, Ericsson, NEC

Agenda item:	17.3

Document Type:	Other

- Introduction

This contribution provided the TP on the on-demand SIB1 based on CB: # R19NES.

5. TP for TS 38.473 – on top of R3-252499

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

8.2.3	F1 Setup

8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the On-demand SIB1 IE is included and set to “Provision” in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information indicated in the od-SIB1-Config IE for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the On-demand SIB1 IE is included and set to “Stop provision” in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, stop the coordination of on-demand SIB1 transmission as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.2.4	gNB-DU Configuration Update

8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the On-demand SIB1 IE is included and set to “Provision” in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information indicated in the od-SIB1-Config IE for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the On-demand SIB1 IE is included and set to “Stop provision” in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, stop the coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

8.2.5	gNB-CU Configuration Update

8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If the On-Demand SIB1 Cell IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider the cell indicated by the NR CGI IE for On-Demand SIB1 operation.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| On-Demand SIB1 Cell |  | 0..1 |  |  | YES | ignore |
| >NR CGI | M |  | 9.3.1.12 |  | - |  |
| >On-Demand SIB1 Indicator | M |  | ENUMERATED (start, stop...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| CHOICE on-demand SIB1 | O |  |  |  | YES | ignore |
| >Provision |  |  |  |  |  |  |
| >>On-demand SIB1 Config | M |  | Octet String | Includes the od-SIB1-Config contained in the SIBxx message as defined in TS 38.331 [8]. |  |  |
| >Stop provision |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

LTMResetInformation,

On-Demand-SIB1-Cell

FROM F1AP-IEs

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-On-Demand-SIB1-Cell,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	Cells-to-be-Activated-List							PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	Cells-to-be-Barred-List								PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 	BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-CellsForSON-List						CRITICALITY ignore	TYPE	CellsForSON-List										PRESENCE optional }|

{ ID id-gNB-CU-Name								CRITICALITY ignore	TYPE	GNB-CU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name					CRITICALITY ignore	TYPE	Extended-GNB-CU-Name									PRESENCE optional	}|

{ ID id-Cells-Allowed-to-be-Deactivated-List			CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List		PRESENCE optional	}|

{ ID id-On-Demand-SIB1-Cell						CRITICALITY ignore	TYPE	On-Demand-SIB1-Cell		PRESENCE optional	},

...

}

Cells-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Deactivated-List-ItemIEs } }

GNB-CU-TNL-Association-To-Add-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Add-ItemIEs } }

GNB-CU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Remove-ItemIEs } }

GNB-CU-TNL-Association-To-Update-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Update-ItemIEs } }

Cells-to-be-Barred-List			::= SEQUENCE(SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { Cells-to-be-Barred-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Allowed-to-be-Deactivated-List-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Allowed-to-be-Deactivated-List-Item	CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List-Item	PRESENCE mandatory },

...

}

Cells-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Deactivated-List-Item						CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List-Item					PRESENCE mandatory	},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-XR-Bcast-Information,

id-MaxDataBurstVolume,

id-BarringExemptionforEmerCallInfo,

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-OnDemandSIB1,

id-NESPagingAdaptationIndication,

maxNRARFCN,

maxnoofErrors,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Cells-Allowed-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

iE-Extensions				ProtocolExtensionContainer { { Cells-Allowed-to-be-Deactivated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-Allowed-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI			NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Deactivated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Barred-Item::= SEQUENCE {

nRCGI			NRCGI	,

cellBarred		CellBarred,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Barred-Item-ExtIEs } }	OPTIONAL

}

Cells-to-be-Barred-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-IAB-Barred	CRITICALITY ignore	EXTENSION IAB-Barred		PRESENCE optional }|

{ ID id-MobileIAB-Barred	CRITICALITY ignore	EXTENSION MobileIAB-Barred		PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

NeighbourNR-CellsForSON-Item ::= SEQUENCE {

nRCGI											NRCGI,

nR-ModeInfoRel16								NR-ModeInfoRel16												OPTIONAL,

sSB-PositionsInBurst							SSB-PositionsInBurst											OPTIONAL,

nRPRACHConfig									NRPRACHConfig													OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { NeighbourNR-CellsForSON-Item-ExtIEs} }	OPTIONAL,

...

}

NeighbourNR-CellsForSON-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NESPagingAdaptationIndication ::= ENUMERATED {true, ...}

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

NGRANAllocationAndRetentionPriority-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- O

OffsetToPointA	::= INTEGER (0..2199,...)

OnDemandPRS-Info ::= SEQUENCE {

onDemandPRSRequestAllowed				BIT STRING (SIZE (16)),

allowedResourceSetPeriodicityValues		BIT STRING (SIZE (24))	OPTIONAL,

allowedPRSBandwidthValues				BIT STRING (SIZE (64))	OPTIONAL,

allowedResourceRepetitionFactorValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedResourceNumberOfSymbolsValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedCombSizeValues					BIT STRING (SIZE (8))	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { OnDemandPRS-Info-ExtIEs} } OPTIONAL,

...

}

OnDemandPRS-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

OnDemandSIB1 ::= CHOICE {

provision						On-demandSIB1Config,

stopProvison					NULL,

choice-extension				ProtocolIE-SingleContainer { { OnDemandSIB1-ExtIEs} }

}

OnDemandSIB1-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

On-Demand-SIB1-Cell ::= CHOICE {

nRCGI							NRCGI,

on-demandSIBIndindicator		ENUMERATED{accepted, notaccepted, ...},,

choice-extension				ProtocolIE-SingleContainer { { On-Demand-SIB1-Cell-ExtIEs} }

}

On-Demand-SIB1-Cell-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Served-Cell-Information ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

fiveGS-TAC						FiveGS-TAC			OPTIONAL,

configured-EPS-TAC				Configured-EPS-TAC 		OPTIONAL,

servedPLMNs						ServedPLMNs-List,

nR-Mode-Info					NR-Mode-Info,

measurementTimingConfiguration	OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RANAC							CRITICALITY ignore	EXTENSION RANAC							PRESENCE optional }|

{	ID id-ExtendedServedPLMNs-List		CRITICALITY ignore	EXTENSION ExtendedServedPLMNs-List	PRESENCE optional }|

{	ID id-Cell-Direction				CRITICALITY ignore	EXTENSION Cell-Direction				PRESENCE optional }|

{	ID id-BPLMN-ID-Info-List			CRITICALITY ignore	EXTENSION BPLMN-ID-Info-List			PRESENCE optional }|

{	ID id-Cell-Type						CRITICALITY ignore	EXTENSION CellType						PRESENCE optional}|

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-AggressorgNBSetID				CRITICALITY ignore	EXTENSION AggressorgNBSetID				PRESENCE optional}|

{	ID id-VictimgNBSetID				CRITICALITY ignore	EXTENSION VictimgNBSetID				PRESENCE optional}|

{	ID id-IAB-Info-IAB-DU				CRITICALITY ignore	EXTENSION IAB-Info-IAB-DU				PRESENCE optional}|

{	ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst			PRESENCE optional }|

{	ID id-NRPRACHConfig					CRITICALITY ignore	EXTENSION NRPRACHConfig					PRESENCE optional }|

{	ID id-SFN-Offset					CRITICALITY ignore	EXTENSION SFN-Offset					PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject 	EXTENSION NPNBroadcastInformation		PRESENCE optional }|

{	ID id-Supported-MBS-FSA-ID-List				CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List						PRESENCE optional }|

{	ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information	PRESENCE optional }|

{	ID id-ERedcap-Bcast-Information		CRITICALITY ignore	EXTENSION ERedcap-Bcast-Information	PRESENCE optional }|

{	ID id-XR-Bcast-Information			CRITICALITY ignore	EXTENSION XR-Bcast-Information			PRESENCE optional }|

{	ID id-BarringExemptionforEmerCallInfo				CRITICALITY ignore	EXTENSION BarringExemptionforEmerCallInfo				PRESENCE optional }|

{	ID id-OnDemandSIB1					CRITICALITY ignore	EXTENSION	OnDemandSIB1				PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

UEPagingCapability ::= SEQUENCE {

iNACTIVEStatePODetermination						ENUMERATED {supported, ...} 	OPTIONAL,

iE-Extension				ProtocolExtensionContainer { { UEPagingCapability-ExtIEs} } 		OPTIONAL,

...

}

UEPagingCapability-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RedCapIndication		CRITICALITY ignore 	EXTENSION RedCapIndication		PRESENCE optional }|

{ 	ID id-NESPagingAdaptationIndication	CRITICALITY ignore	EXTENSION NESPagingAdaptationIndication	PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

UuRLCChannelRequiredToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelRequiredToBeReleasedItem

UuRLCChannelRequiredToBeReleasedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelRequiredToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelRequiredToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

On-demandSIB1Config  ::= OCTET STRING

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-TagIDPointer										ProtocolIE-ID ::= 853

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-OnDemandSIB1										ProtocolIE-ID ::= aaa

id-NESPagingAdaptationIndication					ProtocolIE-ID ::= bbb

id-Cells-to-be-On-Demand-SIB1-List					ProtocolIE-ID ::= ccc

id-Cells-to-be-On-Demand-SIB1-List-Item				ProtocolIE-ID ::= ddd

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>