3GPP TSG-RAN WG3#129bis 	R3-256530

Prague, Czechia, 13-17 October 2025

Agenda item:	9.2.5. R19 Mobility enhancements

Source:	Nokia

Title:	Discussion on remaining LTM Issues

Document for:	Discussion and Decision

# 1	Introduction

This contribution describes remaining issues for Rel-19 Inter-CU LTM and Conditional LTM and provides a way forward to address them.

# 2	Inter-CU LTM Issues

## 2.1 	Conflict for Subsequent Intra-CU LTM after Inter-CU LTM

RAN2 agreed that network includes the NCC value used for security key generation for Inter-CU LTM as part of the cell switch command signalled to the UE. However, for Intra-CU LTM the Rel-18 cell switch command that is signalled to the UE does NOT include the NCC value. This can result in issues for subsequent LTM when Intra-CU LTM at the target gNB is carried out following a prior Inter-CU LTM cell switch.

During LTM inter-CU LTM preparation, the network generates Key-NG-RAN separately for all candidates that belong to same target CU. It means all prepared candidates belonging to same CU shall have generated and stored different AS keys in the candidate context. Upon inter-CU cell switch to one of the candidates of the target CU, the UE generates the corresponding AS keys on receiving NCC from the network in cell switch command. Contrastingly, when the network decides to perform Intra-CU cell switch, the network will send a Rel-18 cell switch command (without NCC), resulting in UE reusing the security keys as the source cell serving the UE. However, the target CU during preparation has received different Key-NG-RAN and hence has different AS keys. This mean RRC Reconfiguration Complete message for subsequent intra CU cell switch received by network before network updates the security keys for target intra CU candidates during previous inter CU cell switch.

Therefore, if a subsequent Intra-CU LTM cell switch(es) takes place following an Inter-CU LTM cell switch, it may result in key mismatches at UE and the network if it is carried out with the earlier prepared candidate configurations. A related issue may also occur with respect to PDCP-reestablishment, since during inter CU LTM preparation, PDCP-reestablishment may have been configured for all the target candidates of the target CU. That is, the UE may not perform the PDCP reestablishment due to same CellNoResetID during Intra-CU LTM cell switch resulting configuration mismatch.

Observation 1: Intra-CU cell switch after Inter-CU cell switch will result in mismatch in UE and network configuration for security and PDCP/RLC re-establishment related configuration resulting in a mobility failure.

A possible solution to tackle this issue is for the network to signal to Candidate gNB-DU (i.e. the new Source gNB-DU after mobility) to halt any further subsequent Intra-CU LTM cell switches until receives further confirmation from gNB-CU that security configurations have been updated. It is worth noting that halting any further events by default is not a desirable outcome as the issue is limited to further Intra-CU mobility events. That is, subsequent Inter-CU LTM cell switches do not encounter this issue.

Proposal 1: Introduce over F1AP signalling from new Source gNB-CU to new Source gNB-DU to halt and resume further subsequent Intra-CU LTM mobility commands toward the UE.

## 2.2 	Semi-Persistent CSI-RS support for LTM

At prior RAN3 meetings, there was discussion on whether the TCI State/QCL-info information associated with a semi-persistent (SP) CSI-RS being activated needs to be signalled from the source gNB-DU to the candidate gNB-DU. However, this issue was left inconclusive.

Currently, each candidate shares the list of TCI states it supports. For each periodic CSI-RS, the candidate also shares the associated QCL-info (via qcl-InfoPeriodicCSI-RS, which indicates a TCI state ID). For SP CSI-RS, this is not possible. The reason is that SP CSI-RSs are pre-configured but only activated dynamically, based on needs determined from periodic SSB or CSI-RS measurements. Furthermore, the beam (TCI state) is dynamically determined for the activated SP CSI-RS, and this is provided to the UE in the same MAC CE that indicates the activation of the SP CSI-RS.

Following the same principle, when the source gNB selects an SP CSI-RS to activate, it should also determine the appropriate TCI state (from the TCI state list of the candidate) and indicate this to the candidate, so that the SP CSI-RS can be properly activated with the correct TCI state. If a static allocation of TCI states were applied to SP CSI-RSs (as for periodic CSI-RSs), the main advantage of dynamic SP CSI-RS activation would be lost.

Observation 2: For each periodic CSI-RS, the QCL-info is shared (via qcl-InfoPeriodicCSI-RS) with an associated TCI state ID. However, for SP CSI-RSs this is not feasible, since they are dynamically activated and their TCI state is determined dynamically and signalled to the UE in the same MAC CE that triggers activation.

Proposal 2: The F1AP DU-CU CSI-RS COORDINATION REQUEST and CU-DU CSI-RS COORDINATION REQUEST messages include a TCI State/QCL-info List which includes the TCI State Index for each SP CSI-RS being activated.

Proposal 3: The XnAP CSI-RS COORDINATION REQUEST message includes a TCI State/QCL-info List which includes the TCI State Index for each SP CSI-RS being activated.

## 2.3 	Missing RACH-based or RACH-less Indication

During LTM execution and upon cell switch decision, the gNB-DU sends a MAC CE command to the UE which includes the TCI state indicating the UE which beams it should use to access the target cell (applicable for both RACH-less and RACH-based mobility), as well as a Timing Advance Command (TAC). Likewise, the value in the TAC indicates to the UE whether it should perform a RACH-less switch.

However, over F1AP and XnAP interfaces, the Cell Switch Notification messages although it includes the TCI State ID, lacks support for indicating whether RACH-less mobility was triggered to the UE. Therefore, the Candidate gNB-DU/Candidate gNB is unaware whether the upcoming UE access is RACH-less or RACH-based. Consequently, it can also not determine whether it should issue a Dynamic Grant (DG) for DG-based RACH-less cell switch, or whether it should activate a Configured Grant (CG) previously provided in the RRC Configuration for CG-based RACH-less switch. Thus, due to this uncertainty, the target cell is forced to issue a DG, even when the triggered switch by the source was RACH-based, leading to PDCCH and PUSCH resources being wasted. Similarly, to support CG-based RACH-less cell switch, the target cell is also forced to activate and monitor the CG, even for the RACH-based cell switch, leading to PUSCH resource waste. For these reasons, an IE from Source gNB-DU/gNB is required over F1 and Xn interfaces to indicate whether the cell switch triggered was RACH-less based correspondingly.

Proposal 4: Add a “Cell Switch Type (ENUMERATED(RACH-less, RACH-based))” in F1AP and XnAP CELL SWITCH NOTIFICATION messages.

## 2.4 	Acquired TA List forwarding between gNBs

RAN3 agreed that also for Inter-CU LTM mobility, the source gNB can provide a list of “valid” TA that have been previously acquired. It was also agreed that ensuring the TA validity was left up to implementation as in Rel-18 intra-gNB LTM.

In our view, already for intra-gNB scenarios, the validity of an earlier acquired TA after mobility can be doubtful for the target which becomes the new source in inter-DU LTM cases. Moreover, for inter-CU LTM, the validity of prior acquired TA values is even more likely to be invalid. Therefore, we think for Rel-19 Inter-CU LTM, the TA validity cannot be left up to implementation as the Xn link delay needs to be accounted for. A possible approach which can be linked to UE signal experienced, is to provide information such as the RSRP which the UE experienced when TA was acquired. This would allow the Target-gNB/DU to better understand the signal conditions associated with a TA value being received, rather than a “time” value tied to the TA acquisition itself. Based on the received RSRP value along with TA value(s) and the new RSRP value, the new source DU can determine the validity of the TA value. Note that such scheme is already possible for SDT and should be considered for LTM.

Without such indication, we deem signaling of the TA values between nodes not only of no use, but rather worse, as having a UE attempt to access a cell with an invalid TA will result in failures. Hence, worsening overall performance and directly affecting user experience.

Proposal 5: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

# 3	Conclusions

Observation 1: Intra-CU cell switch after Inter-CU cell switch will result in mismatch in UE and network configuration for security and PDCP/RLC re-establishment related configuration resulting in a mobility failure.

Proposal 1: Introduce over F1AP signalling from new Source gNB-CU to new Source gNB-DU to halt and resume further subsequent Intra-CU LTM mobility commands toward the UE.

Observation 2: For each periodic CSI-RS, the QCL-info is shared (via qcl-InfoPeriodicCSI-RS) with an associated TCI state ID. However, for SP CSI-RSs this is not feasible, since they are dynamically activated and their TCI state is determined dynamically and signalled to the UE in the same MAC CE that triggers activation.

Proposal 2: The F1AP DU-CU CSI-RS COORDINATION REQUEST and CU-DU CSI-RS COORDINATION REQUEST messages include a TCI State/QCL-info List which includes the TCI State Index for each SP CSI-RS being activated.

Proposal 3: The XnAP CSI-RS COORDINATION REQUEST message includes a TCI State/QCL-info List which includes the TCI State Index for each SP CSI-RS being activated.

Proposal 4: Add a “Cell Switch Type (ENUMERATED(RACH-less, RACH-based))” in F1AP and XnAP CELL SWITCH NOTIFICATION messages.

Proposal 5: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

# Annex	Text Proposal for TS 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.5.5	CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Include the TCI-StateId IE as defined in TS 38.331 [10]. | – |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE as defined in TS 38.331 [10]. | – |  |
| >Cell Switch Type | O |  | ENUMERATED (RACH-less, RACH-based, …) | Indicates the cell switch type triggered. | - |  |
| LTM UE Association Information List |  | 0..1 |  |  | YES | ignore |
| >LTM UE Association Item |  | 1.. <maxnoofLTMCells> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Last Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Cell Switch TA Information List |  | 0..1 |  |  | YES | ignore |
| >Cell Switch TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | – |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [10]. | – |  |
| >>RSRP | O |  | INTERGER (0..127) | Indicates the RSRP the UE experienced when TA was acquired. Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex	Text Proposal for TS 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Cell Switch Type | O |  | ENUMERATED (RACH-less, RACH-based, …) | Indicates the cell switch type triggered. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >>RSRP | O |  | INTERGER (0..127) | Indicates the RSRP the UE experienced when TA was acquired. Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Cell Switch Type | O |  | ENUMERATED (RACH-less, RACH-based, …) | Indicates the cell switch type triggered. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >>RSRP | O |  | INTERGER (0..127) | Indicates the RSRP the UE experienced when TA was acquired. Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. | - |  |
| Last Visited LTM Cells | O |  | 9.3.1.358 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>