3GPP TSG-RAN WG3 Meeting #129bis	R3-256601

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:	11.2.1

Source:	Nokia

Title:	MRO Enhancements for Inter-CU LTM

Document for:	Approval

# 1	Introduction

This is an initial contribution related to MRO enhancements for Inter CU-LTM in Rel.20.

# 2	Discussion

## 2.1 MRO for LTM

### 2.1.1 Agreements achieved in RAN Plenary

The agreements achieved in RAN #108 [1]:

The objective of NR_SON_MDT_Ph5-Core WI is to specify data collection enhancement in NR standalone for SON/MDT purpose. The specific objectives of this work item are:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

If needed, co-operate with RAN1, SA5.

## 2.1.2 3GPP Release 19 Mobility Enhancements – Inter-CU LTM

In Fig. 1 an extraction from the 3GPP TS 38.300 comprising the inter-CU LTM is shown.

Figure 1 Signalling procedure for inter-CU LTM

The difference compared to Rel. 18 LTM is that the functionality is not restricted to the same CU CP (gNB). The source gNB based on L3 measurement report from UE may decide LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s) (steps 1 – 5). The LTM configuration update procedure is then followed between the source and candidate gNBs (step 6 – 7) which is then followed with RRC Reconfiguration procedure to UE and possible early data forwarding (step 8 – 9).    Early data forwarding may be initiated by source gNB in step 9. Consequently, an early DL and UL synchronization to some LTM candidate cell(s) may be performed. With a NW triggered UL synchronization, the UE performs early TA acquisition (step 10 – 11). In case of inter-CU CP LTM new message TA Information transfer in step 11 is transferred from candidate gNB to source gNB. Consequently, based on L1 or L3 measurements from UE the source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE (step 12 – 14). New message Cell switch notification is transmitted from source to target gNB (step 15).

### 2.1.3 Connection failure monitoring for inter-CU LTM

As part of the NR_SON_MDT_Ph4-Core WI in Rel. 19 MRO for Rel. 18 LTM was provided. The focus was related to enhancement of the RLF report with LTM relevant information. Apart of that RAN3 agreed network-based solution for so called case 2 “RACH less LTM cell switch failure due to wrong beam” and case 3 “LTM failure due to outdated TA”. With the introduction of the inter-CU LTM the source DU and target DU may be in different gNBs. Thus, the proposed signaling between the target and source DU will not work for inter-CU LTM.

Observation 1: The network-based concept for case 2 and 3 is insufficient for inter-CU LTM.

Proposal 1: RAN3 agrees to analyse the applicability and possible enhancements of the network-based solution agreed for case 2 and 3 for inter-CU LTM.

With a NW triggered UL synchronization, the UE performs early TA acquisition (step 10 – 11 in Fig.1). However, for inter CU CP LTM new message TA Information transfer in step 11 is transferred from candidate gNB to source gNB which introduces additional delay compared to Rel. 18 LTM. The most critical aspect of the LTM is that the UE has got the correct TA for the moment of the MAC-CE triggered cell switch, i.e. the right timing of the early TA Acquisition (TAA) in relation to the cell switch command (CSC). In case the TAA is not completed before the CSC, the UE will be forced to do normal RACH towards the candidate cell, leading to delays in RACH succeeding and losing the advantage of the LTM mobility procedure. Similarly, the chances of RACH failing also increase. So, it is critical to have the correct value of the TA at the right time at the UE for the overall LTM mobility procedure to succeed.

Observation 2: An extra delay caused by transferring the TA value from candidate to source gNB may have an impact on triggering the Early TA acquisition procedure for inter-CU LTM.

Proposal 2: RAN3 agrees to analyse optimization of triggering the Early TA acquisition procedure for inter-CU LTM.

For Rel. 18 LTM it was agreed in Rel. 19 to re-use the MRO connection failures defined for standard HO and CHO via adding appropriate text into TS 38.300, chapter 15.5.2.2.2 which seems to be valid also for inter- CU LTM.

Proposal 3: RAN3 agrees to re-use the MRO connection failures defined for Rel. 18 LTM also for inter-CU LTM with no enhancement needed in TS 38.300.

### 2.1.4 Near failure monitoring for inter-CU LTM

As part of the NR_SON_MDT_Ph4-Core WI in Rel. 19 MRO for Rel. 18 LTM was provided. The focus was related to enhancement of the SHR report with LTM relevant information. Apart of that RAN3 agreed network-based solution for so called case 1 “BFR shortly after successful LTM cell switch to the wrong beam”. With the introduction of the inter-CU LTM the source DU and target DU may be in different gNBs. Thus, the proposed signaling between the target and source DU will not work for inter-CU LTM.

Observation 3: The network-based concept for case 1 is insufficient for inter-CU LTM.

Proposal 4: RAN3 agrees to analyse the applicability and possible enhancements of the network-based solution agreed for case 1 for inter-CU LTM.

At the end of the Rel. 19 NR_SON_MDT_Ph4-Core WI some of the companies proposed to focus on monitoring of the almost outdated TA value. Even majority of the companies acknowledged the issue there was no consensus on definition of the almost outdated TA value.

Proposal 5: RAN3 agrees to define the almost outdated TA for inter-CU LTM.

Proposal 6: RAN3 agrees to analyse monitoring of almost outdated TA for inter-CU LTM.

# 3	Conclusions

We have the following observations and proposals:

Observation 1: The network-based concept for case 2 and 3 is insufficient for inter-CU LTM.

Proposal 1: RAN3 agrees to analyse the applicability and possible enhancements of the network-based solution agreed for case 2 and 3 for inter-CU LTM.

Observation 2: An extra delay caused by transferring the TA value from candidate to source gNB may have an impact on triggering the Early TA acquisition procedure for inter-CU LTM.

Proposal 2: RAN3 agrees to analyse optimization of triggering the Early TA acquisition procedure for inter-CU LTM.

Proposal 3: RAN3 agrees to re-use the MRO connection failures defined for Rel. 18 LTM also for inter-CU LTM with no enhancement needed in TS 38.300.

Observation 3: The network-based concept for case 1 is insufficient for inter-CU LTM.

Proposal 4: RAN3 agrees to analyse the applicability and possible enhancements of the network-based solution agreed for case 1 for inter-CU LTM.

Proposal 5: RAN3 agrees to define the almost outdated TA for inter-CU LTM.

Proposal 6: RAN3 agrees to analyse monitoring of almost outdated TA for inter-CU LTM.

# References

- RP-251869, RAN #108