3GPP TSG-RAN WG3 Meeting #129bis	R3-256710

Prague, Czech Republic, 13th – 17th October, 2025

Agenda Item:	10.3.2

Source: 	OPPO

Title:  	Discussion on 6G RAN-CN interfaces

Document for:	Discussion and Decision

- Introduction

In Rel-20 6G SID [1], the objectives led by RAN3 are captured as follows.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

Besides, an interim milestone is set for RAN3 as shown below.

#### 4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810]. TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. RAN plenary to make a decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed. NOTE: It is planned to decide on Release-21 timeline in June/2026.

This contribution would like to provide our considerations on RAN-CN interface.

- Discussion

During the migration of core network from 4G to 5G, one of the important evolutions of core network is to transit to service-based architecture, which paves the way for ongoing evolution and enhancements in network capability. Now, in 6G, whether to extend SBA to RAN-CN interface needs to be further studied.

In 5G, NAS signalling from/to UE is carried by NGAP signalling via the NG control plane interface, i.e., relying on AMF to forward the NAS signalling between UE and NFs. The protocol stack is shown as below.

Besides the NAS signalling between UE and AMF for mobility management, i.e., NAS-MM, is transferred over NGAP signalling, the NAS signalling between UE and other NFs, e.g., NAS-SM, SMS, UE policy or LCS, is also transferred over NGAP signalling, which is piggyback on NAS-MM message.

In 5G, the interface between RAN node and AMF is P2P, and the interfaces between AMF and other NFs are SBI-based. In case that heavy NAS signalling overhead is expected in 6G, a centralized AMF and the P2P interface between RAN node and AMF are the bottleneck for NAS signalling handing.

- In case that heavy NAS signalling overhead is expected in 6G, a centralized AMF and the P2P-based RAN-CN interface for control plane connection between RAN node and AMF are the bottleneck for NAS signalling handing.

One potential approach in 6G is to convert P2P-based RAN-CN interface for control plane connection between RAN node and NFs (including AMF) to be SBI-based, which is definitely to be discussed and studied in RAN3. However, for RAN-CN interface for user plane connection, i.e., NG user plane between RAN node and UPF, it seems that no such use case is foreseen currently. Therefore, we understand that it is good to clarify this in the first meeting.

- No use case to support SBI-based RAN-CN interface for user plane connection between RAN node and UPF in 6G.

- RAN3 study SBI-based RAN-CN interface for control plane connection in 6G.

Other than SBI-based RAN-CN interface, another approach in 6G is to consider a design of decoupling “NAS transport” and “access and mobility management” for AMF, with less NAS signalling routing delay and avoiding potential NAS signalling congestion caused by a centralized AMF, i.e., introduce a lightweight NASRF (NAS Routing Function) between 6G RAN node and 6G CN NFs to route the NAS signalling between UE and the according NFs, and 6G AMF only takes responsibility of access and mobility management.

Considering that

1/ NASRF can be deployed closer to 6G RAN node, thus it serves less 6G RAN nodes than a normal AMF,

2/ NASRF is mainly designed for NAS routing, i.e., NASRF is responsible for handling UL/DL NAS TRANSPORT, thus it only aims to support minimal functionality and store the necessary UE context,

the bottleneck for NAS signalling handing due to a centralized AMF and the P2P interface between RAN and AMF could be avoided.

One example of the protocol stack is shown as below.

Note that the interface between 6G RAN node and NASRF is still P2P-based, while the interfaces between NASRF and the according NFs are SBI-based. In this way, RAN3 effort for SBI-based RAN-CN interface could be reduced, since main work is within SA2/CT1 scope.

- RAN3 study RAN3 impact on decoupling “NAS transport” and “access and mobility management” for AMF, to support routing the NAS signalling between UE and the according NFs with less NAS signalling routing delay and avoiding potential NAS signalling congestion caused by a centralized AMF.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- In case that heavy NAS signalling overhead is expected in 6G, a centralized AMF and the P2P-based RAN-CN interface for control plane connection between RAN node and AMF are the bottleneck for NAS signalling handing.

- No use case to support SBI-based RAN-CN interface for user plane connection between RAN node and UPF in 6G.

- RAN3 study SBI-based RAN-CN interface for control plane connection in 6G.

- RAN3 study RAN3 impact on decoupling “NAS transport” and “access and mobility management” for AMF, to support routing the NAS signalling between UE and the according NFs with less NAS signalling routing delay and avoiding potential NAS signalling congestion caused by a centralized AMF.

4. Reference

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO