3GPP TSG-RAN WG3 Meeting #129-bis	R3-256837

Prague, Czech, 13th – 17th Oct 2025

Title: 	Discussion on supporting UL Packet Loss in AI/ML UE Performance feedback

Source: 	CMCC

Agenda item: 	8.1

Document Type:	Discussion and Decision

# Introduction

Currently in R18 AI/ML UE performance feedback, only DL Average Packet Loss is introduced. In RAN3 129bis meeting, there were some discussion on whether it is reasonable to keep the DL Average Packet Loss and whether the UL Average Packet Loss shall also be introduced. The following common sense was captured in chair note:

Wait for Reply LS(es) from SA5

After RAN3 129bis meeting, SA5 replied the LS in R3-253816/S5-253295 to confirm that:

For Average packet loss measurements:

RAN3 would like to further ask whether it is feasible to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity in the UL and DL at PDCP layer, which may also be aggregated to reflect a per-UE Packet Loss Rate metric.

SA5’s answer: SA5 confirms that it is feasible to specify the packet loss/drop measurements with a per-DRB per-UE granularity. SA5 has specified the corresponding packet loss and drop rate measurements in TS 28.558.

And corresponding chapters have been added into TS 28.558 as in R18 CR S5-253899 and R19 CR S5-253900.

# .2	Discussion

This discussion paper will focus on whether per-UE granularity UL Packet Loss.

First, UL Packet Loss is a very important metric to evaluation the UE performance, in 5G, sometimes it is even more important than DL Packet Loss due to the limited uplink bandwidth.

And in R19, per slice per UE level UL Packet Loss is introduced in Slicing use case as one of the UE performance feedback metric.

RAN3 didn’t include UL Packet Loss because no clear definition from SA5 on how to calculate and whether they can get the measurement.

However from LS R3-253816/S5-253295, SA5 includes two CRs (R18 CR S5-253899 and R19 CR S5-253900) which clearly defines the DL PDCP SDU Packet Drop Rate and UL PDCP SDU Packet Loss Rate for both split and non-split architecture.

And in LS reply, SA5 clearly confirms that it is feasible to specify the packet loss/drop measurements with a per-DRB per-UE granularity. SA5 has specified the corresponding packet loss and drop rate measurements in TS 28.558.

So it is clear that SA5 can provide per-UE UL Packet Loss measurement for both split and non-split architecture.

Based on above, we suggest that:

Proposal 1: It is beneficial and possible to introduce per-UE UL Packet Loss metric for both split and non-split architecture.

Proposal 2: It is suggested to discuss and agree R18 CR R3-25xxxx.

Proposal 3: It is suggested to discuss and agree R19 CR R3-25xxxx.

# 3	Summary

In the conclusion, we provide the considerations on UL Packet Loss as below:

Proposal 1: It is beneficial and possible to introduce per-UE UL Packet Loss metric for both split and non-split architecture.

Proposal 2: It is suggested to discuss and agree R18 CR R3-25xxxx.

Proposal 3: It is suggested to discuss and agree R19 CR R3-25xxxx.

# 4	Reference

[1]  RAN3_129_agenda_20250829_1620_EOM, Chair Note

[2] R3-253816, Reply LS on per-UE UE performance metrics

[3] TS 28.558, Management and orchestration; UE level measurements for 5G system

[4] S5-253899, Rel-19 CR TS 28.558 add drop rate measurement for split and non-split architecture

[5] S5-253900, Rel-18 CR TS28.558 Add DL PDCP SDU Drop rate in gNB-CU-UP for UE level