3GPP TSG RAN WG3 Meeting #129-bis	R3-257025

Prague, Czech Republic, 13 - 17 October, 2025

Agenda item:	13.3

Source:	Ericsson, Jio Platforms

Title:	Discussion on RAN-CN procedures and signaling to support ISAC

Document for:	Discussion, Other

# 1	Introduction

In [1], a study item on Integrated Sensing and Communication (ISAC) for NR was approved listing the following RAN3 related objectives:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] - Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary

RAN3#129-bis is the first meeting dedicated to ISAC SI. The RAN3 study on architecture and signaling procedures is expected to follow during five meetings and is expected to be completed at the RAN3#132 meeting (May 2026).

This paper discusses the RAN-CN signaling procedures to support ISAC.

# 2	Discussion

## 2.1	RAN-CN signaling procedures

ISAC (or Sensing) is a new technology that aims to integrate the detection and spatial localization of passive (i.e. unconnected) objects into the 5G RAN services. To support sensing function in NG-RAN, new functionalities for control, sensing resource configuration and sensing processing need to be defined in the 5GS architecture.

For Sensing control, this new functionality relates to the issuance of sensing measurement requests, the management of different sensing sessions and exposure of the sensing results to the sensing client. SA2 have been discussing solutions for a new Sensing Function (SF) in CN as part of their SI TR 23.700-14 [2]. It is likely that a new SF will be defined in the CN with a direct and dedicated interface to NG-RAN for the purpose of sending a request for sensing results from an NG-RAN hosting TRPs capable of monostatic sensing.

- Note: In monostatic sensing setup, a TRP is at the same time transmitting a sensing signal and receiving the echo of this signal from the environment. Compared to legacy TRPs, this requires new transceiver hardware to support Full Duplex operation and self-interference cancellation. However, to bypass this new hardware requirement and complexity to support self-interference cancellation algorithms, it is not precluded that an NG-RAN can support two sensing TRPs: one TRP transmitting and the other TRP receiving while both antennas are slightly separated in distance, so that self-interference is reduced. An example of such is quasi-monostatic setup is detailed here [3]. From NG-RAN and CN interaction perspective, the signaling remains the same.

On the other hand, the Sensing processing relates to functions capable of processing the sensing data captured by the (receiving) TRP to derive detection information such as object presence, object coordinates, range, etc. Such processing functions can include matched filtering, Fast Fourier transform (FFT), Peak detection, clutter removal, triangulation, etc., which for the most part can be considered as baseband level processing functions. Hence, it is logical to consider the processing function as part of the NG-RAN node.

In the same spirit, for sensing resource configuration that includes decision of sensing resources to set up and TRP(s) selection for measurement, as these resources are owned and controlled/managed by NG-RAN such functionality is to be integrated in the NG-RAN. In fact, the choice of TRP should not be mandated by the SF for the following reasons: 1) The SF does not have visibility of the conditions of NG-RAN radio resources for other communication transmission services (nor does it seem to be required to know this). And 2) not all gNBs can support full-duplex operation for monostatic sensing due to hardware limitations, hence the system should allow gNB implementation to support quasi-monostatic sensing with adequate spatial separation between the transmitter (Tx) and receiver (Rx) TRPs.

Proposal 1: The decision about which TRP to select for sensing is up to the gNB. It should not be precluded (up to implementation) for a gNB to select two separate TRPs for quasi-monostatic to bypass transceiver hardware limitation for full duplex mode.

Proposal 2: To support sensing function in NG-RAN, new functionalities for Sensing Control, Sensing Resource configuration and Sensing Processing need to be defined in the 5GS architecture.

- Sensing Control to issue sensing measurement requests, session management and exposure should be hosted by new function in CN.

- Sensing Processing for processing of sensing data into sensing results should be hosted in the NG-RAN.

- Sensing Resource Configuration in terms of TRP selection and resource configuration should be up to NG-RAN node.

Further details on the Sensing architecture are discussed in a separate paper [4].

Based on the above analysis, below we present a signaling flow of the possible RAN-CN interaction for sensing information exchange:

Fig.1 Sensing information exchange between SF and NG-RAN. - The SF sends a SENSING INFORMATION REQUEST message to the NG-RAN node. The message contains list of requested sensing quantities, e.g., type of object to be detected (e.g., drone), target location, velocity, range, etc., with some assistance data (e.g., sensing KPIs, reporting periodicity, FFS) – up to RAN1 to define. - The NG-RAN translates the request and assistance data into resource configuration and selects a suitable TRP to perform the necessary Tx/Rx measurements. (in case of quasi-monostatic, the NG-RAN can select two suitable TRPs, up to NG-RAN implementation). The NG-RAN collects sensing data from the environment and performs internal processing. - Optional: Inter-gNB coordination, if needed, can happen at this point and is up to vendor implementation. - The NG-RAN reports the results of the sensing measurements as processed sensing results to the SF in a SENSING INFORMATION RESPONSE message. The result can contain the presence, velocity, position, etc, of each of the detected objects. - In case of periodic reporting, the NG-RAN initiates the periodic reporting via a SENSING INFORMATION REPORT message including the sensing reports.

The above steps are based on the following principles:

- The SF addresses an NG-RAN node via a new direct P2P connection based on information exchanged via CP signaling. For instance, based on a new NR Sensing Protocol (NRSP). This option has more architecture impacts as it defines a new interface, but brings advantages for latency saving (the message don’t need to be routed via AMF) and flexibility (can be easily enhanced and re-used for 6G sensing).

- For 5G use case (aerial object detection), most processing can be done at RAN to generate the results to be reported: xyz object location, velocity, range, etc. This option avoids burdening the new CP interface with heavy signaling.

Observation 1 : It is possible to design the RAN-CN procedures to support sensing information exchange based on CP signaling via a direct P2P connection. This option has latency savings and ease of upgrading and flexibility advantages.

Proposal 3: Design the RAN-CN procedures to support sensing information exchange based on CP signaling via a direct P2P connection.

Proposal 4: RAN3 to discuss the need to define a sensing information exchange function as part of the new sensing transport protocol.

## 2.2	Association establishment between gNB and SF for Direct interface connection

To support the exchange of controlling messages between the SF and the gNB, the new direct interface is to be considered stateful and the SF needs to be aware of NG-RAN address, so that the nodes can correctly interoperate and be aware of each other’s updated state information. Such state information can include the gNB’s covered sensing areas (FFS on more information). Therefore, we propose that dedicated procedures for Association Establishment, Configuration Update and Association Removal are studied as part of the RAN-CN signaling procedures.

Proposal 5: To support new direct interface between the gNB and the SF to send controlling messages, the SF and gNB need to be aware of each other’s state information. Study procedures for Association Establishment, Configuration Update and Association Removal.

At first, the gNB can be supplied with SF related information based on provisioned OAM information. The gNB initiates the association establishment with SF by sending an Association Establishment Request procedure indicating the gNB’s covered sensing areas. One example of such procedure are described in Figure 2 below.

Fig. 2: NG-RAN initiates the association establishment towards SF - The NG-RAN is configured with the SF address. - The NG-RAN sends an Association Establishment Request to the SF for the sensing service. It includes (among other parameters) its sensing area(s), which is stored by SF. - The SF sends an acknowledgement message to indicate the successful establishment of the association with the NG-RAN. - The SF uses the established association for direct interface to send sensing controlling messages to the NG-RAN (e.g., Sensing Information Request) and the NG-RAN uses the direct interface connection to provide the sensing-related result to the SF.

In case the covered sensing area(s) of the NG-RAN has changed, the NG-RAN sends a Configuration Update request message to the SF. The details are provided below in Figure 3.

Fig.3 Configuration Update of the direct sensing connection between SF and NG-RAN. - The NG-RAN sends a Sensing Configuration Update request to the SF if, e.g., the covered sensing area(s) of NG-RAN node has been changed. The SF updates its stored sensing area if it is required. The SF sends an acknowledgement to the gNB to indicate the successful configuration update.

Finally, the gNB may decide to remove the direct sensing association with the SF. In that case, A Sensing Association Removal Procedure will be needed. The details are described in Figure 4 below.

Fig.4 Removal of the direct sensing connection between SF and NG-RAN. - The NG-RAN sends a Sensing Association Removal request to the SF. The SF updates its stored serving sensing area if it is required. The SF sends an acknowledgement message to NG-RAN about the removal.

Proposal 6: RAN3 to study dedicated procedures for Association Establishment, Configuration Update and sensing association Removal between the NG-RAN and SF to support direct connection setup, update and removal for the new sensing interface.

# 3	Conclusion

This contribution has provided the following observations and conclusions:

Proposal 1: The decision about which TRP to select for sensing is up to the gNB. It should not be precluded (up to implementation) for a gNB to select two separate TRPs for quasi-monostatic to bypass transceiver hardware limitation for full duplex mode.

Proposal 2: To support sensing function in NG-RAN, new functionalities for Sensing Control, Sensing Resource configuration and Sensing Processing need to be defined in the 5GS architecture.

- Sensing Control to issue sensing measurement requests, session management and exposure should be hosted by new function in CN.

- Sensing Processing for processing of sensing data into sensing results should be hosted in the NG-RAN.

- Sensing Resource Configuration in terms of TRP selection and resource configuration should be up to NG-RAN node.

Observation 1 : It is possible to design the RAN-CN procedures to support sensing information exchange based on CP signaling via a direct P2P connection. This option has latency savings and ease of upgrading and flexibility advantages.

Proposal 3: Design the RAN-CN procedures to support sensing information exchange based on CP signaling via a direct P2P connection.

Proposal 4: RAN3 to discuss the need to define a sensing information exchange function as part of the new sensing transport protocol.

Proposal 5: To support new direct interface between the gNB and the SF to send controlling messages, the SF and gNB need to be aware of each other’s state information. Study procedures for Association Establishment, Configuration Update and Association Removal.

Proposal 6: RAN3 to study dedicated procedures for Association Establishment, Configuration Update and Sensing Association Removal between the NG-RAN and SF to support direct connection setup, update and removal for the new sensing interface.

# 4	Reference

- RP-251861, Study on Integrated Sensing And Communication (ISAC) for NR

- SA2 TR 23.700-14 V0.3.0, Study on Integrated Sensing and Communication; Stage 2, (Release 20)

- Y. Lyazidi et al., “ISAC Architecture and Sensing Topology Switching in 6G Cellular Networks”, in proceedings of IEEE CSCN 2024, November 2024. DOI: 10.1109/CSCN63874.2024.10849754

- R3-256922, Discussion on Network Architecture for ISAC, Ericsson, RAN3#129bis

# 5	TP to TR 38.XXX

## Y.1 RAN-CN signaling procedures

### Y.1.1 Association Establishment

Fig. Y1: NG-RAN initiates the association establishment towards SF

- The NG-RAN is configured with the SF address.

- The NG-RAN sends an Association Establishment Request to the SF for the sensing service. It includes (among other parameters) its sensing area(s), which is stored by SF.

- The SF sends an acknowledgement message to indicate the successful establishment of the association with the NG-RAN.

- The SF uses the established association for direct interface to send sensing controlling messages to the NG-RAN (e.g., Sensing Information Request) and the NG-RAN uses the direct interface connection to provide the sensing-related result to the SF.

### Y.1.2 Configuration Update

Fig.Y2 Configuration Update of the direct sensing connection between SF and NG-RAN.

- The NG-RAN sends a Sensing Configuration Update request to the SF if, e.g., the covered sensing area(s) of NG-RAN node has been changed.

- The SF updates its stored sensing area if it is required.

- The SF sends an acknowledgement to the gNB to indicate the successful configuration update.

### Y.1.3 Association Removal

Fig.Y3 Removal of the direct sensing connection between SF and NG-RAN.

- The NG-RAN sends a Sensing Association Removal request to the SF.

- The SF updates its stored serving sensing area if it is required.

- The SF sends an acknowledgement message to NG-RAN about the removal.

### Y.1.4 Sensing Information Exchange

Fig.Y4 Sensing information exchange between SF and NG-RAN.

- The SF sends a SENSING INFORMATION REQUEST message to the NG-RAN node. The message contains list of requested sensing quantities, e.g., type of object to be detected (e.g., drone), target location, velocity, range, etc., with some assistance data (e.g., sensing KPIs, reporting periodicity, FFS) – up to RAN1 to define.

- The NG-RAN translates the request and assistance data into resource configuration and selects a suitable TRP to perform the necessary Tx/Rx measurements. (in case of quasi-monostatic, the NG-RAN can select two suitable TRPs, up to NG-RAN implementation). The NG-RAN collects sensing data from the environment and performs internal processing.

  - Optional: Inter-gNB coordination, if needed, can happen at this point and is up to vendor implementation.

- The NG-RAN reports the results of the sensing measurements as processed sensing results to the SF in a SENSING INFORMATION RESPONSE message. The result can contain the presence, velocity, position, etc, of each of the detected objects.

- In case of periodic reporting, the NG-RAN initiates the periodic reporting via a SENSING INFORMATION REPORT message including the sensing reports.