3GPP TSG-RAN WG3  Meeting #129bis	R3-257053

Czech Republic, October 13th -17th 2025

Source: 	Vodafone

Title: 		Missing UE radio paging capabilities

Document for: decision

Agenda Item: xxx

Introduction

During RAN WG2#129bis, the group revisited the issue of UE radio paging capability loss (e.g., LP-WUS subgrouping). They agreed to develop a new Release-19 container, as outlined in LS R2-2503172.

- Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced.   This is applicable only to new Rel-19 and future capabilities added to paging capabilities.  This will only work for a gNB that supports this Rel-19 “container”.

In our view, the primary reason for this decision was the discovery that the Release 17 container (UE-RadioPagingInfo) was not extendable.

SA2 replied on this LS with S2-2506087 stating that:

- It is essential that operators are able to use RAN equipment from different vendors and that competition between those RAN vendors is not inhibited by RAN 2 requiring all RAN vendors to upgrade their equipment before new UE functionality can be used with the first RAN vendor to develop a solution.

The objective of this paper is to explain the mechanism of capability exchange and to propose an efficient RAN-based solution for deployment scenarios where not all base stations within a PLMN operate on the latest 3GPP release.

UE Capability handling

The UE capability handling can be divided into 2 phases:

Phase 1: The initial part happens when the UE performs Initial Registration to the PLMN, or, the UE performs the first Registration Area Update after Attach on another RAT.

Phase 1:

- The AMF receives NAS signalling containing Initial Registration (or first RAU after attach in another RAT).

- This NAS message (Initial Registration) causes the AMF to send NGAP Initial Context Setup Request message to gNB without UE Radio Capability.

- The absence of the UE Radio Capability in that message causes the gNB to perform the UE Capability Enquiry procedure as per TS 38.331 and,

- After the UE Radio Capability is received from the UE, the gNB will send the NGAP UE Radio Capability Info Indication (which carries the UE Radio Capability) to the AMF.

- Starting with release 15, the gNB analyses the UE Radio Capability; generates the UE Radio Capability for Paging and includes it in the UE Radio Capability Info Indication sent from the gNB to the AMF. The received capabilities are stored in AMF.

Phase 2:

- At subsequent RRC connection establishments the AMF receives a service request as a NAS message,

- This case the AMF to send NGAP Initial Context Setup Request and within this message AMF provides both the UE Radio Capability and the UE Radio Capability for Paging to the gNB

- As“UE Radio Capability for Paging” is included in this message the gNB or ng-eNodeB  can check that the UE Radio Capability for Paging contains information on all the NG-RAN RATs that the UE supports.

- If paging capability information on one of the UE’s RATs is missing, then TS 23.501 section 5.4.4.3 requires the NG-RAN to obtain the missing paging capability information and update what is stored in the AMF so that subsequent NGAP Paging messages carry all relevant information.

TS 23.501, 5.4.4.3 Paging assistance information:

“As the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF and the AMF may be connected to more than one NG-RAN RAT, it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging Information (which is derived by the NG-RAN node) contains information on all NG-RAN RATs that the UE supports in that PLMN. To assist the NG-RAN in this task, as specified in TS 38.413 [34], the AMF provides its stored UE Radio Capability for Paging Information in every NG-AP INITIAL CONTEXT SETUP REQUEST message sent to the NG-RAN.”

Phase 1: Capability Transfer during Initial Registration

Phase 2: The subsequent connections of the UEs.

Phase 2: UE Paging Capability Transfer during Subsequent Service Request

Problem statement

During a subsequent connection, the gNB receives the UE RAC and the UE Radio Capability for Paging information, but it does not know if the gNB that provided the UE Radio Capability for Paging information to the AMF was Release 17, 18 or 19 (See also UERadioPagingInformation in 38.331).

Please note, that Rel 17 includes enhancements for Inactive UEs, Release 18 for PEI and Rel 19 for LP-WUS within UE Radio Capability for Paging.

The LS from SA2 suggests a mechanism (based on the solution for release 15’s “late drop” for LTE base stations connected to 5GC) by which gNB can ensure that the AMF is given all of the UE’s paging capability information:

- The gNB processes the UE Radio Capability in the Initial Context Setup Request and generates the UE Radio Capability for Paging Information from the received UE Radio Capability.

- The gNB compares what it has generated with the UE Radio Capability for Paging received in the Initial Context Setup Request, and

- if they are different, the gNB shall update the AMF with the correct UE Radio Capability for Paging Information.

Observation 1: Every gNB would need to process the UE RAC in every Initial Context Setup Request for every UE.

Proposed Solution:

To reduce the processing load on the gNB, the following is proposed:

A new field can be introduced in the UE Radio Capability for Paging to indicate according to which release of 3GPP the gNB verified the UE Radio Capability for Paging information. When the UE establishes a connection with a subsequent gNB, this field allows new gNB to easily identify if it needs to process UE RAC and update UE Radio Capability for Paging. If the flag is present and indicates UE Radio Capability for Paging Information provided is according to the release of the new gNB, the gNB may skip checking, optimizing processing.

An additional advantage of this solution is its transparency to both the AMF and the UE. It requires only minor enhancements and can be implemented starting from Release 17 onwards.

A suggested field encoding is given below:

#### 9.3.1.68	UE Radio Capability for Paging

This IE contains paging specific UE Radio Capability information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Radio Capability for Paging of NR | O |  | OCTET STRING | Includes the UERadioPagingInformation message as defined in TS 38.331 [18]. |
| UE Radio Capability for Paging of E-UTRA | O |  | OCTET STRING | Includes the UERadioPagingInformation message as defined in TS 36.331 [21]. |
| UE Radio Capability for Paging of NB-IoT | O |  | OCTET STRING | Includes the UERadioPagingInformation-NB message as defined in TS 36.331 [21]. |
| UE Radio Capability for Paging check status | O |  | BIT STRING (SIZE(8)) | This informs the gNB if the UE Radio Capability for Paging has been verified against Release 17 and Rel 18 and Rel 19 TS 38.331 ASN.1 Bit0 is set to 1 if CheckedbygNBagainstRelease17orlater, otherwise 0 Bit1= CheckedbygNBofRelease18 or later, otherwise 0, Bit2= CheckedbygNBofRelease19 or later, otherwise 0 Bit 3-7 are set to 0 and if received as 1, shall be ignored in this release of specification. |

Note: The bit string encoding is targeting the case that if e.g. Rel 18 NB receives the indication that precious gNB checked the status according to release 19 functionalities, the Rel 18 gNB does not process UE RACs.

Proposal: It is proposed to add a new field (UE Radio Capability for Paging check status) to the UE Radio Capability for Paging.

Conclusion:

It is proposed to add a new field (UE Radio Capability for Paging check status)  to the UE Radio Capability.

Annex: Text extract from TS 38.413 (NGAP) and TS 38.331

TS 38.413:

### 9.2.13	UE Radio Capability Management Messages

#### 9.2.13.1	UE RADIO CAPABILITY INFO INDICATION

This message is sent by the NG-RAN node to provide UE radio capability related information to the AMF.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| UE Radio Capability | M |  | 9.3.1.74 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| UE Radio Capability – E-UTRA Format | O |  | 9.3.1.74a |  | YES | ignore |
| XR Device with 2Rx | O |  | ENUMERATED (true, …) | Indicates the UE is a 2Rx XR UE as defined in TS 38.300 [8]. | YES | ignore |

#### 9.2.13.2	UE RADIO CAPABILITY CHECK REQUEST

This message is sent by the AMF to request the NG-RAN node to check the compatibility between the UE radio capabilities and network configuration on IMS voice.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| UE Radio Capability | O |  | 9.3.1.74 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |

#### 9.2.13.3	UE RADIO CAPABILITY CHECK RESPONSE

This message is sent by the NG-RAN node to report IMS voice compatibility between the UE radio capabilities and network configuration.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| IMS Voice Support Indicator | M |  | 9.3.1.89 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

TS 38.331:

– UE-RadioPagingInfo

The IE UE-RadioPagingInfo contains UE capability information needed for paging.

UE-RadioPagingInfo information element

-- ASN1START

-- TAG-UE-RADIOPAGINGINFO-START

UE-RadioPagingInfo-r17 ::= SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

...

}

-- TAG-UE-RADIOPAGINGINFO-STOP

-- ASN1STOP