3GPP TSG-RAN WG3 Meeting #129	R3-255147

Bengaluru, India, 25 – 29 August 2025

# Title: 	Discussion on the configuration in case of CHO with CPAC

# Source: 	ZTE Corporation, CATT, China Telecom, China Unicom

Agenda item:	9.2

# Document for:	Discussion and Decision

- Introduction

In Rel-16, we have introduced the CHO with SN addition feature.

In Rel-17, we have introduced the CHO with CPAC feature.

In this paper, for simplicity, we use the following terminologies:

- “CHO only” to indicate both CHO without SCG and CHO with SCG,

- “CHO with CPAC” to indicate CHO with candidate SCG(s) in TS 37.340.

In current TS38.423, the related IEs are introduced in the SN addition request message as below.

TS38.423

9.1.2.1	S-NODE ADDITION REQUEST

| CHO Information SN Addition | O |  |  |  |
| --- | --- | --- | --- | --- |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  |
| Conditional PSCell Addition Information Request | O |  |  |  |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. |
| >S-CPAC Request Information | O |  | 9.2.3.192 | Indicates that SN addition is for S-CPAC preparation. |
| >S-CPAC Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for S-CPAC is requested. |

After the previous email discussion, we have the following common understanding.

- It is the target MN to decide either CHO only or CHO with CPAC, after receiving HO request message from source MN and before sending SN addition request message to the (candidate) SN.

- If the target MN decides to configure CHO only, it is only allowed to send “CHO Information SN Addition“IE in the SN addition request message, it is not allowed to send “Conditional PSCell Addition Information Request” IE in the SN addition request message.

- If the target MN decides to configure CHO with CPAC, it shall send “Conditional PSCell Addition Information Request” IE in the SN addition request message.

This paper addresses the issue on if the target MN decides to configure CHO with CPAC, whether it can send “CHO Information SN Addition“IE in the SN addition request message.

- Discussion

If the target MN decides to configure CHO with CPAC, it shall send “Conditional PSCell Addition Information Request” IE in the SN addition request message. Then, the target SN can know this configuration.

TS38.423

9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. |
| SCG Activation Status | O |  | 9.2.3.155 |  |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  |
| >Candidate PSCell List |  | 1 |  | Ignored, if the Candidate PSCell with Other Information List IE is included. |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| >Candidate PSCell with Other Information List |  | 0..1 |  |  |
| >>Candidate PSCell with Other Information Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| >>>S-CPAC Complete Candidate Configuration Indicator | O |  | Complete Candidate Configuration Indicator 9.2.3.194 |  |
| CHO Information SN Addition Acknowledge | O |  |  |  |
| >PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 | PCell indicated in the corresponding S-NODE ADDITION REQUEST message. |
| Direct Forwarding Path Availability with source M-NG-RAN node | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target S-NG-RAN node and source M-NG-RAN node. |

Observation 1: The target SN shall know whether CHO only or CHO with CPAC is configured. If CHO only configured, it shall send CG-Config message in the SN to MN RRC container, if CHO with CPAC configured, it shall send CG-CandidateList message in the SN to MN RRC container.

TS38.423

9.1.1.1	HANDOVER REQUEST

| Conditional Handover Information Request | O |  |  |  |
| --- | --- | --- | --- | --- |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  |
| >Conditional Handover Time Based Information | O |  |  | This IE only applies to NTN. |
| >>Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to information provided in the t1-Threshold contained in the ReportConfigNR IE as defined in TS 38.331 [10] |
| >>Handover Window Duration | M |  | INTEGER (1..6000) | Corresponds to information provided in the duration contained in the condEventT1 contained in the ReportConfigNR IE as defined in TS 38.331 [10] |
| >Maximum Number of Conditional Reconfigurations to Prepare | O |  | INTEGER (1..8 , …) | Indicates that the target NG-RAN node may prepare for CHO with candidate PSCell(s) configuration and indicates the maximum number of conditional reconfigurations that the target NG-RAN node may prepare. |

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. Ignored if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  |
| Conditional Handover Information Acknowledge | O |  |  |  |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  |
| >CHO-CPAC Information | O |  | 9.2.3.202 |  |

9.2.3.202	CHO-CPAC Information

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHO-CPAC Configuration Indicator | O |  | ENUMERATED (cho-only-not-prepared, …) | The value “cho-only-not-prepared” indicates that either CHO without SCG or CHO with SCG has not been prepared. |
| Multiple Target S-NG-RAN Node List |  | 1 |  |  |
| >Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  |
| >>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  |
| >>PDU Session Resources Admitted List | M |  | 9.2.1.2 |  |
| >>Candidate PSCell List |  | 1 |  |  |
| >>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |
| >>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| >>>>Target NG-RAN node To Source NG-RAN node Container | M |  | OCTET STRING | Includes the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. |

If the target MN decides to configure CHO with CPAC, after receiving SN addition request ACK message from SN, it shall send “CHO-CPAC Information” IE in the “Conditional Handover Information Acknowledge” IE in the HO requet ACK message. Then the source MN can ignore the related mandatory IEs in the HO request ACK message.

Observation 2: If CHO with CPAC configured, the “CHO-CPAC Configuration Indicator” IE set to "cho-only-not-prepared"to shall be included in the HO request ACK message, then the source MN can ignore the mandatory IEs.

Based on the above common understanding and observations, we further discuss the following issues.

- If the target MN decides to configure CHO with CPAC, whether it is allowed to send “CHO Information SN Addition“IE in the SN addition request message.

- If the target SN receives both “CHO Information SN Addition“ IE and ”Conditional PSCell Addition Information Request“ IE in the SN addition request message, how the SN knows whether it is CHO only or CHO with CPAC.

Solution 1: In case of CHO with CPAC configured, “CHO Information SN Addition” IE can be either included or not included in the SN addition request message.

If the target SN receives “Conditional PSCell Addition Information Request“ IE in the SN addition request message, no matter receiving or not receiving “CHO Information SN Addition“ IE in the same message, it always considers it is the CHO with CPAC configuration.

Solution 2: In case of CHO with CPAC configured, “CHO Information SN Addition” IE cannot be in the SN addition request message.

The target MN shall not send “CHO Information SN Addition” IE but shall send “Conditional PSCell Addition Information Request” IE, in the same SN addition request message.

Solution 3: In case of CHO with CPAC configured, “CHO Information SN Addition” IE must be in the SN addition request message.

Proposal 1: RAN3 shall clarify that in case of CHO with CPAC configured, whether or not“CHO Information SN Addition” IE can be in the SN addition request message.

Proposal 2: In case of CHO with CPAC configured,

Solution 1: “CHO Information SN Addition” IE is allowed in the SN addition request message, or

Solution 2 “CHO Information SN Addition” IE cannot be in the SN addition request message, or

Solution 3: “CHO Information SN Addition” IE must be in the SN addition request message

- Conclusion

Based on the above analysis, we provide the following observations and proposals.

Observation 1: The target SN shall know whether CHO only or CHO with CPAC is configured. If CHO only configured, it shall send CG-Config message in the SN to MN RRC container, if CHO with CPAC configured, it shall send CG-CandidateList message in the SN to MN RRC container.

Observation 2: If CHO with CPAC configured, the “CHO-CPAC Configuration Indicator” IE set to "cho-only-not-prepared"to shall be included in the HO request ACK message, then the source MN can ignore the mandatory IEs.

Proposal 1: RAN3 shall clarify that in case of CHO with CPAC configured, whether or not“CHO Information SN Addition” IE can be in the SN addition request message.

Proposal 2: In case of CHO with CPAC configured,

Solution 1: “CHO Information SN Addition” IE is allowed in the SN addition request message, or

Solution 2 “CHO Information SN Addition” IE cannot be in the SN addition request message, or

Solution 3: “CHO Information SN Addition” IE must be in the SN addition request message

For the solution 1, current specification does not need to be modified. For the solution 2 and 3, we provide the following draft CR in the annex.

We also provide a separate CR (R3-255148) for the solution 2 and solution 3.

- Annex

  - Solution 2 “CHO Information SN Addition” IE cannot be in the SN addition request message

////////////////////////////////////////////////////////////// Start of change /////////////////////////////////////////////////////////////////////

10.19	Conditional Handover with Secondary Node

<Skip unchanged part>

10.19.2	MR-DC with 5GC

The Conditional Handover with Secondary Node procedure is used for configuration and execution of CHO with SN or CHO with candidate SCG(s). This procedure includes the cases where the SN is kept, changed or added. If the SN is kept, the UE context at the SN is kept. If the SN is changed, the UE context at the source SN is moved to the target SN.

CHO with candidate SCG(s) is not supported for NE-DC and NGEN-DC.

<Skip unchanged part>

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. In case of CHO with SN addition, within the SN Addition Request message, the candidate MN also includes the CHO related information, i.e., the source MN ID and the MN UE XnAP ID in the source MN, in order to indicate that the SN Addition Preparation procedure is triggered in relation to a CHO and to enable the SN to identify requests related to the same UE. In case of CHO with candidate SCG(s), the candidate MN also provides the candidate PSCells recommended by the candidate MN via the latest measurement results for the candidate SN(s) to choose and configure the candidate SCG cell(s), and provides the maximum number of PSCells that the candidate SN can prepare for the UE in the SN Addition Request message.

NOTE 3a:	The target MN and other potential target MNs may trigger the SN Addition Preparation procedure to the same (target) SN.

NOTE 3b:	The source MN may initiate additional Xn Handover Preparation procedures towards the same or other target MNs. Based on each Xn Handover Preparation procedure, each target MN may decide to trigger SN Addition Preparation procedure.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration. In case of CHO with candidate SCG(s), within the list of cells as indicated within the measurement results indicated by the candidate MN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the candidate MN) and, for each prepared PSCell, the candidate SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the candidate MN in an NR RRC reconfiguration** message contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s).

NOTE 4:	In CHO with SCG configuration, it is up to the candidate MN implementation to make sure that the CG-Config provided from the (candidate) SN can be used in all CHO preparations.

NOTE 4A1:	In case of CHO with candidate SCG(s), the (candidate) SN assigns the same data forwarding addresses for multiple data forwarding requests from different candidate MNs and may set up only one bearer context. Then the (candidate) SN indicates to the candidate MN direct data forwarding path availability with the source SN and/or source MN, if applicable.

<Skip unchanged part>

////////////////////////////////////////////////////////////// End of change /////////////////////////////////////////////////////////////////////

  - Solution 3: “CHO Information SN Addition” IE must be in the SN addition request message

///////////////////////////////////////////////////////////// Start of change /////////////////////////////////////////////////////////////////////

10.19	Conditional Handover with Secondary Node

<Skip unchanged part>

10.19.2	MR-DC with 5GC

The Conditional Handover with Secondary Node procedure is used for configuration and execution of CHO with SN or CHO with candidate SCG(s). This procedure includes the cases where the SN is kept, changed or added. If the SN is kept, the UE context at the SN is kept. If the SN is changed, the UE context at the source SN is moved to the target SN.

CHO with candidate SCG(s) is not supported for NE-DC and NGEN-DC.

<Skip unchanged part>

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. In case of both CHO with SN and CHO with candidate SCG(s), within the SN Addition Request message, the candidate MN also includes the CHO related information, i.e., the source MN ID and the MN UE XnAP ID in the source MN, in order to indicate that the SN Addition Preparation procedure is triggered in relation to a CHO and to enable the SN to identify requests related to the same UE. In case of CHO with candidate SCG(s), the candidate MN also provides the candidate PSCells recommended by the candidate MN via the latest measurement results for the candidate SN(s) to choose and configure the candidate SCG cell(s), and provides the maximum number of PSCells that the candidate SN can prepare for the UE in the SN Addition Request message.

NOTE 3a:	The target MN and other potential target MNs may trigger the SN Addition Preparation procedure to the same (target) SN.

NOTE 3b:	The source MN may initiate additional Xn Handover Preparation procedures towards the same or other target MNs. Based on each Xn Handover Preparation procedure, each target MN may decide to trigger SN Addition Preparation procedure.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration. In case of CHO with candidate SCG(s), within the list of cells as indicated within the measurement results indicated by the candidate MN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the candidate MN) and, for each prepared PSCell, the candidate SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the candidate MN in an NR RRC reconfiguration** message contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s).

NOTE 4:	In CHO with SCG configuration, it is up to the candidate MN implementation to make sure that the CG-Config provided from the (candidate) SN can be used in all CHO preparations.

NOTE 4A1:	In case of CHO with candidate SCG(s), the (candidate) SN assigns the same data forwarding addresses for multiple data forwarding requests from different candidate MNs and may set up only one bearer context. Then the (candidate) SN indicates to the candidate MN direct data forwarding path availability with the source SN and/or source MN, if applicable.

<Skip unchanged part>

////////////////////////////////////////////////////////////// End of change /////////////////////////////////////////////////////////////////////