3GPP TSG-RAN WG3 Meeting #129	R3-255156

Bengaluru, Karnataka, India, August 25th – 29th 2025

Agenda Item:	12.2

Source:	Ericsson, Jio Platforms

Title:	Functional Aspects of WAB-Nodes

Document for:	Agreement

# Introduction

In this paper we continue the normative discussion on functional aspects of WAB-nodes.

A reply to a previous LS from SA2 is provided in the Annex.

# Discussion

We discuss the following functional aspects of WAB-nodes:

- Additional ULI.

- Xn connection management.

- Indication of backhaul link type.

- Multihop topology prevention for WAB.

## Additional ULI

The RAN3#127-bis meeting notes capture the following:

RAN3 to agree the WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures.

It is FFS whether the additional ULI shall be removed from NG UE associated procedures

In our understanding, AULI was introduced in UE-associated (UA) NGAP signalling and non-UE-associated (NUA) NGAP signalling, but for different reasons. Namely, AULI was introduced in UA NGAP signalling to support location-based services for UEs served by WAB-nodes. On the other hand, the reason for introducing the ULI in NUA NGAP signalling was to properly support the PWS message delivery. Therefore, as explained in an earlier contribution (R3-251583), if the AULI is only delivered by means of UA NGAP signalling, then the AMF has up-to-date AULI only if there is at least one UE in RRC_CONNECTED state served by the WAB-node. Consequently, to remove the dependency of accurate PWS delivery on whether there are RRC_CONNECTED UEs in the WAB-gNB cell, RAN3 decided to introduce AULI in NGAP NUA signalling.

Observation 1: Additional ULI was introduced in UE-associated NGAP signalling and non-UE-associated NGAP signalling for different reasons.

With respect to the FFS, we think that removing AULI from NUA NGAP signalling is not preferred because it would impact the AMF logic. Roughly speaking, an AMF maintains UE contexts separately from “gNB contexts”. If AULI is kept in both NUA and UA NGAP signalling, an AMF receiving AULI in a UA message is directly able to associate the AULI with a UE, based on the NGAP UE IE received in the UA message. On the other hand, if AULI is removed from UA NGAP signalling, and delivered only by means of NUA NGAP signalling (which does not contain any NGAP UE IDs), to associate the AULI with relevant UEs, the AMF needs to implement an extra logic to search the gNB context and identify the relevant UEs (i.e., the UEs served by the sending WAB-gNB).

Based on the above, we propose keeping the Additional ULI IE in NGAP UA messages.

Proposal 1: Keep the Additional ULI IE in NGAP UE-associated messages.

## Xn connection management

The agreements from RAN3#127-bis and RAN3#128 state:

Include a WAB-MT Identifier in the XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK e.g. for colocation discovery for resource multiplexing or for WAB node indication.

The “WAB-MT ID” sent from the WAB-gNB to the BH-gNB consists of the WAB-MT’s C-RNTI assigned by the BH-gNB and the cell id of BH-gNB´s cell serving the WAB MT.

It is possible to establish an Xn connection between two WAB-gNBs. It is possible to prevent establishment of such connections.

Meanwhile, two related TBCs from the RAN3#128 meeting state:

The WAB-gNB should be notified about the target BH-gNB before the WAB-MT HO.

The BH-gNB can provide the TNL information of neighbour gNBs to the WAB node.

We think that the first TBC should be turned into an agreement. In general, the Xn interface was not designed for scenarios where it would be dynamically set up and released. Xn interface setup may take prohibitive amounts of time in WAB scenarios where the WAB-MT is handed over to a target BH-gNB that had no Xn connection with the WAB-gNB prior to the HO.

If the WAB-gNB did not have an Xn connection with the target BH-gNB prior to WAB-MT HO, Xn interface setup towards the target BH-gNB can be accelerated by source BH-gNB notifying the WAB-gNB about the target (BH-)gNB prior to the WAB-MT HO.

In case the WAB-gNB already has an Xn connection with the target BH-gNB (e.g., because the target BH-gNB is a neighbour of the WAB-gNB), the WAB-gNB needs to know which of its Xn neighbours is the new target BH-gNB. Based on this knowledge, the WAB-gNB will be able to indicate to the correct (i.e., target) BH-gNB that it is co-located with the WAB-MT. This also speaks in favour of notifying the WAB-gNB about the target BH-gNB for the WAB-MT HO. The most useful way to identify the target BH-gNB is via its TNL information.

Proposal 2-1: The source BH-gNB can provide to the WAB-gNB the TNL information of the target BH-gNB before the WAB-MT HO.

We also support turning the second TBC into an agreement because it enables quicker establishment of WAB-gNB’s neighbour relations.

Proposal 2-2: The source BH-gNB can provide to the WAB-gNB the TNL information of its neighbour gNBs.

## Indication of backhaul link type

At the RAN3#128 meeting, it was discussed whether the UE’s 5GC should be aware that there exists an NTN link on the path towards the WAB-gNB. The two corresponding TBCs state:

If non-terrestrial link is used between WAB MT and BH gNB and/or between BH gNB and BH CN, the WAB-gNB informs UE’s CN that the BH includes a non-terrestrial link.

FFS how a WAB node know the BH-gNB is using a non-terrestrial link. Possible options include BH-gNB informs WAB-gNB via Xn.

We think that the UE’s 5GC should be aware of whether any of the BH links on the path between the UE’s 5GC and the WAB-node is a terrestrial (TN) or a non-terrestrial one (NTN). This information could be used by the UE’s 5GC for setting a proper QoS of the PDU sessions towards the UEs.

According to the TBCs above, we can distinguish between two cases with respect to the placement of an NTN link in the WAB architecture:

- Case 1: The NTN link is between the BH-gNB and the WAB-MT.

- Case 2: The NTN link is between the BH-gNB and the BH-5GC.

In Case 1, the WAB-MT is aware of the NTN BH link. This is because the WAB-MT needs to be aware of its GNSS position and needs to compensate for Doppler and propagation delay incurred by the NTN link. Given that the WAB-gNB and the WAB-MT are in the same physical “box”, the WAB-gNB is aware of the NTN nature of the WAB-MT’s BH link as well.

In Case 2, the WAB-node is unaware of whether the BH-gNB uses a TN or an NTN BH. If the UE’s 5GC is to be informed about an NTN link on BH-gNB’s BH, the BH-gNB first needs to inform the WAB-gNB, which, in turn, needs to inform the UE’s 5GC. This requires the inclusion of a wireless BH type indication in the XnAP signalling for interface management, and in the corresponding NGAP signalling.

Proposal 3: Include an indication of wireless backhaul type (terrestrial or non-terrestrial) in:

- NGAP interface management signalling, between a WAB-gNB and the UE’s 5GC.

- XnAP interface management signalling, between a WAB-gNB and the BH-gNB serving the WAB-MT co-located with the WAB-gNB.

## Multihop topology prevention for WAB

At the RAN3#127-bis meeting, RAN3 acknowledged that Solution 1 for multihop topology prevention is implementation-specific and not subject to specifications changes. In addition, the RAN3#127-bis meeting notes capture the following FFS:

It is FFS whether Solution 3, requiring impacts to SIBs, can be agreed. If there is consensus in RAN3 on an agreement for solution 3, RAN2 involvement is needed.

In our view, Solution 1 offers sufficiently many options of multihop prevention, and no further discussion is necessary.

Proposal 4: No further discussion on WAB multihop topology prevention in Rel-19.

# Conclusion

This paper discusses the functional aspects of WAB-node. The following is observed and proposed:

Observation 1: Additional ULI was introduced in UE-associated NGAP signalling and non-UE-associated NGAP signalling for different reasons.

Proposal 1: Keep the Additional ULI IE in NGAP UE-associated messages.

Proposal 2-1: The source BH-gNB can provide to the WAB-gNB the TNL information of the target BH-gNB before the WAB-MT HO.

Proposal 2-2: The source BH-gNB can provide to the WAB-gNB the TNL information of its neighbour gNBs.

Proposal 3: Include an indication of wireless backhaul type (terrestrial or non-terrestrial) in:

- NGAP interface management signalling, between a WAB-gNB and the UE’s 5GC.

- XnAP interface management signalling, between a WAB-gNB and the BH-gNB serving the WAB-MT co-located with the WAB-gNB.

Proposal 4: No further discussion on WAB multihop topology prevention in Rel-19.

# Annex: [DRAFT] Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control)

3GPP TSG-RAN WG3 Meeting #129	R3-25xxxx

Bengaluru, Karnataka, India, August 25th – 29th 2025

Title:               Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access)

Response to:	R3-250071/S2-2501324

Release:	Rel-19

Work Item:	FS_VMR_Ph2, NR_WAB_5GFemto-Core

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact Person:

Name:		Filip Barac

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  -

1. Overall Description:

RAN3 thanks SA2 for the LS reply on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI) in S2-2501324.

For access control upon mobility, RAN3 captured the following in the WAB BL CR for TS 38.401:

Upon Xn-based WAB-MT handover, the target WAB-gNB can optionally reject a Handover Preparation procedure that includes PDU session(s) mapped to the WAB-dedicated S-NSSAI(s) used for backhauling.

For access control upon initial access, RAN3 captured the following in the RAN3#127-bis meeting:

Solution 1 is implementation specific and it is not subject to specifications changes.

As a reminder, the definition of Solution 1 is:

For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meeting:

RAN3#129-bis		Oct 13th – 17th 2025		Prague, Czech Republic