3GPP TSG-RAN WG3 Meeting #129	R3-255165

Bangalore, India, 25 – 29 August, 2025

Agenda item:	10.2

Source:	Nokia

Title:	MRO Enhancements for CHO with Candidate SCG(s)

Document for:	Discussion

# 1	Introduction

This is a contribution related to MRO for CHO with candidate SCG(s) in Rel-19.

# 2	Discussion

## 2.1 MRO for CHO with Candidate SCG(s)

### 2.1.1 Agreements achieved in RAN3

The agreements achieved in RAN3 #128 [1]:

MRO for CHO with Candidate SCG(s)

Reuse the existing “HO too early” and “HO to wrong cell” in case of too early CHO execution and CHO execution to wrong cell.

Handover Report message is not needed for case 7a.

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

The agreements achieved in RAN3 #127bis [2]:

No additional information is needed in handover report message.

Check stage 3 TP on the Target cell CGI related in handover report message.

One SCGFailureInfomation is included in SCG Failure Indication message, no need aggregation.

Respective MN in stage2 is at least a candidate MN.

Information needed in the SCG Failure Indication message: The Xn UE AP ID in the receiving node.

Additional information reported from the UE in SCGFailureInformation: The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered.

No support of near failure case in R19.

Whether Handover Report message is needed for case 7a?

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

The agreements achieved in RAN2 #130 [3]:

FFS if we add some correlation indication.

We will not add any optimizations to avoid duplicated info in correlated reports.

The agreements achieved in RAN2 #129bis [4]:

No new triggering conditions such as time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) is above a threshold, can be considered for SHR/SPR procedure.

No new triggering conditions for SHR/SPR procedure for CHO with candidate SCG.

UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs.

UE includes the target PCell ID in SPR for successful CHO with candidate SCG.

In general, and where applicable, agreements valid for SHR, RLF reports and SCG failure info applies also to SPR.

### 2.1.3 Connection Failure Monitoring for CHO with Candidate SCG(s)

In RAN3 #127bis the following was proposed still clarified in the next meeting:

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

CPAC execution conditions are provided by target MN. Thus, in case the SCG Failure Indication is communicated to source MN which applies for case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) it is feasible for target MN to identify the CPAC Failure type however such information has no added value for source MN as intention is to modify CHO execution conditions and the applicable CPAC Failure type is only CPA/CPC to wrong cell. Too late CPA/CPC not applicable at all as CHO with candidate SCG(s) was executed. Nor too early CPC is applicable as problem is related to delayed CHO execution conditions fulfilment.

Proposal 1-1: RAN3 agrees not to send CPAC Failure type in SCG Failure Indication message from target to source MN.

During the RAN3 # 128 meeting some of the companies insisted on usefulness of sending the CPAC Failure type in SCG Failure Indication message from source to target MN which applies for case 1b, 3b, 9b. Their argument was that it is source MN which receives SCG Failure Information from UE and thus may provide the CPAC Failure Type. It shall be however noted that providing such CPAC failure type would have no practical meaning for target MN. The target MN must provide an own analysis to identify for example whether in case of CPAC to wrong cell the suitable PSCell was included in the candidate target PSCells list or not to properly identify it is the target MN or target SN issue (refer to TS 37.340). It shall be noted that for source MN the candidate target PSCell list provided by target MN is not known at all.

Proposal 1-2: RAN3 agrees not to send CPAC Failure type in SCG Failure Indication message from source to target MN.

# 3	Conclusions

We have the following observations and proposals to enhance MRO for CHO with candidate SCG(s):

Proposal 1-1: RAN3 agrees not to send CPAC Failure type in SCG Failure Indication message from target to source MN.

Proposal 1-2: RAN3 agrees not to send CPAC Failure type in SCG Failure Indication message from source to target MN.

# References

- Report of 3GPP TSG RAN3 meeting #128

- Report of 3GPP TSG RAN3 meeting #127bis

- Report of 3GPP TSG RAN2 meeting #130

- Report of 3GPP TSG RAN2 meeting #129bis