3GPP TSG-RAN WG3 Meeting #129	R3-255196

Bengaluru, IND, 25th–29th August, 2025

Title	:	Open issues related to Rel 19 AIoT Procedures

Source	:	Tejas Networks Limited

Agenda item	: 16.2

Document Type	: Discussion

# 1.Introduction

In this Tdoc, we propose solutions to address some of the remaining open issues that need to be resolved during the RAN3#129 meeting.

# 2. Discussion

2.1 Time Interval for result aggregation in Inventory Request message

It is FFS on the Time Interval as assistance information in the Inventory Request message. Clarify with SA2 on the usage of this information in NG-RAN node.

It is FFS on the Time Interval as assistance information in the Inventory Request message. Clarify with SA2 on the usage of this information in NG-RAN node.

During the last meeting, there was a discussion on whether the time interval should be included in the Inventory Request message, and what advantages it might offer. The following is the related FFS from RAN3#128 chair note.

The concept of an optional time interval for aggregation is already defined in 3GPP TS 23.369. We also propose for including the time interval in the Inventory Request message. This enhancement would help reduce signalling frequency between the gNB and the AIOTF. Moreover, since a single AIOTF may serve multiple NG-RANs, this Information Element (IE) would facilitate synchronization across the system, enabling the AIOTF to send a consolidated response.

Proposal 1: Introduce an optional Time Interval parameter in the Inventory Request, as recommended in 3GPP TS 23.369. This enhancement will help reduce signalling overhead and enable better synchronization across the service.

2.2 Requirement of Inventory complete Indication for command after inventory procedure

In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message.

In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

Introduce a class2 procedure to allow NG-RAN node to trigger Session Release procedure towards AIoT CN.

In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message.

In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

Introduce a class2 procedure to allow NG-RAN node to trigger Session Release procedure towards AIoT CN.

RAN3#128 Chair note:

Inventory Complete Indication in Inventory-Only Procedure

In the last meeting, it was agreed to include an Inventory Complete Indication in the Inventory-Only procedure. However, there was no consensus on how this indication should be conveyed from the gNB to the AIoT CN, nor on whether it is necessary in the Command-After-Inventory scenario.

The optional inclusion of the Inventory Complete Indication in the Command-After-Inventory scenario has already been agreed upon in 3GPP TS 23.369 (SA2). This indication is valuable for enabling nodes to understand when the service has been fully completed, especially in scenarios involving partial inventory results.

For example, consider a case where the AIOTF intends to command 100 devices and includes an approximate number of devices as 100 in the Inventory Request. If the gNB returns an Inventory Response for only 80 devices—having failed to access the remaining 20—the AIOTF will proceed to command the 80 devices. However, without an explicit indication that the inventory process is complete, the AIOTF cannot be certain whether the service request has been fully addressed. This uncertainty can lead to delays in session release and unnecessary resource consumption.

Proposal 2: Propose to introduce inventory complete indication in the command after inventory scenario also. This will ensure clarity on service completion, reduce resource wastage, and improve overall system efficiency.

To introduce the Inventory Complete Indication in the Inventory-Only scenario, several proposals were discussed during the last meeting. These included:

- Adding an Inventory Complete Indication IE in the final Inventory Report.

- Defining a new procedure specifically to signal inventory completion.

- Reusing the existing Session Release Request procedure from the gNB, with an added IE to indicate inventory completion.

Indicating inventory complete procedure to the AIOTF via session release request procedure is not an ideal way. Because when we consider indicating inventory completion after inventory in the inventory after command scenario this will trigger the release of session in between the ongoing command procedure.

Observation 1: Using the Session Release Request procedure which was agreed upon in the last meeting for indicating Inventory Completion may lead to premature session termination in Command-After-Inventory scenarios. This approach risks releasing the session before the service is fully completed, potentially interrupting the ongoing command procedure.

Another suggestion discussed was to introduce an Inventory Report Complete Indication IE in the final inventory report. While this approach is straightforward, but it creates a potential challenge.

There may be scenarios where the gNB needs to send an inventory report without any device information, solely to indicate the completion of the inventory process. For example, if the gNB is configured to report periodically to the AIOTF, it may only determine after sending the last inventory report that no additional devices are reachable. This could be due to the expiry of paging messages or internal mechanisms that detect unreachable devices.

In such cases, the gNB would need to send an inventory report containing only the Inventory Complete Indication IE, without any device data.

Observation 2: There is a possibility that the inventory report may contain only the Inventory Complete Indication IE, which is not ideal.

The most suitable approach is to introduce a separate Class 2 procedure for indicating Inventory Completion.

This method is applicable to both Inventory-Only and Command-After-Inventory scenarios. It allows the gNB to notify the AIoT CN of inventory completion independently, without relying on or interfering with other procedures, ensuring flexibility and procedural clarity.

Inventory Complete Indication

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Complete IndicationTransfer | M |  | OCTET STRING | Containing the Inventory Complete IndicationTransfer IE | YES | reject |

Inventory Complete Indication Transfer IE

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Inventory Completion Indicator | M |  | Enumerated(true, ) | Indicate the completion of inventory |

Proposal 3: Support the introduction of the Inventory complete indication as a separate Class 2 procedure.

2.3 Command Type in Command request

The main interest of introducing Command Type in Command request is to implicitly inform the gNB the expected size of D2R message and allocate the necessary resources for it. But Expected D2R size is already agreed and included in the command request. So, we propose to not include Command Type in the command request.

Proposal 4: Propose to not include the Command Type in the Command request message.

3.Conclusions

Proposal 1: Introduce an optional Time Interval parameter in the Inventory Request, as recommended in 3GPP TS 23.369. This enhancement will help reduce signalling overhead and enable better synchronization across the service.

Proposal 2: Propose to introduce inventory complete indication in the command after inventory scenario also. This will ensure clarity on service completion, reduce resource wastage, and improve overall system efficiency.

Observation 1: Using the Session Release Request procedure which was agreed upon in the last meeting for indicating Inventory Completion may lead to premature session termination in Command-After-Inventory scenarios. This approach risks releasing the session before the service is fully completed, potentially interrupting the ongoing command procedure.

Observation 2: There is a possibility that the inventory report may contain only the Inventory Complete Indication IE, which is not ideal.

Proposal 3: Support the introduction of the Inventory complete indication as a separate Class 2 procedure.

Proposal 4: Propose to not include the Command Type in the Command request message.