3GPP TSG-RAN WG3 #129	R3-255215

Bangalore, India, 25th – 29th of August 2025

Agenda Item:	21.3

Source:	NEC

Title: 	Discussion on RLC enhancement

Document for:	Discussion

1	Introduction

In RAN3#128 meeting, following agreements and the remaining discussion points have been made.

CU sends the uplink rate control indication per QoS flow over F1 to DU. Other additional assistance information from CU to DU? - Recommended UL bit rate infor per QoS flow - Measured bit rate per QoS flow - No additional information No NGAP signaling enhancement is needed for the support of non-homogeneous deployment? Introduce a F1AP IE, similar to NGAP Indication of Bitrate Adaptation IE. Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement. For uplink, FFS on whether/how gNB-DU can configure the autonomous retransmission and/or enhanced polling thresholds for the DRB. For downlink, FFS on whether/how enhance Signalling between CU and DU to support timely retransmission.

In this contribution, we describe our view for remaining points.

2	Discussion

2.1 UL rate control

In the last RAN3#128 meeting, following discussion has been held and there is no consensus for this topic. [1]

Further discussion on whether Other additional assistance information from CU to DU. Following options were proposed: - Option 1: Recommended UL bit rate info per QoS flow - Option 2: Measured bit rate per QoS flow - Option 3: No additional information

In the context of UL traffic, it is assumed that information regarding the uplink data rate is maintained only at the UE, and such information is not available from OTT servers or upper layers. Therefore, gNB is required to perform UL rate control based on the wireless environment. As a result, no additional assistance information from the CU is necessary, and the process can be completed within the DU. Thus, further assistance information is not needed.

Proposal 1:	No need to introduce additional assistance information from CU to DU for UL rate control.

2.2 Unnecessary RLC retransmission avoidance

In the last RAN3#128 meeting, following discussion has been held and there is no consensus for this topic.

Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement.

In the DU, the activation of the RLC retransmission avoidance feature depends on whether this functionality is enabled for the corresponding UE. According to the RAN2 RRC running CR[2], the uplink configuration is indicated via stopReTxObsoleteSDU, while the downlink configuration is indicated via t-RxDiscard. Therefore, the DU should determine whether to apply the RLC retransmission avoidance feature based on the presence of these IE settings for the UE. Consequently, it is necessary for the CU to notify the DU of these two IEs. We propose to add these IEs to the F1-AP as per the corresponding TP.

On the other hand, regarding the NR-U plane protocol, there is currently no explicit specification identifying which RLC SDUs are subject to discard. Although there is no defined behavior for which RLC SDUs should not be discarded, we understand that this aspect has already been addressed through implementation-specific mechanisms. Therefore, we conclude that the RLC retransmission avoidance feature does not impact the NR-U plane protocol.

Proposal 2: Introduce new F1-C IE to know whether RLC retransmission can be terminated, and agree TP for F1-AP.

2.3 Timely RLC retransmission

In the last RAN3#128 meeting, following discussion has been held and there is no consensus for this topic.

For uplink, FFS on whether/how gNB-DU can configure the autonomous retransmission and/or enhanced polling thresholds for the DRB. For downlink, FFS on whether/how enhance Signalling between CU and DU to support timely retransmission.

In the last RAN2#130 meeting, following agreements has been made.[3]

Agreements on RLC timely retransmissions - (RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this - (RLC-9) No additional conditions are needed for the polling enhancement. - (RLC-7) RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT. - (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed. - (RRC-3, RRC-4) Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance - (RRC-8) The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

For uplink, regarding the autonomous retransmission and enhanced polling mechanisms, these are UE behaviours that can be controlled independently within the UE. As such, no specific signalling from the CU to the DU is required for these functionalities.

For downlink, only PDCP layer knows remaining time for each PDCP SDU.

There are two options to achieve this function.

option1 : CU sends Conditions (e.g., PDCP discard timer values, and autonomous retransmission/ enhanced polling threshold) and remaining timer value for each PDCP SDU to DU.

option2 : CU sends trigger for autonomous retransmission/ enhanced polling when condition is met to DU.

when option1 is introduced, Discard timers are runnning on both CU and DU. we think it is not efficient design.So we prefer to go with option2. Then we propose TP for NR-u protocol.

Proposal 3: Introduce new F1-U IE to trigger autonomous retransmission/ enhanced polling when condition is met, and agree TP for NR-u protocol.

3	Summary

In this paper, XR enhancements were discussed and the following proposals were made:

Proposal 1: No need to introduce additional assistance information from CU to DU for UL rate control.

Proposal 2: Introduce new F1-C IE to know whether RLC retransmission can be terminated, and agree TP for F1-AP.

Proposal 3: Introduce new F1-U IE to trigger autonomous retransmission/ enhanced polling when condition is met, and agree TP for NR-u protocol.

4	References

- R3-253773, Summary of offline discussion on CB: # R19XR , Nokia

- R2-2503787, Introduction of R19 XR enhancements for RRC spec, Huawei

- R2-2504675, Report from session on R19 XR and LTE Broadcast, Session chair (Huawei)

5	TP for TS38.473

<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>

8.3.1	UE Context Setup

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

<<<<<<<<<unaffected text is skipped>>>>>>>>>

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the Stop ReTx Obsolete SDU UL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to set the parameters for uplink timely retransmission configuration as specified in TS 38.331 [8].

If the t-RxDiscard DL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to set the parameters for downlink timely retransmission configuration as specified in TS 38.331 [8]

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

<<<<<<<<<unaffected text is skipped>>>>>>>>>

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| <<<<<<<<<unaffected text is skipped>>>>>>>>> |  |  |  |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>Stop ReTx Obsolete SDU UL | O |  | ENUMERATED {enabled, disabled} | Corresponds to information provided in the stopReTxObsoleteSDU in the RLC-Config IE as defined in TS 38.331 [8]. | YES | ignore |
| >>>>t-RxDiscard DL | O |  | ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, ms3000} | Corresponds to information provided in the t-RxDiscard in the RLC-Config IE as defined in TS 38.331 [8]. | YES | ignore |
| <<<<<<<<<unaffected text is skipped>>>>>>>>> |  |  |  |  |  |  |

5	TP for TS38.425

<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>

5.2	NR user plane protocol layer services

NOTE 1:	In this section, NR user plane protocol layer services are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

The following functions are provided by the NR user plane protocol:

-	Provision of NR user plane specific sequence number information for user data transferred from the node hosting NR PDCP to the corresponding node for a specific data radio bearer.

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for user data associated with a specific data radio bearer.

-	Information of NR PDCP PDUs that were not delivered to the UE or not transmitted to the lower layers.

-	Information of NR PDCP PDUs transmitted to the lower layers for user data associated with a specific data radio bearer.

-	Information of downlink NR PDCP PDUs to be discarded for user data associated with a specific data radio bearer;

-	Information of the currently desired buffer size at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer.

-	Information of the currently desired data rate in bytes at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer;

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for retransmission user data associate with a specific data radio bearer;

-	Information of NR PDCP PDUs transmitted to the lower layers for retransmission user data associated with a specific data radio bearer.

-	Information of the specific events at the corresponding node.

-	Information on Radio Link Quality from the corresponding node for user data associated with a specific data radio bearer.

-	Information for QoS monitoring from the corresponding node for user data associated with a specific data radio bearer.

-	Information of NR PDCP PDUs that were needed to trigger RLC Polling for timely RLC re-transmission.

-	Information of NR PDCP PDUs that were needed to trigger RLC re-transmission for timely RLC re-transmission.

<<<<<<<<<unaffected text is skipped>>>>>>>>>

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>

#### 5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU.

The following shows the respective DL USER DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 |  | 5 |  |  | 4 |  | 3 |  | 2 |  | 1 |  |  | 0 |  |
| PDU Type (=0) |  |  |  |  |  |  |  | Spare |  | DL Discard Blocks |  | DL Flush |  |  | Report polling | 1 |
| New IE Flags | BSSI |  |  | TTNBI |  | Request OutofSeq Report |  | Report Delivered |  | User data existence flag |  | Assistance Info. Report Polling Flag |  |  | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| BSSize |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| TTNB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 2 |
| New IE flag 7(E) |  | New IE flag 6 |  |  | New IE flag 5 |  | New IE flag 4 |  | New IE flag 3 |  | New IE flag 2 |  | New IE flag 1 | New IE flag 0 |  | 1 Octet New IE Flags |
| RLC Polling triggered NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| RLC ReTx triggered NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

The New IE Flag in bit 7 of 1st octet in DL USER DATA (PDU Type 0) Format indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in DL USER DATA (PDU Type 0) Format indicates if the RLC Polling is present (1) or not (0).

Bit 1 of New IE Flags Octet in DL USER DATA (PDU Type 0) Format indicates if the RLC ReTx is present (1) or not (0).

<<<<<<<<<unaffected text is skipped>>>>>>>>>

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>

### 5.5.3	Coding of information elements in frames

NOTE 1:	In this section, information elements are also applicable to E-UTRA PDCP unless specified otherwise. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP unless specified otherwise.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

5.5.3.58	Feedback Delay Result

Description: This parameter indicates the feedback delay time measured at the corresponding node in milliseconds from the time of reception of the Polling Frame that triggered transmission of the DL DATA DELIVERY STATUS to the time of transmission of the DL DATA DELIVERY STATUS. It is encoded as an Unsigned32 binary integer value.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.x	DL PSI Based Discard Indication

Description: This field indicates the presence of the DL PSI Based Discard Activation Suggestion. This information element is not applicable to E-UTRA PDCP.

Value range: {0= DL PSI Based Discard Activation Suggestion not present, 1= DL PSI Based Discard Activation Suggestion present}.

Field length: 1 bit.

#### 5.5.3.y	DL PSI Based Discard Activation Suggestion

Description: This parameter indicates the suggestion given by the corresponding node on whether DL PSI based PDCP discard should be activated or not. This information element is not applicable to E-UTRA PDCP.

Value range: {0= Not activate, 1= Activate}.

Field length: 1 bit.

#### 5.5.3.x1	UL Available Data Rate Ind

Description: This parameter indicates the presence of UL Available Data Rate. This information element is not applicable to E-UTRA PDCP.

Value range: {0= UL Available Data Rate not present, 1= UL Available Data Rate present}.

Field length: 1 bit.

#### 5.5.3.y1	DL Available Data Rate Ind

Description: This parameter indicates the presence of DL Available Data Rate. This information element is not applicable to E-UTRA PDCP.

Value range: {0= DL Available Data Rate not present, 1= DL Available Data Rate present}.

Field length: 1 bit.

#### 5.5.3.x2	UL Available Data Rate

Description: This field indicates the value of uplink available data rate for the QoS flow. This information element is not applicable to E-UTRA PDCP. The unit is Mbps.

Value range: {0..FFS}.

Field length: FFS octet.

#### 5.5.3.y2	DL Available Data Rate

Description: This field indicates the value of downlink available data rate for the QoS flow. This information element is not applicable to E-UTRA PDCP. The unit is Mbps.

Value range: {0..FFS}.

Field length: FFS octet.

#### 5.5.3.c	BSSI (Burst Size Indicator)

Description: This parameter indicates the presence of Burst Size (BSSize).

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

#### 5.5.3.d	Burst Size (BSSize)

Description: This parameter indicates the total size of the burst.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.e	TTNBI (TTNB Indicator)

Description: This parameter indicates the presence of Time To Next Burst (TTNB).

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

#### 5.5.3.f	Time To Next Burst (TTNB)

Description: This parameter indicates the the approximate time in tenth of milliseconds to the next burst.

Value range: {0..216-1}.

Field length: 2 octets.

Editor Note: FFS whether to capture the Burst Size and Time To Next Burst related information in the DL USER DATA frame.

#### 5.5.3.g	RLC Polling triggered NR PDCP PDU SN

Description: This parameter indicates the NR PDCP PDUs that were needed to trigger RLC Polling for timely RLC re-transmission.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.h	RLC ReTx triggered NR PDCP PDU SN

Description: This parameter indicates the NR PDCP PDUs that were needed to trigger RLC retransmission for timely RLC re-transmission.

Value range: {0..224-1}.

Field length: 3 octets.