3GPP TSG-RAN WG3 Meeting #129	R3-255239

Bengaluru, India, 25th - 29th Aug, 2025

Title: 	(TP to BLCR for TS 38.473 and TS 38.470) Discussion on on-demand SSB SCell operation

Source: 	Huawei

Agenda item:	17.2

Document Type:	 Other

# 1. Introduction

The previous RAN3#126 meeting discussed the on-demand SSB SCell operation, with the following agreement.

- For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

In this contribution, we intend to finalize the discussion on on-demand SSB SCell operation, based on RAN1 and RAN2 progress.

# 2. Discussion

## 2.1 The allowed Cell list

The RAN3-127bis meeting discussed whether and how to introduce the allowed cell list to allow on-demand SSB SCell with the following contents in [1].

It is proposed to take the following as baseline and to be discussed at the next meeting (no need to capture into the Chairman notes). Take the following texts as baseline: If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may take it into account to enable the on-demand SSB SCell operation for the indicated cells when they are operating as SCells.

There are two main issues discussed online as follows.

- Issue 1: Whether there is any NBC issue to reuse the legacy IE for both the SSB beam deactivation, and the OD-SSB SCell?

- Issue 2: Whether the DU should strictly follow the allowed Cell list, or make decisions for those cells not included in the Cell list?

For the first issue, we consider there is no NBC issue. The Cells Allowed to be Deactivated List IE was introduced in R18 WI. Based on the information, the DU can decide to deactivate some beams or even the whole cell itself and then to notify CU the deactivated SSB beams. Since the intension is to deactivate some SSB beams, it is also possible that all SSB beams are allowed to be deactivated. That means, the existing IE already covers the scenario when the whole Cell could be switched off.

- Observation 1: The existing IE (Cells Allowed to be Deactivated List IE) already covers the scenario when the whole Cell could be switched off.

Then for the case 1 no always on SSB case, this legacy Cells allowed list IE can be reused to indicate these Cells which are allowed to be totally switch-off when the OD-SSB is not broadcast. For case 2 always on SSB case, the legacy IE can also be reused to indicate these cells where the on-demand SSB could be activated or deactivated dynamically. It is up to DU to decide which mode to go, i.e., case 1 or case 2.

There is no NBC issue for the CU to reuse the existing IE (Cells Allowed to be Deactivated List IE) to also indicate the allowed cells for on-demand SSB operation to the DU.

For the second issue, we consider that the logic could be the same as the Rel-18, where the DU behaviour is specified as follows. The DU would just consider those indicated Cells to deactivate the SSB beams. Then for the OD-SSB SCell, the same principles apply as well. Hence the corresponding procedure texts are provided as follows.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose. The gNB-DU shall, if supported also consider that it is allowed to enable the on-demand SSB SCell operation within the indicated Cells for network energy saving purpose as specified in TS 38.300 [6].

Further, the CU can infer the cell switch off due to the all beams deactivation and the cell switch off due to the  OD-SSB no broadcasting.

- For the former, the DU will send the Coverage Modification Notification IE containing all the deactivated SSB beams.

- For the latter, the DU will send the Cells Status Item IE to indicate the out-of-service state for case 1.

Based on the above analysis, the existing IE could be reused to indicate the on-demand SSB SCell operation. With the legacy cells allowed list, it is up to the DU to decide to only deactivate some SSB beams, or all SSB beams, or enable on-demand SSB SCell operation. If the above not agreed, a new allowed cell list can be introduced

The same R18 logic to use the legacy Cells Allowed to be Deactivated List IE applies for R19, and RAN3 to agree the above stage3 procedure texts.

If the above not agreed, to introduce a new On-demand Cells allowed list IE.

## 2.2 Other issues for Case 1 – No Always-on SSB Transmission

First, for Case #1, the SSB is not continuously transmitted then this Cell can be regarded as deactivated cell or switched off due to the energy saving by the neighbouring nodes. According to the current XnAP specification, the procedure for such cell activation/deactivation via backhaul is already supported, allowing an NG-RAN node to request the activation of a cell by a neighbouring NG-RAN node. Hence, the existing cell activation message can be re-used without changes to the specification.

For L3 measurement, RAN2 agreed to use the OD-SSB specific SMTC to measure when the OD-SSB is activated and SCell is activated. According to the running RAN2 RRC CR [3], the OD-SSB specific SMTC is included in the CellGroupConfig IE, which would be determined by the DU itself, without any impact on RAN3. About the neighbour cell measurements, though there is no firm RAN2 agreements, it seems no additional specification impact on RAN3.

For Case #1, with no always-on SSB transmission, RAN3 to agree that:

- no spec impact on the Xn specification for cell (de)activation.

- the OD-SSB specific SMTC could be decided by the DU itself.

For the RRC and MAC CE based signaling to indicate on-demand SSB transmission, below provides the latest progress in RAN1 and RAN2. Also, previous RAN3 meeting agreed that it is up to the DU to decide the activation/deactivation state of OD-SSB configuration for RRC based signalling.

Agreement@RAN1#119 For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 FFS: Whether the value of N can be implicitly determined using a timer Agreement at RAN1#120bis For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions Agreement at RAN2#127bis meeting 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. 2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. Agreement at RAN2#129bis meeting 1. For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). 2. A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case. 3. For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. 4. OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB. 5. RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS. 6. Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

It can be observed that multiple OD-SSB parameters (e.g., OD-SSB periodicity, od-ssb-PositionsInBurst, od-ssb-nrofBurst) can be configured in the RRC. The MAC CE can be used to indicate one configuration index for each SCell activating OD-SSB. The deactivation can be realized by explicit and implicit indication.

Due to the fact that the OD-SSB transmission could be used for synchronization, cell measurement, SCell activation etc in different scenarios. The notification to CU should be discussed by considering different scenarios’ requirement. For example, if the OD-SSB transmission is used for L3 measurement, there is a need to notify the CU so that the CU may be anticipated to receive L3 measurement results from the UE. Furthermore, the activation status also can be changed by RRC signalling. CU can also deactivate the OD-SSB transmission activated by DU with the notification of OD-SSB transmission activation indication from DU. RAN3 can discuss whether to reuse the existing Service Status IE including in-service, out-of service, or introduce a new IE.

When the DU sends on-demand SSB MAC CE to the UE and begins the SSB broadcast, the CU should be notified by the DU (e.g., the activation status also can be changed by RRC signalling or CU may be anticipated to receive L3 measurement results from the UE).

- In addition, when the DU sends the on-demand SSB activation MAC CE to the UE, RAN3 can discuss how the DU decides and sends the SCell activation MAC CE to the UE. One possible way is to rely on the L1 measurement based on on-demand SSB, as agreed in RAN1 to use periodic, semi-persistent, and aperiodic L1 measurement reports by using existing CSI framework. Another way is to rely on the L3 measurement result. In this case, the DU may need the assistance from the CU to decide whether to activate the SCell or not.

RAN3 to discuss how the DU decides the SCell activation based on the L1 measurement and/or the L3 measurement result after sending the on-demand SSB activation MAC CE.

## 2.2 Other issues for Case 2 - Always-on SSB is periodically transmitted on the cell

For Case #2, the always-on SSB is periodically transmitted on the SCell. From RAN3 perspective, this cell can be considered as a normal working cell. For example, this SCell can be already activated for one UE while being deactivated for another. Also, RAN1 agreed that the always-on SSB could be multiplexed with on-demand SSB as follows.

Agreement at RAN1-118bis meeting For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain Agreement at RAN1#119 Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress. Agreement at RAN1#119 Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Agreement at RAN2#130 5. In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation: • The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation. • The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly. Spec impact can be further discussed in running CR preparation. 6. When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO. 7. The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured. 8. RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement. 9. RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported. 10. When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

Over Xn interface, neighbouring NG-RAN nodes can treat this cell as a normal served cell since there is always on SSB transmission, i.e., without deactivation indication. As a result, there is no need to activate such cell.

Similarly, over F1 interface, the CU can treat this cell as an activated and “in service” cell. For L3 measurement, the OD-SSB specific SMTC to measure OD-SSB is determined by the DU. Following the analysis in case 1, we assume no additional specification impact on RAN3 on L3 measurement.

For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to agree that no Xn specification impact for cell (de)activation.  The rest can follow the conclusion for case#1.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

The allowed Cell list

- Observation 1: The existing IE (Cells Allowed to be Deactivated List IE) already covers the scenario when the whole Cell could be switched off.

- There is no NBC issue for the CU to reuse the existing IE (Cells Allowed to be Deactivated List IE) to also indicate the allowed cells for on-demand SSB operation to the DU.

The same R18 logic to use the legacy Cells Allowed to be Deactivated List IE applies for R19, and RAN3 to agree the above stage3 procedure texts.

If the above not agreed, to introduce a new On-demand Cells allowed list IE.

Case#1: No always-on SSB transmission

For Case #1, with no always-on SSB transmission, RAN3 to agree that:

- no spec impact on the Xn specification for cell (de)activation.

- the OD-SSB specific SMTC could be decided by the DU itself.

When the DU sends on-demand SSB MAC CE to the UE and begins the SSB broadcast, the CU should be notified by the DU (e.g., the activation status also can be changed by RRC signalling or CU may be anticipated to receive L3 measurement results from the UE).

RAN3 to discuss how the DU decides the SCell activation based on the L1 measurement and/or the L3 measurement result after sending the on-demand SSB activation MAC CE.

- Case#2: Always-on SSB is periodically transmitted on the cell

For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to agree that no Xn specification impact for cell (de)activation.  The rest can follow the conclusion for case#1.

- The TPs for TS 38.473 and TS 38.470 to implement the Proposal 1 and 2 are provided in Annex.

# 3. Reference

- R3-252357 Summary of Offline Discussion on on-demand SSB SCell operation, Huawei (Moderator)

- Chair notes RAN1#117 eom4.

- R2-250xxx Running RRC CR NES post130_v1.docx

# 5. TP to BLCR for TS 38.473 against R3-255081

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose. The gNB-DU shall, if supported also consider that it is allowed to enable the on-demand SSB SCell operation within the indicated Cells for network energy saving purpose as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 6. TP for TS 38.470

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

## 5.2	F1-C functions

### 5.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and the gNB-CU.

The F1 setup function allows to exchange application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface, and exchange the intended TDD DL-UL configuration originating from the gNB-DU or destined to the gNB-DU. The F1 setup is initiated by the gNB-DU. The F1 setup function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, and may activate or deactivate cells. The gNB-CU Configuration Update function may activate SSB beams of cells. The gNB-CU Configuration Update function may indicate the cells where the gNB-DU is allowed to deactivate the SSB beams or to enable the on-demand SSB operation. With the gNB-CU Configuration Update function, energy saving with cell activation and cell deactivation can be supported as defined in TS 38.300 [8]. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. The gNB-DU Configuration Update function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>