3GPP TSG-RAN WG3 Meeting #129	R3-255287

Bengaluru, India, 25th - 29th August 2025

Agenda Item:	14.3

Source:    TCL.

Title:         Remaining Issues on Support of NTN Regenerative Architecture

Document for:	Discussion and Decision

# Introduction

This report discusses remaining issues on support of NTN regenerative architecture, mainly NG interface interruption due to hard feeder link switch over.

# Discussion

During RAN3-127bis and RAN3-128, the issues of NG management in regenerative NTN architectures where gNBs are deployed on satellites were discussed and the following were agreed:

RAN3-127bis [1]:

Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any).

RAN3-128 [2]:

WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure.

Check the use cases including Hard Feederlink Switch, other use cases to be further justified.

Unlike terrestrial networks, NG interfaces in NTN face frequent disruptions due to satellite mobility and feeder link switches (FLSO). These interruptions impact critical NG procedures (e.g., initial UE context setup, paging, DL data/signaling), affecting UE RRC states and service continuity as detailed in [3]. To mitigate such interruption due to hard feeder link switch over while preserving AMF stability:

- NG disruptions must be handled without triggering AMF reselection.

- The 5GC requires mechanisms to suspend/resume NG connections during link recovery).

Based on t above we propose:

Proposal 1: Introduce NG Connection Control IE to enable 5GC-controlled suspension/resumption of DL transmissions during NG interruptions, including link status and recoverability parameters.

Proposal 2: Contain UE NG procedures (e.g., initial UE message/context setup) within NG signaling to maintain AMF consistency during feeder link switches

# Conclusion

This contribution has discussed NG interface management during feeder link switch and their impacts on UE NG related procedures and proposed:

Proposal 1: Introduce NG Connection Control IE to enable 5GC-controlled suspension/resumption of DL transmissions during NG interruptions, including link status and recoverability parameters.

Proposal 2: Contain UE NG procedures (e.g., initial UE message/context setup) within NG signaling to maintain AMF consistency during feeder link switches

# References

- R3-25xxxx,” RAN3 #127-bis Meeting Report” ETSI MCC,  07-11 April 2025

- R3-25xxxx,” RAN3 #128 Meeting Report”, ETSI MCC, 19 - 23 May 2025, Malta, MT.

- R3-253609, “Further Discussion on Support of NTN Regenerative Architecture”， TCL， 19 - 23 May 2025, Malta, MT.