3GPP TSG-RAN WG3 Meeting #129	R3-255301

Bengaluru, India, Aug 25th - Aug 29th, 2025

Agenda item:	13.2

Source:	Qualcomm Incorporated

Title:	Signalling enhancements for Inter-CU LTM handover

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #128 [1]), RAN3 made significant progress towards completion of the specification work for Inter-CU LTM. However, many open issues remain, which need to be addressed during this meeting. The list of agreements from the last RAN3 meeting are as below.

RAN3 #128 agreements:

Inter-CU LTM:

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

Add description in Stage 2 TS 38.401 for describing that CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE message.

For Inter-CU LTM, LTM CONFIGURATION UPDATE procedure is per node level basis with a list of cells, and security key is per cell.

Remove Note in TS 38.300 BL CR “Editor’s Note: step 6 and 7 are optional.”

Inter-CU LTM in DC:

Update the text description of CELL SWITCH NOTIFICATION message to capture the RAN3#127bis agreement: Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message, and reusing the existing IEs.

Update the TA INFORMATION TRANSFER message to provide a list of TA values of multiple candidate cells.

The SCG reference configuration is provided by an implicit way in the CG-Config RRC container in the SN Change Required message. FFS on the SN Addition Request Acknowledge message from the source SN/candidate SN to the MN.

The SCG reference configuration is provided by an implicit way in the CG-ConfigInfo RRC container in the SN Addition Request message from the MN to the candidate SN.

A per cell indicator indicating complete candidate configuration is included in the SN Addition Request Acknowledge message.

The SN Change Required message and the SN Modification Request message from MN to source SN should design a mechanism to support multiple candidate SNs, i.e., SN can include multiple candidate SNs information in a single SN Change Required message, and MN includes multiple candidate SNs information in a single SN Modification Request message.

The SN Addition Request message should design a mechanism to support single candidate SN, i.e., MN sends SN addition request towards only one candidate SN.

Update the online agreement to: CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message.

Reuse the CSI-RS coordination procedure over F1AP and XnAP for source gNB/gNB-DU to activate or deactivate the SP CSI-RS resource for CSI acquisition in candidate cell.

In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message.

Include the Rel-19 set IDs of source cell and each candidate cell(s) in UE Context Modification Request message. Introduce Rel-19 set IDs into LTM Security Information IE.

FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2:

Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure.

FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and UE Context Modification Request message.

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

The suggested PSCell list should be explicitly included in the SN Change Required message.

The maximum number of PSCells that can be prepared by each candidate SN is included in the SN Change Required message.

The LTM Configuration ID Mapping List IE may be included in the SN Change Required message and the SN Addition Request message.

The LTM Security Configuration List IE including a list of pair of {security key, sk-counter} is included in the SN Addition Request message and the SN Modification Request message to support subsequent inter-CU SCG LTM.

The SN Security Key IE included in the SN Addition Request message should be ignored if the procedure is triggered for the LTM.

WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM.

If the source SN has the SCG reference configuration, the source SN will provide the SCG reference configuration in the SN Change Required message and thus the MN will not request source SN to provide the SCG reference configuration.

Early data forwarding is supported in inter-CU LTM in DC.

Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation.

Enhance XN-U ADDRESS INDICATION message and define IE to cover Inter-CU MCG LTM case. FFS on Inter-SN LTM case.

MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

The source SN generates the common LTM CSI resource configuration for inter-CU SCG LTM and then provides the common LTM CSI resource configuration to the MN via the SN Modification Request ACK message.

For the SSB based L1 measurement, the common CSI resource configuration may be included in the SN Change Required message and the SN Addition Request message.

The CSI Resource Configuration IE is the common CSI resource configuration, which refers to the ltm-CSI-ResourceConfig-r18 IE in the RRC spec.

The L1 Configuration IE is the L1 RS configuration per candidate PSCell, which refers to the ltm-SSB-Config-r18 IE in the RRC spec for the SSB based L1 measurement, or the ltm-NZP-CSI-RS-ResourceConfigToAddModList-r19 IE associated with the ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 IE in the RRC spec for the CSI-RS based L1 measurement.

For the avoidance of simultaneous configurations of inter-CU MCG LTM and inter-CU SCG LTM, no RAN3 impact is foreseen.

FFS on whether LTM modification/cancel related procedures are needed.

The SP CSI-RS activation/(de)activation for the inter-CU LTM in DC is deprioritized in rel-19.

In this contribution, we continue the discussion on signalling enhancements for inter-CU LTM handover, focusing on the following procedures and scenarios:

- Transmission of CSI-RS-based beam measurements over F1-AP for L3 measurements-based LTM,

- Configuration of semi-persistent (SP) CSI-RS resources for LTM,

- Support of CSI-RS resources for LTM CSI acquisition for candidate cells,

- Configuration of periodic and SP CSI-IM resources for LTM,

- UE association information signalling upon LTM cell switch,

- Remaining issues in Early TA RACH configuration and TA Information Transfer procedures,

- LTM reference configuration,

- Remaining aspects of the Cell Switch Notification procedure.

# 2 Transmission of CSI-RS-based beam measurements over F1-AP for L3 measurements-based LTM

In Rel-18 LTM, for the L3 measurements-based LTM solution, only SSB-based beam measurement results were agreed to be included in the F1-AP CU-DU Mobility Initiation Request message transmitted by the CU to the DU. However, the CU also has CSI-RS-based beam measurement results that it can receive from RRC Measurement Reports from the UE. We therefore propose that CSI-RS-based beam measurement results should also be sent from the CU to the DU in the F1-AP CU-DU Mobility Initiation Request message.

Proposal 1. In Rel-19 LTM, for L3 measurements-based LTM solution, the serving gNB-CU shall send CSI-RS-based beam measurements to the serving gNB-DU in the F1-AP CU-DU Mobility Initiation Request message, in addition to the SSB-based beam measurements and cell level measurements.

# 3 Configuration of semi-persistent (SP) CSI-RS resources for LTM

In this section, we discuss an issue that we found with the current Rel-19 signalling procedure for SP CSI-RS activation/deactivation and propose a solution to the issue.

In legacy NR, for a serving cell, an SP CSI-RS resource has no pre-defined TCI state ID, and it is dynamically assigned when SP CSI-RS is activated using the SP CSI-RS/CSI-IM activation/deactivation MAC CE.

Observation 1. In legacy NR, for a serving cell, an SP CSI-RS resource has no pre-defined TCI state ID, and it is dynamically assigned when SP CSI-RS is activated. The TCI state ID of the activated SP CSI-RS resource is indicated in the SP CSI-RS/CSI-IM activation/deactivation MAC CE.

In Rel-19 LTM, RAN3 defined a signalling procedure for activation/deactivation of SP CSI-RS resources of an LTM candidate cell.

Observation 2a. In Rel-19 LTM, RAN3/RAN2 has defined the following signalling procedure for activation/deactivation of SP CSI-RS resources of an LTM candidate cell:

- The source DU indicates in signalling messages via F1 and Xn to a candidate DU activation/deactivation of SP CSI-RS resources of a candidate cell.

- The candidate DU confirms the activation/deactivation in its response message to the source DU.

- The source DU then transmits the activation/deactivation indication to the UE in the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE.

The issue that we found is captured in the following observation.

Observation 2b. In the response message to the source DU, the candidate DU does not indicate the TCI state IDs for the activated SP CSI-RS resources, so that the source DU is unable to include these TCI state IDs along with the activated SP CSI-RS resources in the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE (defined for LTM) to the UE.

A simple solution to the above issue that we have identified is to configure the TCI states for SP CSI-RS resources of LTM candidate cells during the LTM preparation procedure. The observation below describes the advantages of this approach.

Observation 3. If TCI states are configured for SP CSI-RS resources of an LTM candidate cell during the preparation procedure for LTM configuration, then there are the following advantages.

- Signalling of TCI states from candidate DU to the source DU during activation/deactivation of SP CSI-RS resources of candidate cells, can be avoided.

- TCI state ID information does not need to be included in the activation/deactivation signalling using MAC CE from the source DU to the UE.

The proposal below describes the procedure to configure the TCI states for SP CSI-RS resources of LTM candidate cells during the LTM preparation procedure.

Proposal 2. During LTM preparation, upon receiving a request from the source gNB-CU (via a candidate gNB-CU) for CSI-RS resource configuration, a candidate gNB-DU provides the configuration of TCI state IDs for SP CSI-RS resources configured for a candidate cell in its response message (UE Context Setup Resp or UE Context Modification Resp).

- The configuration of TCI state IDs for SP CSI-RS resources is forwarded to the source gNB-CU by the candidate gNB-CU in the Handover Req Ack message.

- The source CU includes the provided configuration in the LTM configuration signalled to the UE.

Further details regarding the specific IEs in the F1 messages that can include the TCI state configuration for configured SP CSI-RS resources of candidate cells is described in the observation below. We note that the solution only has RRC specification impact and there is no RAN3 signalling impact, since the TCI state configuration information can be provided using the RAN2 RRC IE.

Observation 4. The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell is included in the CSI-RS Resource Configuration To AddModList IE in the UE Context Setup Resp and UE Context Modification Resp messages, and this IE contains the ltm-CSI-ResourceConfigToAddModList, as defined in the running RRC CR for Rel-19 LTM.

As the running RRC CR for Rel-19 LTM indicates (see below), the ltm-NZP-CSI-RS-ResourceSet refers to the RRC IE NZP-CSI-RS-Resource, and we propose an enhancement to this basic RRC IE in Observation 5 below to include the configuration of TCI state IDs for SP CSI-RS resources of an LTM candidate cell.

- LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations.

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18         LTM-SSB-ResourceSet-r18,

...,

[[

ltm-NZP-CSI-RS-ResourceSet-r19      LTM-NZP-CSI-RS-ResourceSet-r19                     OPTIONAL, -- Need R

ltm-CSI-IM-ResourceSet-r19          LTM-CSI-IM-ResourceSet-r19                         OPTIONAL, -- Need R

resourceType-r19                    ENUMERATED {periodic, semiPersistent}              OPTIONAL  -- Cond CSI-RS

]]

}

LTM-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-NZP-CSI-RS-ResourceSet-r19 ::=     SEQUENCE {

ltm-CSI-RS-ResourceList-r19         SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF NZP-CSI-RS-ResourceId,

ltm-CandidateIdList-r19             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-CSI-IM-ResourceSet-r19 ::=     SEQUENCE {

ltm-CSI-IM-ResourceSetId-r19        CSI-IM-ResourceSetId,

ltm-CandidateId-r19                 LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}       OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset      OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                           OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                    OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                        OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                OPTIONAL    -- Cond LTM

qcl-InfoSemiPersistentCSI-RS        TCI-StateId                          OPTIONAL,   -- Cond LTM

]]

}

Observation 5. The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell is provided by enhancing the RRC NZP-CSI-RS-Resource IE as follows, for example:

Observation 6. The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell as described in Observations 4 and 5, involves RRC specification impact, but no impact to RAN3 signalling.

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

Regarding the following FFS issue identified in the last RAN3 #128 meeting [1], it seems that RAN2 has not concluded on it yet.

Observation 7. For both F1AP and XnAP, whether the activation/deactivation of SP CSI-RS resources in an LTM candidate cell is performed at the level of CSI-RS resource sets or individual CSI-RS resources is FFS, pending RAN2 conclusion.

# 4 Support of CSI-RS resources for LTM CSI acquisition for candidate cells

Support for configuration of periodic and semi-persistent (SP) CSI-RS resources for the purposes of CSI acquisition on LTM candidate cells is introduced in the LTM preparation procedure in the same way as for CSI-RS resources for the purposes of L1 measurement and reporting on candidate cells.

Proposal 3. The source gNB-CU requests its underlying candidate DUs and a candidate gNB-CU for configuration of periodic and SP CSI-RS resources for CSI acquisition on the candidate cells in the Handover Req message.

A candidate gNB-CU, in turn, requests its underlying candidate DUs for configuration of periodic and SP CSI-RS resources for CSI acquisition on the candidate cells in the UE Context Setup Req message.

Proposal 4. The underlying candidate DUs provide the configurations for periodic and SP CSI-RS resources for CSI acquisition for each prepared LTM candidate cell in the UE Context Setup Resp message to the source gNB-CU and the candidate gNB-CU.

The candidate gNB-CU forwards the configurations to the source gNB-CU in the Handover Req Ack message.

The configuration of CSI-RS resources for CSI acquisition can be different than the configuration of CSI-RS resources for L1 measurements and reporting – e.g., higher number of antenna ports may be needed for rank measurements in case of CSI acquisition. Therefore, it makes sense that the configuration of CSI-RS resources for CSI acquisition is separately signalled than the configuration of CSI-RS resources for L1 measurements and reporting, e.g., using different IEs.

Proposal 5. The source gNB-CU separately requests CSI-RS resource configurations for L1 measurements and reporting and for CSI acquisition in the UE Context Modification Req and Handover Req messages to its underlying candidate DUs and a candidate gNB-CU, respectively.

Similarly, a candidate gNB-CU separately requests its underlying candidate DUs for CSI-RS resource configurations for L1 measurements and reporting and for CSI acquisition using the UE Context Setup Req message.

Proposal 6. The underlying candidate DUs separately provide the configurations for CSI-RS resources for CSI acquisition and for L1 measurements and reporting for each prepared LTM candidate cell in the UE Context Setup Resp message to the source gNB-CU and the candidate gNB-CU.

The candidate gNB-CU forwards the configurations to the source gNB-CU in the Handover Req Ack message.

# 5 Configuration of periodic and SP CSI-IM resources for LTM

During LTM preparation, upon receiving a request for CSI-IM resource configuration, a candidate DU may configure periodic and/or SP CSI-IM resources for LTM CSI acquisition. The proposals are similar as for CSI-RS resource configuration for LTM CSI acquisition.

Proposal 7. The network signalling procedure for configuration of periodic and/or SP CSI-IM resources for LTM CSI acquisition is similar as that for configuration of CSI-RS resources for LTM CSI acquisition.

# 6 UE association information signalling upon LTM cell switch

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

In the previous RAN3 meeting (RAN3 #127-bis [2]), the following agreement was reached regarding UE association information signaling upon LTM cell switch. Signaling of this information from the new serving gNB to a candidate gNB enables the candidate gNB to identify the UE context.

We address the highlighted FFS issue. Upon LTM cell switch, the old serving gNB can use the Cell Switch Notification message to transmit the old target UE XnAP IDs to the new serving gNB. It is assumed that, during and after LTM cell switch, each target gNB stores the old target UE XnAP ID that it used with the old serving gNB.

Proposal 8. Upon LTM cell switch, the old serving gNB uses the Cell Switch Notification message to transmit the old target UE XnAP IDs to the new serving gNB, for supporting subsequent LTM.

# 7 Remaining issues in Early TA RACH configuration and TA Information Transfer procedures

In the RAN3 meetings so far, Early TA RACH configuration for initial LTM handover was covered. Below we discuss some aspects of the procedure for configuration for subsequent LTM.

For subsequent LTM, the serving gNB-DU needs the Early TA RACH configuration for all the configured LTM candidate cells.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

It was agreed in a previous meeting RAN3 #127 [4] that, to sync up the Early TA RACH configurations between the candidate gNBs, the LTM Configuration Update procedure may be triggered at any later point in time depending on network implementation.

For obtaining the Early TA RACH configuration, in Handover Request Ack, each candidate gNB-CU provides (candidate gNB ID, List of IDs of Early TA RACH configuration).

Proposal 9. For obtaining the Early TA RACH configuration, each candidate gNB-CU provides the pair (candidate gNB ID, List of IDs of Early TA RACH configuration) to the source gNB-CU in Handover Request Ack.

The source gNB-CU includes the information regarding the pair of (candidate gNB ID, List of IDs of Early TA RACH configuration) provided by a candidate gNB-CU, in the LTM Configuration Update procedure with the other candidate gNB-CUs, for obtaining the Early TA RACH configuration for subsequent LTM.

Include the following information in the TA information transfer message:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- FFS on the XnAP IDs.

In the previous RAN3 127-bis meeting [2], including information regarding XnAP IDs in the TA Information Transfer message over Xn was left FFS.

Since the TA Information Transfer message over Xn was agreed to be a non-UE associated procedure, the XnAP IDs do not need to be included in the message.

We think that the following additional information needs to be included in the TA Information Transfer messages over Xn and over F1 from a candidate gNB-DU to the candidate gNB-CU, so that the messages are forwarded to the correct source gNB and source gNB-DU.

- ID of Early RACH configuration,

- Source gNB ID.

Proposal 10. Since the TA Information Transfer message over Xn was agreed to be a non-UE associated procedure, the XnAP IDs are not included in the message.

Proposal 11. Beside the information already agreed, the following additional information needs to be included in the TA Information Transfer messages over Xn and over F1 from a candidate gNB-DU to the candidate gNB-CU, so that the messages are forwarded to the correct source gNB and source gNB-DU:

- ID of Early RACH configuration,

- Source gNB ID.

# 8 LTM reference configuration

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

In the previous RAN3 #127 meeting [4] the following issue was kept FFS.

Our thinking is that the source gNB generating the reference configuration for LTM is a good enough solution.

Companies proposing that a candidate gNB may also provide the LTM reference configuration were considering the case where a group of the candidate gNBs belong to a different network vendor than the source gNB. In that case, they argued that a candidate gNB belonging to the group may be able to generate a reference configuration that works for the configured candidate cells that belong to the group.

However, in that case the signaling may become more complicated, and the benefits are not clear. The source gNB can ask a candidate gNB from the group to provide the reference configuration. Then another two rounds of signaling is required so that the source gNB can send a request including the reference configuration to the other candidate gNBs, and then each candidate gNB responds with either a delta configuration with respect to the reference or with a complete configuration. Additionally source gNB itself will be from another infra vendor and candidate gNB provided configuration will not work for source gNB and source gNB has to provide full configuration for its cells. In 3GPP specification, source gNB does not know whether candidate gNBs are from same vendor or different vendor unless some OAM based approach is used, which is undesirable.

Observation 8. The case where the source gNB requests a candidate gNB to provide a reference configuration involves more signaling and complexity.

Proposal 12. The agreed solution where the source gNB generates a reference configuration for LTM as baseline is enough, and the source gNB does not request a candidate gNB to provide a reference configuration.

# 9 Remaining aspects of the Cell Switch Notification procedure

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

The following agreement was reached in a previous meeting on the Cell Switch Notification procedure.

As in Rel-18 Intra-CU LTM, it is useful to transmit information regarding the valid TA values of LTM candidate cells from the source gNB-DU to the target gNB-DU upon LTM cell switch, since the UE does not need to invoke the Early TA acquisition procedure when it is in the target cell, for those LTM candidate cells for which the TA information is already available. This impacts the Cell Switch Notification procedure since this information needs to be carried over it.

Proposal 13. The Xn Cell Switch Notification message includes the TA information list, which consists of a list of pairs of (LTM candidate cell ID, TA value). The source gNB-DU ensures that only valid TA values are transmitted, and in the message provides the TA validity timer values associated with the TA values.

# 10 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

Transmission of CSI-RS-based beam measurements over F1-AP for L3 measurements-based LTM

Proposal 1. In Rel-19 LTM, for L3 measurements-based LTM solution, the serving gNB-CU shall send CSI-RS-based beam measurements to the serving gNB-DU in the F1-AP CU-DU Mobility Initiation Request message, in addition to the SSB-based beam measurements and cell level measurements.

Configuration of semi-persistent (SP) CSI-RS resources for LTM

Observation 1. In legacy NR, for a serving cell, an SP CSI-RS resource has no pre-defined TCI state ID, and it is dynamically assigned when SP CSI-RS is activated. The TCI state ID of the activated SP CSI-RS resource is indicated in the SP CSI-RS/CSI-IM activation/deactivation MAC CE.

Observation 2a. In Rel-19 LTM, RAN3/RAN2 has defined the following signalling procedure for activation/deactivation of SP CSI-RS resources of an LTM candidate cell:

- The source DU indicates in signalling messages via F1 and Xn to a candidate DU activation/deactivation of SP CSI-RS resources of a candidate cell.

- The candidate DU confirms the activation/deactivation in its response message to the source DU.

- The source DU then transmits the activation/deactivation indication to the UE in the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE.

Observation 2b. In the response message to the source DU, the candidate DU does not indicate the TCI state IDs for the activated SP CSI-RS resources, so that the source DU is unable to include these TCI state IDs along with the activated SP CSI-RS resources in the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE (defined for LTM) to the UE.

Observation 3. If TCI states are configured for SP CSI-RS resources of an LTM candidate cell during the preparation procedure for LTM configuration, then there are the following advantages.

- Signalling of TCI states from candidate DU to the source DU during activation/deactivation of SP CSI-RS resources of candidate cells, can be avoided.

- TCI state ID information does not need to be included in the activation/deactivation signalling using MAC CE from the source DU to the UE.

Proposal 2. During LTM preparation, upon receiving a request from the source gNB-CU (via a candidate gNB-CU) for CSI-RS resource configuration, a candidate gNB-DU provides the configuration of TCI state IDs for SP CSI-RS resources configured for a candidate cell in its response message (UE Context Setup Resp or UE Context Modification Resp).

- The configuration of TCI state IDs for SP CSI-RS resources is forwarded to the source gNB-CU by the candidate gNB-CU in the Handover Req Ack message.

- The source CU includes the provided configuration in the LTM configuration signalled to the UE.

Observation 4. The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell is included in the CSI-RS Resource Configuration To AddModList IE in the UE Context Setup Resp and UE Context Modification Resp messages, and this IE contains the ltm-CSI-ResourceConfigToAddModList, as defined in the running RRC CR for Rel-19 LTM.

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}       OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset      OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                           OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                    OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                        OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                OPTIONAL    -- Cond LTM

qcl-InfoSemiPersistentCSI-RS        TCI-StateId                          OPTIONAL,   -- Cond LTM

]]

}

Observation 5. The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell is provided by enhancing the RRC NZP-CSI-RS-Resource IE as follows, for example:

Observation 6. The configuration of TCI state IDs for SP CSI-RS resources of a candidate cell as described in Observations 4 and 5, involves RRC specification impact, but no impact to RAN3 signalling.

Observation 7. For both F1AP and XnAP, whether the activation/deactivation of SP CSI-RS resources in an LTM candidate cell is performed at the level of CSI-RS resource sets or individual CSI-RS resources is FFS, pending RAN2 conclusion.

Support of CSI-RS resources for LTM CSI acquisition for candidate cells

Proposal 3. The source gNB-CU requests its underlying candidate DUs and a candidate gNB-CU for configuration of periodic and SP CSI-RS resources for CSI acquisition on the candidate cells in the Handover Req message.

A candidate gNB-CU, in turn, requests its underlying candidate DUs for configuration of periodic and SP CSI-RS resources for CSI acquisition on the candidate cells in the UE Context Setup Req message.

Proposal 4. The underlying candidate DUs provide the configurations for periodic and SP CSI-RS resources for CSI acquisition for each prepared LTM candidate cell in the UE Context Setup Resp message to the source gNB-CU and the candidate gNB-CU.

The candidate gNB-CU forwards the configurations to the source gNB-CU in the Handover Req Ack message.

Proposal 5. The source gNB-CU separately requests CSI-RS resource configurations for L1 measurements and reporting and for CSI acquisition in the UE Context Modification Req and Handover Req messages to its underlying candidate DUs and a candidate gNB-CU, respectively.

Similarly, a candidate gNB-CU separately requests its underlying candidate DUs for CSI-RS resource configurations for L1 measurements and reporting and for CSI acquisition using the UE Context Setup Req message.

Proposal 6. The underlying candidate DUs separately provide the configurations for CSI-RS resources for CSI acquisition and for L1 measurements and reporting for each prepared LTM candidate cell in the UE Context Setup Resp message to the source gNB-CU and the candidate gNB-CU.

The candidate gNB-CU forwards the configurations to the source gNB-CU in the Handover Req Ack message.

Configuration of periodic and SP CSI-IM resources for LTM

Proposal 7. The network signalling procedure for configuration of periodic and/or SP CSI-IM resources for LTM CSI acquisition is similar as that for configuration of CSI-RS resources for LTM CSI acquisition.

UE association information signalling upon LTM cell switch

Proposal 8. Upon LTM cell switch, the old serving gNB uses the Cell Switch Notification message to transmit the old target UE XnAP IDs to the new serving gNB, for supporting subsequent LTM.

Remaining issues in Early TA RACH configuration and TA Information Transfer procedures

Proposal 9. For obtaining the Early TA RACH configuration, each candidate gNB-CU provides the pair (candidate gNB ID, List of IDs of Early TA RACH configuration) to the source gNB-CU in Handover Request Ack.

Proposal 10. Since the TA Information Transfer message over Xn was agreed to be a non-UE associated procedure, the XnAP IDs are not included in the message.

Proposal 11. Beside the information already agreed, the following additional information needs to be included in the TA Information Transfer messages over Xn and over F1 from a candidate gNB-DU to the candidate gNB-CU, so that the messages are forwarded to the correct source gNB and source gNB-DU:

- ID of Early RACH configuration,

- Source gNB ID.

LTM reference configuration

Observation 8. The case where the source gNB requests a candidate gNB to provide a reference configuration involves more signaling and complexity.

Proposal 12. The agreed solution where the source gNB generates a reference configuration for LTM as baseline is enough, and the source gNB does not request a candidate gNB to provide a reference configuration.

Remaining aspects of the Cell Switch Notification procedure

Proposal 13. The Xn Cell Switch Notification message includes the TA information list, which consists of a list of pairs of (LTM candidate cell ID, TA value). The source gNB-DU ensures that only valid TA values are transmitted, and in the message provides the TA validity timer values associated with the TA values.

# References

[1] Chair notes, RAN3 #128 meeting.

[2] Chair notes, RAN3 #127-bis meeting.

[3] Chair notes, RAN2 #129-bis meeting.

[4] Chair notes, RAN3 #127 meeting.