3GPP TSG-RAN WG3 Meeting #129		R3-255389

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:	Discussion on XR rate control

Agenda Item:	21.3

Document for:	Discussion and Decision

- Introduction

In last RAN3 meeting, UL rate control for XR has been discussed and achieved the following agreements [1].

CU sends the uplink rate control indication per QoS flow over F1 to DU. Introduce a F1AP IE, similar to NGAP Indication of Bitrate Adaptation IE.

RAN3 also sent the related agreement in a LS [2] to RAN2 with the below content.

To support UL rate control, RAN3 has agreed to introduce a per-QoS Flow indication over F1 interface, to enable the gNB-CU to inform the gNB-DU that the QoS Flow allows rate adaptation in the uplink direction.

In this contribution, we will discuss the remaining open issues related to UL XR rate control on CN coordination and rate control negotiation between two RAN nodes in case of DC configuration.

- Discussion

  - Discussion on CN coordination on XR rate control

In last RAN2 meeting, it was agreed UL XR rate control can be used for both GBR and non-GBR services, and corresponding LS was sent to SA2 [3]. If such UL XR rate control is applied for GBR service, it is not clear whether RAN node could adjust UL bit rate lower than GBR value and whether such rate control mechanism should be depended on AQP. As a result, some coordination is needed between CN and RAN.

Furthermore, since CN has already defined ECN marking for L4S to expose congestion information so that application could adjust its codec accordingly. Once radio congestion happened, gNB based UL rate control would send to UE to adjust application codec rate. CN may also need to be informed so that it can decide to adjust related function. For example, CN may de-active ECN marking for L4S UL to avoid duplicated operation.

Proposal 1: gNB may inform SMF/UPF when UL rate control MAC CE send to UE in order to coordinate with CN for congestion handling.

  - Discussion on XR rate control in DC configuration

In last RAN2 meeting, it was agreed that “No optimization is needed to address DC case for XR rate control MAC CE, and captured in stage-2 that XR rate indication is for application and not for MAC entity”.

In Rel-18, we have discussed the PSI-based SDU discard activation in case DC is configured, and agreed that NG RAN nodes will not perform co-ordination and UE will always follow the latest activation or deactivation command via MAC CE. However, in case of XR rate control, if the indication from gNB-DU is used to suggest the maximum bit rate for certain application according to RAN2 agreement. Thus, such bit rate cannot be only determined by radio condition of one RAN node in DC configuration. For example, if one RAN node is fully congested, it may suggest UE to adjust to a very low UL bit rate value, however, the actual available uplink bit rate could be higher due to available radio resources in another RAN node. It may affect UE experience to simply adjust UE UL bit rate according to rate control suggestion independently.

As a result, it is suggested M-gNB and S-gNB perform bit rate co-ordination and send the overall value to UE for simplicity. One detailed example would be S-gNB-DU send its suggested bit rate per QoS flow to M-gNB-DU via F1AP and XnAP message. And only M-gNB DU is allowed to send the XR rate control MAC CE to affect UE. Furthermore, user plane GTP-U header could be also used to send such suggested bit rate information.

Proposal 2: In case of DC configuration, S-NG RAN node and M-NG RAN node need to coordinate the suggested bit rate for XR rate control and one rate control MAC CE would be sent to UE.

- Conclusion

In this contribution, we discussed open issues related to UL XR rate control on CN coordination and rate control negotiation between two RAN nodes in case of DC configuration. Based on the discussion, we have the following proposals:

Proposal 1: gNB may inform SMF/UPF when UL rate control MAC CE send to UE in order to coordinate with CN for congestion handling.

Proposal 2: In case of DC configuration, S-NG RAN node and M-NG RAN node need to coordinate the suggested bit rate for XR rate control and one rate control MAC CE would be sent to UE.

- References

- Draft_RAN3#128 Meeting Report

- R3-253927, LS on uplink rate control

- R2-2504815, Reply LS to SA2 on XR rate control