3GPP TSG-RAN WG3 #129	R3-255398

Bangaluru, India, 25th – 29th August 2025

Agenda Item:	11.4

Source:	Lenovo

Title: 		(TP to BLCR 37.340) Discussion on UE performance collection in AIML for NR-DC

Document for:	Discussion and Approval

# Introduction

In this paper, we discuss the left issues related to AI in NR DC.

# Discussion

One aspect to be discussed is if the existing list of UE performance metrics can be reused for DC scenario or not. Note that after SN addition or SN change, the following types of radio bearers could exist depending on where it is anchored.

- MN terminated MCG/SCG/split bearer with QoS flow anchored at MN

- SN terminated MCG/SCG/split bearer with QoS flow anchored at SN

There are six types of radio bearers in DC scenario: MN terminated MCG bearer, MN terminated SCG bearer, MN terminated split bearer, SN terminated MCG bearer, SN terminated SCG bearer, SN terminated split bearer

In the previous RAN3 meeting, the following has been agreed

- Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec

- DL/UL UE Throughput Measurement will not be supported in split architecture in R19.

- Introduce the start and stop indication over F1 for Delay measurement.

- Introduce Data Collection ID in UE associated messages over E1

Above agreements basically mean for NR DC, only UE performance of SN terminated MCG/SCG/Split bearers will be measured and reported from SN to MN, wherein the PDCP entity locates in SN CU (UP). And it is suggested to clarify in stage 2 37.340 BL CR.

RAN3 confirms that the UE performance reported from SN to MN after SN addition/change concerns only the SN terminated MCG/SCG/Split bearers. TP to BLCR 37.340 is proposed in the Annex to clarify.

Another aspect raised by some companies is that if the start/stop of UE performance collection after SN addition/change should consider SCG activation/deactivation, i.e., UE performance can only be collected if SCG is activated.

In our view, it can be handled by network implementation considering:

- MN is aware of the SCG activation/deactivation state during or after the SN addition/change.

- SCG can be activated or deactivated many times within the data collection duration.

- It is possible that MN can add a SN with SCG deactivated, in this case a reasonable MN implementation shall not initiate the UE performance collection after SN addition/change.

In other word, it seems nothing breaks if SCG deactivation is not specified as an exit condition that terminates the UE performance collection after SN addition/change.

SCG deactivation is not specified as an exit condition that terminates the UE performance collection after SN addition/change

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	There are six types of radio bearers in DC scenario: MN terminated MCG bearer, MN terminated SCG bearer, MN terminated split bearer, SN terminated MCG bearer, SN terminated SCG bearer, SN terminated split bearer

Based on the discussion above, we propose:

Proposal 1	RAN3 confirms that the UE performance reported from SN to MN after SN addition/change concerns only the SN terminated MCG/SCG/Split bearers. TP to BLCR 37.340 is proposed in the Annex to clarify.

Proposal 2	SCG deactivation is not specified as an exit condition that terminates the UE performance collection after SN addition/change

# Annex: TP to BLCR 37.340

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>

## 13.x	AI/ML for NR-DC

In NR-DC, an MN that supports AI/ML for NG-RAN as specified in TS 38.300 [3], can configure the collection and reporting of UE performance measurements concerning SN terminated radio bearers from the SN after successful SN addition or SN change, if supported.

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>