3GPP TSG-RAN WG3 Meeting #129	R3-255434

Bengaluru, India, 25 – 29 August, 2025

Agenda Item:	11.3

Source:	Nokia

Title:	(TP for BLCR to TS 38.300, 38.401, 38.423, 38.473) Remaining issues for AI/ML-based CCO

Document for:	Discussion and TP

# 1	Introduction

We here give our view on how to close remaining details for AI/ML-based CCO. These are:

- Stage 2: We propose description for TS 38.300 and TS 38.401. Stage 2 description is required to complete the work item, so we believe it should have high priority in this meeting.

- Update/cancel mechanism for future CCO state decision: An initial agreement was taken at last meeting, and we here provide further discussion and proposals.

- Stage 3 details: We provide discussion to consolidate the updates of IEs used for signalling of future coverage modification to neighbour DUs and neighbour nodes, and we address the question of related updates of the procedural text. We also look at issues of table dimensioning and ASN.1.

# 2	Discussion

## 2.1	Stage 2

For non-split architecture, legacy CCO is described in TS 38.300 clause 15.5.5 while support of AI/ML for NG-RAN is described in clause 16.20. This latter clause includes clause 16.20.3 which describes data collection and reporting. However, in line with the conclusions from the study item, the main Rel-19 work for AI/ML-based CCO is related to signalling for inter-node coordination in case of future CCO state change, which is different from data collection and reporting for AI/ML. We therefore believe that main stage 2 description for the Rel-19 AI/ML-based CCO feature needs to be described in clause 15.5.5, and we also propose to introduce a reference to this description from clause 16.20.3.

Proposal 1: For non-split architecture, introduce main stage 2 description for the Rel-19 AI/ML-based CCO feature in TS 38.300 clause 15.5.5 with reference from clause 16.20.3.

Similarly, we propose to describe AI/ML-based CCO for non-split architecture in TS 38.401 clause 7.9, with reference from clause 7.11.

Proposal 2: Describe AI/ML-based CCO for split architecture in TS 38.401 clause 7.9 with reference from clause 7.11.

## 2.2	Update/cancel of future CCO state decision

At last RAN3 meeting the following agreement was taken for an update/cancel mechanism for predictions:

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

The purpose of AI/ML based CCO is to enable proactive resolution or mitigation of CCO issues. In the current signalling the gNB-DU indicates decided future coverage configuration changes with the intention that neighbour nodes can apply corresponding matching coverage configurations in a planned manner. This may improve e.g., resource management in neighbour nodes since the required resources may be planned in advance. Cancellation or update of the future coverage configuration decision removes this benefit and therefore in our view should not be considered a nominal scenario, and should be avoided when possible.

Observation 1: Because AI/ML based CCO is intended to enable proactive resolution or mitigation of CCO issues and also give visibility of future coverage state, cancellation or update of such future coverage state is not a nominal scenario and should be avoided when possible.

This is also why we didn’t agree at last meeting to introduce a fully flexible signalling framework for cancellation of future coverage changes (e.g. including transaction ID). Still, in line with the agreement at last meeting, cancel or update such coverage state change may still need to be done in some cases where there is a legitime reason to do so. In this context we remind that the prediction of a CCO issue needs to be made based on information about the cell state. Thus, if the cell state need to be modified this is  a legitime reason to cancel or update the corresponding decision of future coverage state.

Observation 2: Change of the cell state after CCO issue prediction is a reason to cancel or update the corresponding decision of future coverage state.

We next look into scenarios where the cell state used for CCO issue prediction would change after the prediction was made.

Scenario of second CCO issue appearing in same cell where initial prediction was made

One scenario which includes cell state change is when an immediate or imminent CCO issue appears in the cell where the initial prediction was made and needs to be resolved before the coverage configuration modification resulting from the initial predicted CCO issue is executed.

Observation 3: Resolving an immediate or imminent CCO issue in the same cell as where an initial CCO issue was predicted will change the cell state and hence the initial prediction may not be valid anymore.

Such scenario could be considered as originating in a prediction error, but a robust AI/ML CCO solution should in our view support handling of this scenario. We therefore propose to support cancel/update of a decided future coverage state change in this scenario.

Proposal 3: Support cancel/update of an initially decided future coverage state change if an immediate or imminent CCO issue in the same cell needs to be resolved before the initially decided future coverage state change is executed.

Signalling for such scenario is illustrated in Figure 1 below.

Figure 1: Detection of immediate CCO issue (left) or prediction of imminent CCO issue (right) after an initial future coverage modification has been notified over Xn but not executed. The corresponding CCO issues are located in the same cell. “Future Coverage Modification” and “Immediate Coverage Modification” correspond to Coverage Modification List IE and the Future Coverage Modification List IE, respectively, contained in the NG-RAN NODE CONFIGURATION UPDATE message.

As shown in Figure 1 the CCO state change resulting from the immediate or imminent CCO issue replaces the initially decided CCO state change. For example, for a new imminent CCO issue (right hand side of Figure 1), the gNB-A predicts a future CCO issue in the same cell as the initial CCO issue and decides to apply a new coverage state (A2) at t2 which expires before t1. Of course, in split architecture it will be the gNB-CU that predicts the imminent issue, and will be up to the gNB-DU to determine any new coverage state A2 that can resolve or mitigate the issue and the activation time of A2 (named t2) in the drawing. The handling on the gNB-A side therefore strictly follows the currently agreed behaviour both in non-split and split architecture, supported in the current stage 3, with the notable exception that the initially decided future coverage state A1 will not be applied. This will need to be described in the specification.

The behaviour in case of immediate CCO issue (left hand side of Figure 1) is similar, except the new coverage state is applied immediately so no new timer t2 is started. Also, in this case the initially decided future coverage state A1 will not be applied.

The only thing that is needed is to clarify in the present scenario is therefore that the latest received coverage modification notification replaces an earlier received but not executed notification. This relates to interaction between two signalling procedures on the same interface and could therefore either be captured in stage 3 or in stage 2. However, we think that stage 2 description would be the simplest solution, and therefore propose the following text for TS 38.300:

“If a gNB receives a new notification about modified coverage as described in clause 15.5.5.3 or future coverage state before execution of a previously received notification about future coverage state for the same cell(s), the gNB takes into account the latest received notification replacing the previously received non executed notification.”

For split architecture the following description could be captured in TS 38.401:

“If a gNB-DU receives a new notification about modified coverage as described in clause 7.9.3 or future coverage state before execution of a previously received notification about future coverage state for the same cell(s), the gNB-DU takes into account the latest received notification replacing the previously received non executed notification.”

Proposal 4: Capture above TPs for cancel/update in stage 2 (TS 38.300, TS 38.401).

Scenario of second CCO issue appearing in a cell different from where the initial prediction was made

The cell state used for the prediction of a CCO issue in a cell (Cell A) may be updated also in case an immediate or imminent CCO issue is detected/predicted in a neighbour cell if Cell A needs to adopt a new matching coverage state due to resolution of an immediate or imminent CCO issue in a neighbour cell.

Observation 4: Adoption of a matching coverage state in a cell A due to resolution of CCO issue in neighbour cell will change cell state that was used for the CCO prediction in cell A.

An important difference between this neighbour cell scenario and the scenario described in Figure 1 (same cell), is that a model deployed to detect CCO issues in a cell A would not be expected to be able to take into account potential CCO issues in neighbour cells. And this is even more true if the neighbour cell belongs to a different gNB. We therefore believe the standard should support cancel/update of an initial decided future coverage state change in this case.

Proposal 5: Support cancel/update of an initial decided future coverage state change in a cell if adoption of a matching CCO state, due to resolution of an immediate or imminent CCO issue in a neighbour cell, is required before the initial decided future coverage state change is executed.

A corresponding signalling flow is shown in Figure 2. Cell A’s neighbour cell could as mentioned be a cell served by the gNB-A or some neighbour gNB, e.g. gNB-B. However, in order to describe a generic scenario, we consider in the figure below that this neighbour is served by a third gNB, named gNB-C. The concerned cell in gNB-C has a neighbour relation with cell A, and may or may not have a neighbour relation with cell served by gNB-B.

Figure 2: Detection of immediate CCO issue (left) or prediction of imminent CCO issue (right) after an initial future coverage modification has been notified over Xn but not executed. The corresponding CCO issues are located in different cells. An applicable cause value for immediate/imminent CCO state change (in red) is currently not defined. “Future Coverage Modification” and “Immediate Coverage Modification” correspond to Coverage Modification List IE and the Future Coverage Modification List IE, respectively, contained in the NG-RAN NODE CONFIGURATION UPDATE message.

Figure 2 shows that the CCO state change resulting from the immediate or imminent CCO issue in a neighbour cell (in this example belonging to a cell controlled by gNB-C) will result in replacement of the initially decided CCO state change due to matching coverage state needed to be adopted in cell A. In this scenario, the gNB-A receives the activation time t2 (related to resolution of the second CCO issue) from a neighbour cell (or neighbour node gNB-C as illustrated). The gNB-A determines that t2 will expire before t1 and decides to apply coverage state A2 at expiry of t2. Coverage state A1 will not be applied. In split architecture the gNB-CU will simply forward the received incoming notification to the concerned gNB-DU, and it will be up to the gNB-DU to determine the matching coverage state A2. It will also be the gNB-DU that determines that t2 will expire before t1, and it will in this case need to notify the gNB-CU about the decision to activate a new coverage state A2. The gNB-CU can then inform other gNB-DUs and neighbour nodes over Xn about this decision so that they will be aware that the initially decided coverage state A1 will not be applied. This can be done based on the already agreed signalling, however we believe that a new coverage modification cause (or future coverage modification cause) is required for this purpose. This is because the legacy coverage modification causes are intended to indicate “the reason for the predicted coverage modification in NG-RAN node1” (as expressed in XnAP). In the present example the “NG-RAN node1” corresponds to the gNB-C, while the decision to apply coverage state A2 in gNB-A comes as a result of adoption of a matching coverage state so hence not as a result of a CCO issue in the gNB-A. It is important that neighbour nodes have correct information about the cause of the coverage modification change, because they use this information to determine the correct matching coverage state on their side (e.g. a change triggered by ‘coverage’ issue will not lead to the same matching coverage state in a neighbour node as a change triggered by ‘cell edge capacity’ issue).

Based on this we conclude from Figure 2 that the signalling is in essence already supported from stage 3 point of view, with the exception of the cause value for the immediate or imminent CCO state change which is currently not defined. Therefore, in the present scenario it should be possible to signal that the new CCO state in cell A comes as a consequence of resolution of a CCO issue in a neighbour cell (served by gNB-C in the example of Figure 2).

Proposal 6: Introduce a new code-point for coverage configuration modification cause (F1, Xn) reflecting adoption of matching CCO state following resolution of CCO issue in neighbour cell.

This new codepoint could be named e.g. “Matching coverage state” or “CCO issue in other cell”.

## 2.3	Remaining stage 3 details

### 2.3.1	Signalling of future coverage modification to neighbour DUs and neighbour nodes

RAN3#128 agreed that the gNB-CU directly forwards the received future CCO state of neighbor cells to gNB-DU. A corresponding stage 3 TP was also agreed, introducing a new IE carrying this information. In order to ensure that the specification remains consistent after this change, we here provide some further analysis of use of the new IE.

The current option is that the new F1AP Neighbour Future Coverage Modification Notification IE (9.3.1.C) will always be signalled together with the Predicted CCO Assistance Information IE (9.3.1.A) within the GNB-CU CONFIGURATION UPDATE message. This is shown in the following signalling flow:

Figure 3: Illustration of F1 and Xn signalling for a predicted CCO issue

As shown in Figure 3, CU1 predicts the CCO issue that involves DU1 and informs it using Predicted CCO Assistance Information IE (9.3.1.A). In response DU1 determines and provides the Future CCO state in Future Coverage Modification Notification IE. CU1 informs of this Future CCO state of DU1, i.e. cells/beams of DU1, to its other served DUs (DU2, DU3, …, DUn) using Predicted CCO Assistance Information IE (9.3.1.A) and Neighbour Future Coverage Modification Notification (9.3.1.C). Based on the information received, these DUs may determine the matching Future CCO state for their cells (as the NR CGI IE contained in the Predicted Affected Cells and Beams IE will not be served by these DUs).

Further, CU1 provides the Future CCO state information of its DUs to the neighbouring nodes, herein CU2, using Future Coverage Modification List IE that also contains the Predicted Coverage Modification Cause indicating the cause of planned Future CCO state change. Upon receiving this information over Xn, CU2 informs its served DUs (DU1’, DU2’, …, DUm’) using Predicted CCO Assistance Information IE (9.3.1.A) and Neighbour Future Coverage Modification Notification IE (9.3.1.C). Using this information the DUs (DU1’, DU2’, …, DUm’) may determine the matching Future CCO state for their cells.

Since the Future Coverage Modification Cause IE was considered redundant in the presence of the Predicted CCO issue IE contained in the Predicted CCO Assistance Information IE (9.3.1.A), it was omitted from the new IE (9.3.1.C). However, designing a suitable procedural text that can ensure inclusion of both 9.3.1.A and 9.3.1.C in the GNB-CU CONFIGURATION UPDATE message is not straight forward. A better solution would be to further update the new IE (9.3.1.C) to simply reflect what is sent over Xn, which would mean to include the Predicted Coverage Modification Cause IE (or simply name it Future Coverage Modification Cause) and the Predicted Affected Cells and Beams IE (which the CU2 will populate based on cell/beam information received over Xn). Then, it would not be needed to send the Predicted CCO Assistance Information IE if the Neighbour Future Coverage Modification Notification IE is sent.

Proposal 7: Include the Future Coverage Modification Cause IE and the Predicted Affected Cells and Beams IE in the new Neighbour Future Coverage Modification Notification IE (9.3.1.C).

Current F1AP procedural text requires that the receiving gNB-DU shall use the content of Predicted Affected Cells and Beams IE to determine whether the predicted CCO issue applies to a cell served by the gNB-DU, or if the notification is a request to adopt a matching future CCO state following a future CCO decision taken by another gNB-DU. However, the introduction of the new Neighbour Future Coverage Modification Notification IE makes it possible to use the received IE to determine the nature of the CCO action to be taken by the gNB-DU. We therefore propose to update the F1AP procedural text accordingly.

Proposal 8: Update the F1AP procedural text to enable the gNB-DU to use the received IE to determine the nature of the CCO action to be taken.

A corresponding update is provided in the F1AP TP in annex of this paper.

### 2.3.2	Dimensioning of the Future Coverage Modification List

The new IE 9.3.1.C is intended to convey information received in the NG-RAN NODE CONFIGURATION UPDATE message. However, we notice that the XnAP Future Coverage Modification List IE in this message as per current BLCR may contain information for 16384 cells:

| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Future Coverage Modification Item |  | 1..<maxnoofCellsinNG-RAN node> |  |  | – |  |

It therefore seems needed that also the F1AP Neighbour Future Coverage Modification Notification List IE in 9.3.1.C is dimensioned for 16384 cells.

Proposal 9: Update the max number of cells in Neighbour Future Coverage Modification Notification List in 9.3.1.C to 16384.

### 2.3.3	ASN.1: Extendibility of the Future Coverage Modification Cause

In the current BLCR for F1AP [2] as shown below both futureCoverageModificationCause IE and predicted-CCO-issue-detection IE are using enumeration Predicted-CCO-issue-detection that has two codepoints coverage and cell-edge-capacity.

Due to this, futureCoverageModificationCause and predicted-CCO-issue-detection IEs are bound to the enumerated Predicted-CCO-issue-detection. This necessarily means that change in the code point of one IE cannot be done independently, for e.g. the possibility to add new code point to futureCoverageModificationCause IE without adding it to and predicted-CCO-issue-detection IE is not possible, i.e., Predicted CCO issue IE and Future Coverage Modification Cause IE should have distinct ASN.1 definitions, in order to enable additional codepoints in one IE without touching the other IE. For instance, NES cause might in future need to be added in the Future Coverage Modification Cause IE but not in the Predicted CCO issue IE. Therefore, we propose that futureCoverageModificationCause IE should have separate enumeration. i.e. not using Predicted-CCO-issue-detection.

Proposal 10: RAN3 to define separate ENUMERATED for futureCoverageModificationCause IE i.e. not using the Predicted-CCO-issue-detection ENUMERATED (F1AP).

# 3	Conclusion

We have made the following observations and proposals:

Proposal 1: For non-split architecture, introduce main stage 2 description for the Rel-19 AI/ML-based CCO feature in TS 38.300 clause 15.5.5 with reference from clause 16.20.3.

Proposal 2: Describe AI/ML-based CCO for split architecture in TS 38.401 clause 7.9 with reference from clause 7.11.

Observation 1: Because AI/ML based CCO is intended to enable proactive resolution or mitigation of CCO issues and also give visibility of future coverage state, cancellation or update of such future coverage state is not a nominal scenario and should be avoided when possible.

Observation 2: Change of the cell state after CCO issue prediction is a reason to cancel or update the corresponding decision of future coverage state.

Observation 3: Resolving an immediate or imminent CCO issue in the same cell as where an initial CCO issue was predicted will change the cell state and hence the initial prediction may not be valid anymore.

Proposal 3: Support cancel/update of an initially decided future coverage state change if an immediate or imminent CCO issue in the same cell needs to be resolved before the initially decided future coverage state change is executed.

Proposal 4: Capture above TPs for cancel/update in stage 2 (TS 38.300, TS 38.401).

Observation 4: Adoption of a matching coverage state in a cell A due to resolution of CCO issue in neighbour cell will change cell state that was used for the CCO prediction in cell A.

Proposal 5: Support cancel/update of an initial decided future coverage state change in a cell if adoption of a matching CCO state, due to resolution of an immediate or imminent CCO issue in a neighbour cell, is required before the initial decided future coverage state change is executed.

Proposal 6: Introduce a new code-point for coverage configuration modification cause (F1, Xn) reflecting adoption of matching CCO state following resolution of CCO issue in neighbour cell.

Proposal 7: Include the Future Coverage Modification Cause IE and the Predicted Affected Cells and Beams IE in the new Neighbour Future Coverage Modification Notification IE (9.3.1.C).

Proposal 8: Update the F1AP procedural text to enable the gNB-DU to use the received IE to determine the nature of the CCO action to be taken.

Proposal 9: Update the max number of cells in Neighbour Future Coverage Modification Notification List in 9.3.1.C to 16384.

Proposal 10: RAN3 to define separate ENUMERATED for futureCoverageModificationCause IE i.e. not using the Predicted-CCO-issue-detection ENUMERATED (F1AP).

Corresponding TPs to TS 38.300, TS 38.401, TS 38.423 and TS 38.473 are provided in annexes of this paper.

# References

[1]		R3-253986, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN

[2]		R3-253988, (BL CR to 38.473) Support of enhancements on AI/ML for NG-RAN

# Annex	- TP for BLCR to TS 38.300

<<< start of changes >>>

### 15.5.5	Support for Coverage and Capacity Optimisation

#### 15.5.5.1	General

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues, e.g. coverage and cell edge interference issues. The objective of AI/ML enabled CCO is to predict a future CCO issue and to prevent (or limit at an early stage) the rise of this future CCO issue.

#### 15.5.5.2	OAM requirements

Each NG-RAN node may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

#### 15.5.5.3	Dynamic coverage configuration changes

An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator.

The coverage state indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the coverage state indicator to retrieve a previously stored Mobility Robustness Optimisation state. The coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells coverage configurations.

If the list includes indication about planned reconfiguration and possibly a list of replacing cells, the receiving NG-RAN node may use this to avoid connection or re-establishment failures during the reconfiguration. Also, if the sending NG-RAN node adds cells in inactive state, the receiving NG-RAN node may use this information to avoid connection or re-establishment failures. The receiving NG-RAN node may also use the notification to reduce the impact on mobility. The receiving NG-RAN node should avoid triggering handovers towards cell(s) that are indicated to be inactive.

#### 15.5.5.x	AI/ML enabled Coverage and Capacity Optimization

When a CCO issue is predicted the gNB determines a corresponding future coverage state and time at which this coverage state will be applied. The gNB may notify its neighbour gNBs ahead of execution of the change using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with Future Coverage State and the associated Coverage Modification Cause and time information.

If a gNB receives a new notification about modified coverage as described in clause 15.5.5.3 or future coverage state before execution of a previously received notification about future coverage state for the same cell(s), the gNB takes into account the latest received notification replacing the previously received non executed notification.

<<< next change >>>

## 16.20	Support of AI/ML for NG-RAN

### 16.20.1	General

Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization, Network Slicing, Coverage and Capacity Optimization.

### 16.20.2	Principles

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

Support of AI/ML for NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [64].

AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [64].

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information, including radio resource status per slice;

-	UE performance feedback, including per slice UE performance;

-	Measured UE trajectory;

-	Energy Cost (EC);

-	Predicted slice available capacity

The collection and reporting of the above information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Signalling support related to AI/ML enabled Coverage and Capacity Optimization is described in clause 15.5.5.x and in case of split gNB in TS 38.401 [4].

<<< end of changes >>>

# Annex	- TP for BLCR to TS 38.401

<<< start of changes >>>

## 7.9	Support for CCO

### 7.9.1	General

The NR Capacity and Coverage Optimization (CCO) Function and AI/ML enabled CCO function in non-split gNB case are specified in TS 38.300 [2]. The objective of this function is to detect and mitigate coverage and cell edge interference issues. The objective of AI/ML enabled CCO is to predict a future CCO issue and to prevent (or limit at an early stage) the rise of this future CCO issue.

### 7.9.2	OAM requirements

Each gNB-DU may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

### 7.9.3	Dynamic coverage configuration changes

In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves the CCO issue concerning own served cell by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the CCO configuration. The gNB-DU informs the gNB-CU of the new coverage states adopted.

### 7.9.x	AI/ML enabled CCO

In case of split gNB architecture, CCO issue prediction function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the predicted CCO issue with the predicted affected cells and beams and the time for predicted CCO issue. Based on this information and local information, the gNB-DU generates future coverage state(s) for own served cell(s) and the time when the future coverage state(s) will be applied, aiming at resolving the predicted CCO issue. The gNB-DU may also take into account information received for other cells when generates the future CCO configuration. The gNB-DU informs the gNB-CU of the future coverage state and the time when the future coverage state will be applied. The gNB-CU may forward this information over the F1 interface to gNB-DUs serving neighbour cells, and/or over the Xn interface to gNBs serving neighbour cells.

Upon reception over the Xn interface of information about future coverage state and the time when the future coverage state will be applied in a cell, the gNB-CU may forward this information to concerned gNB-DUs.

If a gNB-DU receives a new notification about modified coverage as described in clause 7.9.3 or future coverage state before execution of a previously received notification about future coverage state for the same cell(s), the gNB-DU takes into account the latest received notification replacing the previously received non executed notification.

<<< end of changes >>>

# Annex	- TP for BLCR to TS 38.423

<<< start of changes >>>

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Update of Cell Coverage:

If the Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information in the Cell Coverage State IE to identify the cell deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9].

-	If the Cell Deployment Status Indicator IE is present in the Coverage Modification List IE, the NG-RAN node2 shall consider the cell deployment configuration of the cell to be modified as the next planned configuration and shall remove any planned configuration stored for this cell.

-	If the Cell Deployment Status Indicator IE is present and the Cell Replacing Info IE contains non-empty cell list, the NG-RAN node2 may use this list to avoid connection or re-establishment failures during the reconfiguration, e.g. consider the cells in the list as possible alternative handover targets.

-	If the Cell Deployment Status Indicator IE is not present, the NG-RAN node2 shall consider the cell deployment configuration of cell to be modified as activated and replace any previous configuration for the cells indicated in the Coverage Modification List IE.

If the SSB Coverage Modification List IE is present in the Coverage Modification List IE, the NG-RAN node2 may use the information in the SSB Coverage State IE to identify the SSB beam deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

If the Coverage Modification Cause IE set to "coverage" or "cell edge capacity" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue detected at NG-RAN node1 and for configuring coverage state of its served cell(s).

If the Coverage Modification Cause IE set to "matching coverage state" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for configuring coverage state of its served cell(s).

If the Coverage Modification Cause IE set to "network energy saving" is present and the SSB Coverage State IE is zero for a set of SSB beams in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to decide the SSB beam activation for the concerned beams when necessary.

Update of Future Cell Coverage:

If the Future Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to identify the future cell deployment configuration to be enabled by the NG-RAN node1 at a time indicated by Time for Future Coverage State IE and for configuring mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9].

If the Future SSB Modification List IE is present in the Future Coverage Modification List IE, the NG-RAN node2 may use the information to identify the future SSB beam deployment configuration to be enabled by the NG-RAN node1 at a time indicated by Time for Future Coverage State IE and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

If the Predicted Coverage Modification Cause IE set to "coverage" or "cell edge capacity" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue predicted in NG-RAN node1 and for configuring coverage state of its served cell(s).

If the Predicted Coverage Modification Cause IE set to "matching coverage state" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for configuring coverage state of its served cell(s).

Interactions with other procedures:

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier within the Neighbour NG-RAN Node List IE identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value ‘0’ indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of a cell that may replace all or part of the coverage of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value ‘0’ indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving, matching coverage state) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |
| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified. | YES | ignore |
| >Future Coverage Modification Item |  | 1..<maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 | NG-RAN Cell Global Identifier of the cell whose coverage will be modified. | – |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicates that the cell will be active and also indicates the future coverage configuration of the concerned cell. | – |  |
| >>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams whose coverage will be modified. | – |  |
| >>>Future SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB index | M |  | INTEGER (0..63) | Identifier of the SSB beam whose coverage will be modified. | – |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also Indicates the future coverage configuration of the concerned SSB beams. | – |  |
| >>Predicted Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, matching coverage state, ...) | Indicates the reason for the predicted coverage modification in NG-RAN node1. | – |  |
| >>Time for Future Coverage State | O |  | INTEGER (1..60, …) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information, in seconds. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

# Annex	- TP for BLCR to TS 38.473

<<< start of changes >>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.If the CCO issue detection IE set to "network energy saving" is included in the CCO Assistance Information IE, the gNB-DU may consider the indicated SSB beams by the Affected Cells and Beam IE are deactivated due to network energy saving.

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message,  the gNB-DU may use it to determine a future cell and/or beam configuration.

If the Neighbour Future Coverage Modification Notification IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may use it to adjust the coverage of its cells.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.211	CCO Assistance Information

This IE provides assistance information for the Capacity and Coverage (CCO) actions for specific CCO issues detected, and for network energy saving.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CCO issue detection | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving, matching coverage state) | Indicates the type of CCO issue detected, network energy saving cause or CCO state change caused to match the coverage configuration change in neighbour cell. |
| Affected Cells and Beams | O |  | 9.3.1.212 |  |

#### 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  | - |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | - |  |
| >>SSB Coverage Modification List |  | 0..1 |  |  | - |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …, network energy saving, matching coverage state) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

#### 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, matching coverage state, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<< end of changes >>>

#### 9.3.1.C	Neighbour Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by one or more neighbour node(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Future Coverage Modification Notification List |  | 1 |  |  |
| >Neighbour Future Coverage Modification Notification Item |  | 1..< maxnoofCellsinNG-RANnode > |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Neighbour Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Neighbour Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Neighbour Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Neighbour Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Neighbour Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |
| Future Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, matching coverage state, …) |  |
| Predicted Affected Cells and Beams | O |  | Affected Cells and Beams 9.3.1.212 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

### 9.4.5	Information Element Definitions

-- ASN1START

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

maxnoofTimeWindowMea,

maxnoPreconfiguredSRS,

maxnoHopsMinusOne,

maxnoAggCombinations,

maxnoAggregatedPosSRSCombinations,

maxnoofCellsinNG-RANnode

#### -- F

Future-Coverage-Modification-Notification ::= SEQUENCE {

future-coverage-Modification-List			Future-Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Future-Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxCellingNBDU)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

futurecellCoverageState				FutureCellCoverageState,

futureSSBCoverageModificationList	FutureSSBCoverageModification-List	OPTIONAL,

timeforFutureCoverageModificaiton	TimeforFutureCoverageModification	OPTIONAL,

futureCoverageModificationCause		FutureCoverageModificationCause		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>

#### -- N

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

Neighbour-Future-Coverage-Modification-Notification ::= SEQUENCE {

neighbour-future-coverage-Modification-List			Neighbour-Future-Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Neighbour-Future-Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Neighbour-Future-Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Neighbour-Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1.. maxnoofCellsinNG-RANnode)) OF Neighbour-Future-Coverage-Modification-Item

Neighbour-Future-Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

neighbourfuturecellCoverageState				NeighbourFutureCellCoverageState,

neighbourfutureSSBCoverageModificationList	NeighbourFutureSSBCoverageModification-List	OPTIONAL,

timeforneighbourFutureCoverageModificaiton	TimeforNeighbourFutureCoverageModification	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Neighbour-Future-Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Neighbour-Future-Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourFutureCellCoverageState ::= INTEGER (0..63, ...)

NeighbourFutureSSBCoverageModification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF NeighbourFutureSSBCoverageModification-Item

NeighbourFutureSSBCoverageModification-Item::= SEQUENCE {

sSBIndex						INTEGER(0..63),

neighbourfutureSSBCoverageState			NeighbourFutureSSBCoverageState,

iE-Extensions		ProtocolExtensionContainer { { NeighbourFutureSSBCoverageModification-Item-ExtIEs} }	OPTIONAL,

...

}

NeighbourFutureSSBCoverageModification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourFutureSSBCoverageState ::= INTEGER (0..15, ...)

NodeAssociatedInfoResult ::= SEQUENCE {

energyCost				EnergyCost			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { NodeAssociatedInfoResult-ExtIEs} } OPTIONAL,

...

}

NodeAssociatedInfoResult-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- P

Predicted-CCO-Assistance-Information ::= SEQUENCE {

predicted-CCO-issue-detection	Predicted-CCO-issue-detection	OPTIONAL,

predictedAffectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

timeforPredictedCCOIssue		TimeforPredictedCCOIssue,	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Predicted-CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

Predicted-CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Predicted-CCO-issue-detection	::=	ENUMERATED {coverage, cell-edge-capacity, ...}

FutureCoverageModificationCause	::=	ENUMERATED {coverage, cell-edge-capacity, matching-coverage-state, ...}

<<<<<<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

maxnoAggregatedPosSRSCombinations		INTEGER ::= 32

maxnoofCandidateCells					INTEGER ::= 8

maxnoofSSBIndices						INTEGER ::= 64

maxnoofCellsinNG-RANnode				INTEGER ::= 16384

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

<<< end of changes >>>