3GPP TSG-RAN WG3 Meeting #129	R3-255471

Bengaluru - India, 25th – 29th August 2025

Title: 	UE radio measurements for CCO

Source: 	Ericsson, Jio Platforms, Orange, Deutsche Telekom

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution we provide further discussion on the collection of UE radio measurements for AI/ML enabled CCO.

# 2. Discussion

## Validation of a predicted CCO issue

During the past meetings, different opinions have been expressed concerning “what is” feedback for AI/ML enabled CCO, and no consensus could be reached so far. Some companies questioned the possibility of collecting feedback for a predicted CCO issue, claiming that, if a future CCO state is applied to prevent a predicted CCO issue, the issue will not manifest itself, and for that, there would not be feedback to collect. Some other companies consider that the feedback relates to the future CCO state applied to prevent the predicted CCO issue, and there is a need to judge whether the action taken to prevent the predicted CCO issue, i.e., applying a future CCO state, was a good choice or not.

It should be clarified that “feedback” information is needed to build training data or to gather information that can lead to knowledge of the ground truth corresponding to the inference output of an AI/ML model. For this reason, our position is that feedback relates to the predicted CCO issue, which is the result of the AI/ML model inference. Indeed, according to the general framework agreed for AI/ML in RAN, feedback is used to verify whether a prediction in output from an AI/ML inference function was good or not, and the only prediction the standard supports for AI/ML enabled CCO is a predicted CCO issue.

We would like to also emphasise that not because a predicted CCO issue is resolved by the application of a Future CCO State it means that radio measurements will not reveal the build up to a CCO issue. To understand this, we consider the case of high levels of cross cell interference, i.e. CCO capacity issues. In this case, RSRQ measurements taken by UEs at cel edge will reveal that there is more and more cross cell interference, until a CCO state change is applied. If the application of the future CCO state resolves the issue, the RSRQ measurements will reveal no or moderate cross cell interference, otherwise cross cell interference will remain high or increase. It can therefore be concluded that radio measurements collected by UEs help understanding whether a CCO issue would occur, even in cases where the CCO issue is resolved by the application of a Future CCO State. The figure below explains the use case in more details.

Figure 1: Example of RSRQ trend during the build up to a coverage CCO Issue. Two cases are shown with respect to application of a Future Coverage State application

As it can be seen in Figure 1, the RSRQ curve will have an inflexion point when the Future Coverage State is applied, assuming that such action would resolve the Predicted CCO Issue. Otherwise, the RSRQ curve will continue rising. Hence, knowing the RSRQ measurements taken by UEs at cell edge reveals information on whether the Predicted CCO issue would have occurred (in cases where it was prevented) or if it has occurred (in cases where it was not prevented).

A similar case can be drawn for coverage CCO issues, where RSRP and SINR measurements would reveal similar information concerning the issue.

Observation 1: UE radio measurements, like cell level RSRP, RSRQ, SINR can reveal whether coverage or cross cell interference starts to degrade, before a CCO issue is predicted.

Observation 2: UE radio measurements, like cell level RSRP, RSRQ, SINR can reveal whether coverage or cross cell interference degrade or improve, before and after the CCO issue is predicted.

Observation 3: UE radio measurements, like cell level RSRP, RSRQ, SINR can reveal whether coverage or cross cell interference degrade or improve, before and after a future CCO state is applied.

We also note that, due to the fact that UE radio measurements can reveal whether a CCO issue is present or has been solved, the same measurements can be used by CU1 also to verify whether a predicted CCO issue is still valid, needs to be updated (e.g., different cells or beams are affected), or needs to be cancelled.

Proposal 1: Use UE radio measurements like cell level RSRP, RSRQ, SINR, as input to predict a CCO issue.

Proposal 2: Use UE radio measurements like cell level RSRP, RSRQ, SINR, to verify whether a predicted CCO issue is not present after the future CCO state is applied.

Proposal 3: CU can indicate that a predicted CCO issue is cancelled or indicate an update of previously sent predicted CCO issue (e.g., different affected cells and beams).

There could also be the case where a certain CCO issue is predicted, and even if a certain future CCO state is applied, it will not be able to fully prevent the issue to occur. The CU1 still needs to evaluate whether the coverage and cross cell interference are optimal. One obvious reason is that, if the coverage or the cross-cell interference will further degrade, we can expect that CU1 will eventually detect a CCO issue. In this case, the legacy CCO solution will apply.

Observation 4: If a future CCO state does not prevent a CCO issue, the legacy CCO solution applies.

Back to the importance of UE radio measurements, we discuss a simple example to understand for which UEs it is beneficial to collect such information. Let’s assume that in gNB1, DU1 determines to proactively address a CCO issue modifying the coverage of cell 1. To adapt to such future change, at gNB2, DU2 determines that it will change the coverage of neighbor cell 2. Due to these changes in cell coverage, some UEs (in the example, UE2 and UE3), that were served by one cell will be handed over to another cell. Some other UEs will continue to be served by the same gNB (in the example, UE1 and UE4). In the figure below: UE2 is handed over from cell 1 to cell 2, UE3 is handed over from cell 2 to cell 1, UE1 and UE4 will continue to be served by the same cell.

With the help of radio measurements collected before and after handover for UEs handed over between gNB1 and gNB2, the source gNB can understand whether a predicted CCO issue materialized, if it was successfully prevented, and if the predicted CCO issue would have occurred in case the future CCO state changes were not applied. We note that outgoing handover (as in the case of UE2) are of interest as they can help the gNB1 to evaluate whether the predicted CCO issue was successfully prevented or not (in which case measurements would reveal that the issue occurred).

For example, if the predicted CCO issue consisted of interference at cell edge, and if the predicted CCO issue was accurate, gNB1 should first monitor increasing interference measured by UE2, until the moment when the CCO state changes are applied. At this point in time UE2 would be handed over to Cell 2, and UE2 would report a good SINR as part of the UE performance feedback. This indicates that the CCO issue was successfully prevented and that it would have occurred if no CCO actions were taken. Similarly, if the predicted CCO issue was related to coverage, gNB1 would measure a decreasing RSRP before the CCO state changes. At CCO state change, UE2 would hand over to cell 2 and a good RSRP would be reported by UE2 in the area affected by the CCO state changes, indicating that this was successfully prevented and that it would have occurred if no CCO actions were taken.

Observation 5: From the perspective of gNB1, radio measurements of UEs subject to outgoing handovers can be used to evaluate the predicted CCO issue.

For UEs subject to outgoing handover, the gNB1 knows the UE radio measurements before the handover, and needs to receive the same type of information, collected at gNB2 after the handover. To achieve this, we can extend the Report Characteristics IE in the DATA COLLECTION REQUEST, to request UE radio measurement. To report the collected measurement, the UE Performance reporting framework can be reused.

Proposal 4: Extend the Report Characteristics in the Data Collection Reporting Request to enable the request of UE radio measurements.

Proposal 5: Extend the UE Performance IE to report UE radio measurements.

An example of how the Report Characteristics can be extended to request UE radio measurements is provided in the table below.

| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory, Tenth Bit = Predicted Slice Available Capacity Eleventh Bit = Slice Average UE Throughput DL Twelfth Bit = Slice Average UE Throughput UL Thirteenth Bit = Slice Average Packet Delay Fourteenth Bit = Slice Average Packet Loss DL Fifteenth Bit = UE measured RSRP Sixteenth Bit = UE measured RSRQ Seventeenth Bit = UE measured SINR Other bits are ignored by the NG-RAN node2. Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

An example of how UE Measurement Report that can be used to extend UE performance contained in the Xn Data Collection Update is provided in the table below.

9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |
| UE Measured RSRP | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |
| UE Measured RSRQ | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |
| UE Measured SINR | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |

Proposal 6: Agree the TP to XnAP provided in R3-255472.

# 3. Conclusions

Based on the discussion in this paper, the observations and proposals listed below are derived.

Observation 1: UE radio measurements, like cell level RSRP, RSRQ, SINR can reveal whether coverage or cross cell interference starts to degrade, before a CCO issue is predicted.

Observation 2: UE radio measurements, like cell level RSRP, RSRQ, SINR can reveal whether coverage or cross cell interference degrade or improve, before and after the CCO issue is predicted.

Observation 3: UE radio measurements, like cell level RSRP, RSRQ, SINR can reveal whether coverage or cross cell interference degrade or improve, before and after a future CCO state is applied.

Proposal 1: Use UE radio measurements like cell level RSRP, RSRQ, SINR, as input to predict a CCO issue.

Proposal 2: Use UE radio measurements like cell level RSRP, RSRQ, SINR, to verify whether a predicted CCO issue is not present after the future CCO state is applied.

Proposal 3: CU can indicate that a predicted CCO issue is cancelled or indicate an update of previously sent predicted CCO issue (e.g., different affected cells and beams).

Observation 4: If a future CCO state does not prevent a CCO issue, the legacy CCO solution applies.

Observation 5: From the perspective of gNB1, radio measurements of UEs subject to outgoing handovers can be used to evaluate the predicted CCO issue.

Proposal 4: Extend the Report Characteristics in the Data Collection Reporting Request to enable the request of UE radio measurements.

Proposal 5: Extend the UE Performance IE to report UE radio measurements.

Proposal 6: Agree the TP to XnAP provided in R3-255472.