3GPP RAN WG3 Meeting #129	R3-255497

# Bengaluru, IN, 25th – 29th Aug, 2025

Source:	CATT

Title:	(TP for TS38.423) Discussion on slice level UE performance

Agenda Item:	11.2

Document for:	Discussion and decision

- Introduction

In last meeting there were some FFSs left. This document provides some analysis on the open issues.

- Discussion

Misalignment between tabular and ASN.1

On the definition of Slice UE Performance IE, tabular and ASN.1 is not aligned in current TP.

The tabular is as below:

- 9.2.3.x2	Slice UE Performance

This IE indicates per Slice UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice UE Performance |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI UE Performance List |  | 1 |  | Indicates the UE performance per network slice |
| >>S-NSSAI UE Performance Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Slice Based UE Performance | M |  | 9.2.3.x4 |  |

The ASN.1 is as below:

SliceUEPerformance ::= SEQUENCE {

pLMNIdentity								PLMN-Identity,

s-NSSAIUEPerformance-List					S-NSSAIUEPerformance-List,

iE-Extensions								ProtocolExtensionContainer { { SliceUEPerformance-ExtIEs} } OPTIONAL,

...

}

SliceUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

In the tabular, multiple PLMNs (maxnoofBPLMNs) are supported when collecting slice UE performance. While in ASN.1, only one PLMN is included in sliceUEPerformance.

For UE Performance Collection, the following conditions are defined to terminate the collection:

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell.

.Based on the above statement, only for the case that intra-cell inter PLMN handover occurs during the Collection Time Duration, multiple PLMNs could be included in UE performance feedback.

In our opinion, intra-cell inter PLMN handover is a corner case, and multiple PLMNs are not necessary for UE performance feedback.

Proposal 1: It is proposed to align the tabular with ASN.1, i.e. Slice UE Performance IE in tabular is modified to include one PLMN rather than PLMN list.

FFS on Packetloss

On pack loss, there are some open issues as below:

| Average Slice Packet Loss DL | O |  | INTEGER (0..10000000, ...) [FFS on the range] | Corresponds to DL PDCP SDU Drop Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.2.1. |
| --- | --- | --- | --- | --- |
| Average Slice Packet Loss UL | O |  | INTEGER (0..10000000, ...) [FFS on the range] | Corresponds to • UL PDCP SDU Loss Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.1.1. |

Editor’s note: the semantics description of Average Slice Packet Loss to be further checked based on potential LS from SA5.

In our opinion, current range can meet the accuracy requirement and FFS can be removed.

Proposal 2: It is proposed to remove the FFS on the range of Packet Loss.

For Editor’s note, it could be removed at this moment. Later if LS from SA5 is received, RAN3 can make further update.

Proposal 3: It is proposed to remove the Editor’s note.

- Conclusion

Proposal 1: It is proposed to align the tabular with ASN.1, i.e. Slice UE Performance IE in tabular is modified to include one PLMN rather than PLMN list.

Proposal 2: It is proposed to remove the FFS on the range of Packet Loss.

Proposal 3: It is proposed to remove the Editor’s note.

- TP for TS 38.423

- 9.2.3.x2	Slice UE Performance

This IE indicates per Slice UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.2.4 |  |
| S-NSSAI UE Performance List |  | 1 |  | Indicates the UE performance per network slice |
| >S-NSSAI UE Performance Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  |
| >>Slice Based UE Performance | M |  | 9.2.3.x4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

- 9.2.3.x4	Slice Based UE Performance

This IE represents UE performance metrics per S-NSSAI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Slice UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.1. |
| Average Slice UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.2. |
| Average Slice Packet Delay | O |  | Average Packet Delay 9.2.3.187 | Corresponds to the Average Packet Delay per S-NSSAI as specified in TS 28.558 [XX], clause 6.3.1.1. |
| Average Slice Packet Loss DL | O |  | INTEGER (0..10000000, ...) | Corresponds to DL PDCP SDU Drop Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.2.1. |
| Average Slice Packet Loss UL | O |  | INTEGER (0..10000000, ...) | Corresponds to • UL PDCP SDU Loss Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.1.1. |