3GPP RAN WG3 Meeting #129	R3-255498

# Bengaluru, IN, 25th – 29th Aug, 2025

Source:	CATT

Title:	Open issues on the CCO use case

Agenda Item:	11.3

Document for:	Discussion and decision

- Introduction

In the last RAN3 meeting, we further discussed the issues on the CCO use case. According to the discussion, some agreements are achieved, as below:

Agreements:

- Adopt Opt2 (gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU)

- Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

However, there’re still some open issues, listed in below:

Timing information for predicted CCO issue over Xn is not needed?

Current mechanism enables the update of predicted CCO issue and/or future CCO state. A new detected CCO issue/a new predicted CCO issue for the same affected cells and beams after a predicted CCO issue will cancel the prediction.

Check the scenario whether has the possibility that there will be isolated multiple CCO issues detected for different cells or beams? Specific Cancel towards each CCO issue is needed?

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

In this contribution, we will further discuss the open issues on the CCO use case, and provide corresponding observations and proposals.

- Discussion

Whether to exchange time for predicted CCO issue over XnAP

The critical information for the receiving node is the time for the predicted future CCO state, as this enables the neighboring node to align its coverage configuration effectively. Once the gNB adjusts its coverage, the predicted CCO issue is resolved and will no longer occur. Given this dynamic, exchanging the time of the predicted CCO issue over the Xn interface becomes unnecessary. Therefore, the time for the predicted CCO issue does not need to be exchanged over the Xn interface.

Proposal 1: It is proposed that the time for the predicted CCO issue does not need to be exchanged over the Xn interface.

How to update/cancel the predicted CCO issue and/or future CCO state (F1, Xn)

The last RAN3 meeting has already agreed that specify mechanisms (F1, Xn) to update/cancel a prediction, and no new IE is needed to identify the previous prediction. For indicating update/cancel the predicted CCO issue, we need to discuss which mechanism to adopt separately for two scenarios:

- Case 1: Update the predicted CCO issue. For example, an updated predicted CCO issues or a real-time CCO issue are received through F1/Xn before the previously predicted CCO issues occur.

- Case 2: Cancel the predicted CCO issue itself. For example, a RAN node predicts that a coverage or capacity issue will occur somewhere, but later predicts that it will not occur.

For Case1, we think that current mechanism is sufficient for updating the predicted CCO issue. If updated predicted CCO issues or a real-time CCO issue is received through F1/Xn interfaces before the previously predicted CCO issues occur, this implicitly supersedes the previously predicted CCO issues. And the same mechanism applies to the future CCO state. If updated future CCO state is received through F1/Xn before the previously future CCO state change, this implicitly indicates that the previously future CCO state is unavailable

Proposal 2: If updated predicted CCO issues or a real-time CCO issue is received through F1/Xn before the previously predicted CCO issues occur, this implicitly supersedes the previously predicted CCO issues and the same mechanism applies to the future CCO state.

For Case 2, it is more appropriate to use a clear indication rather than an implicit one to indicate the cancellation of the CCO issue. Therefore, for indicating cancel of the predicted CCO issue, it is proposed to introduce a new code point i.e. CCO issue cancelled in Predicted CCO issue IE to cancel the predicted CCO issue over F1. And every planned coverage modification should be cancelled, with an indication specifying that the cause is due to the cancellation of the CCO issue. So it is also proposed to introduce a new code point i.e. CCO issue cancelled in Future Coverage Modification Cause IE to cancel the future CCO state over F1 and Xn. Additionally, for indicating cancel of coverage modification itself, it is feasible to reuse the “Future Coverage Modification List” to indicate no change by setting the “Future Cell/SSB Coverage State” of the concerned cell/SSB-area the same value as the current status.

As above, the corresponding stage 3 TP for 38.473 and 38.423 BL CR is provided in [2]. RAN3 is requested to discuss and agree the TP.

Proposal 3: For indicating cancel of the predicted CCO issue, it is proposed to introduce a new code point i.e. CCO issue cancelled in Predicted CCO issue IE and Future Coverage Modification Cause IE over F1 and Xn. Discuss and agree the provided TP in [2].

Whether the gNB-CU should analyze (e.g. predicts) the received modification list

There is some divergence on what a gNB-CU may do upon receiving a Future Coverage Modification List from its neighbour gNB. Two typical understanding includes:

- Alt 1: The gNB-CU forwards the cause of the received modification list toward the gNB-DU.

- Alt 2: The gNB-CU analyses (e.g. predicts) whether there will be any coverage or capacity problem after its neighbour implements the indicated modification, and if yes, where it will occur and what type it will be. The gNB-CU sends this analysis toward the corresponding gNB-DU(s).

There is a problem on Alt 1. In 38.401, for legacy CCO, it is stated “In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams” which implies that gNB-CU needs to detect the CCO issue based on current coverage status in the neighbourhood rather than just forwarding the “cause values” received from the neighbour CU.

On the other hand, Alt 2 follows the principle that AI/ML function should be located at the gNB-CU, and reuses the same mechanism on F1AP as in the triggering gNB (i.e. low complexity on specification). Therefore, we prefer Alt 2.

Proposal 4: gNB2-CU predicts whether there will be any coverage or capacity problem after gNB1-CU implements the indicated modification, and if yes, sends the predicted CCO issue (which is predicted to occur after gNB1 adjusts its coverage) toward gNB2-DU.

Node-level performance

The previous RAN3 meeting has already agreed that legacy UE performance measurement metrics can be reused for CCO. Additionally, node-level performance information can be a type of feedback information, e.g. used as inputs in calculating the reward in the training phase or as monitor metrics. We consider it especially useful for the CCO use case as coverage adjustment may equally affects the UEs subject of handover and the UEs kept in the original cell. We propose discussing what Node-level performance report (e.g. what metric in TS 28.552) can be introduced for the CCO use case.

Proposal 5: To discuss what Node-level performance report (e.g. what metric in TS 28.552) can be introduced for the CCO use case.

Miscellaneous

There is a few miscellaneous issues regarding the current version of F1AP BL CR:

The places of the future coverage modification list are different: It is directly within the XnAP NG-RAN NODE CONFIGURATION UPDATE message, but within the F1AP “Future Coverage Modification Notification” IE, which is a SEQUENCE ASN.1 type. It seems not needed to introduce such SEQUENCE ASN.1 type, so we prefer aligning F1AP with XnAP, and the F1AP “Neighbour Future Coverage Modification Notification” IE requires the same modification.

Proposal 6: To address the above mentioned miscellaneous issues. Discuss and agree the provided F1AP TP in [2].

- Conclusion

Proposal 1: It is proposed that the time for the predicted CCO issue does not need to be exchanged over the Xn interface.

Proposal 2: If updated predicted CCO issues or a real-time CCO issue is received through F1/Xn before the previously predicted CCO issues occur, this implicitly supersedes the previously predicted CCO issues and the same mechanism applies to the future CCO state.

Proposal 3: For indicating cancel of the predicted CCO issue, it is proposed to introduce a new code point i.e. CCO issue cancelled in Predicted CCO issue IE and Future Coverage Modification Cause IE over F1 and Xn. Discuss and agree the provided TP in [2].

Proposal 4: gNB2-CU predicts whether there will be any coverage or capacity problem after gNB1-CU implements the indicated modification, and if yes, sends the predicted CCO issue (which is predicted to occur after gNB1 adjusts its coverage) toward gNB2-DU.

Proposal 5: To discuss what Node-level performance report (e.g. what metric in TS 28.552) can be introduced for the CCO use case.

Proposal 6: To address the above mentioned miscellaneous issues. Discuss and agree the provided F1AP TP in [2].

- Reference

- RAN3_128_agenda_20250523_1530_EOM

- R3-255499 (TP to 38.423 and 38.473) Open issues on the CCO use case