3GPP RAN WG3 Meeting #129	R3-255500

Bengaluru, India, 25th – 29th August 2025

Source:	CATT

Title:	(TP on 37.340) Discussion on AI/ML in DC scenario

Agenda Item:	11.4

Document for:	Discussion and decision

- Introduction

This document is a revise of R3-253620, with minor changes including:

- Emphasise that it requires no Stage 3 change to support delivering UE performance delay results for split RBs.

- Change the place from TS 38.300 toward TS 37.340 to capture support of delivering UE performance delay results for split RBs.

- Discussion

  - Granularity of UE performance

We have agreed that the UE performance results on XnAP in handover scenario should be either per UE or per slice, whereas the results on E1AP should be per DRB. Now it is time to decide the granularity on XnAP for DC scenario, for SN-terminated bearers at least.

It seems a relatively simple approach to introduce essentially per-DRB ones. Considering that the MN cannot understand the DRB ID assigned by the SN, we prefer using a list of QFI to identify DRBs.

Proposal 1: Confirm that essentially per-DRB UE performance should introduced into XnAP DATA COLLECTION UPDATE message for DC purpose.

  - Merging measurement results for split DRB

DRBs can be established as split bearer (using both the MCG and the SCG) in DC scenario, and at least the delay metrics require measurement at nodes hosting the RLC layer.

The case for delay (both directions) is clear: in Section 4.2.1.9 of TS 38.314 it is already defined that how to generate the delay result for a split DRB depends on whether PDCP duplication is enacted for this DRB:

- If PDCP duplication is enacted (Section 4.2.1.9.1 and 4.2.1.9.2), the delay result of this DRB is defined as the smaller value (i.e. mininum) between the total delay averaged among all the packets delivered through the MCG and the total delay averaged among all the packets delivered through the SCG.

- If PDCP duplication is not enacted (Section 4.2.1.9.3 and 4.2.1.9.4), the delay result of this DRB is defined as the “weighted averaged” value, e.g.:

Table 4.2.1.9.3-1: Definition for total DL delay for DL PDCP non-duplicated packets per Split-DRB per UE Definition; Total DL delay for DL PDCP non-duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total DL delay for a Split-DRB when PDCP duplication is not enabled for the entire measurement period. Detailed Definition: M_DL(T,drbid)=I_DLMCG(T,drbid)×M_DLMCG(T,drbid)+I_DLSCG(T,drbid)×M_DLSCG(T,drbid)I_DL(T,drbid) I_DLT,drbid=I_DLMCG(T,drbid)+I_DLSCG(T,drbid), where explanations can be found in the table 4.2.1.9.3-2 below.. Table 4.2.1.9.3-2: Parameter description for total DL delay for DL PDCP non-duplicated packets per Split-DRB per UE M_DL(T,drbid); Total DL Delay per Split-DRB per UE during the time period T.. M_DLMCG(T,drbid); Total DL Delay on the MCG per split DRB per UE during the time period T.. M_DLSCG(T,drbid); Total DL Delay on the SCG per split DRB per UE during the time period T.. I_DLT,drbid; Total number of DL PDCP non-duplicated packets sent per Split-DRB per UE during the time period T.. I_DLMCG(T,drbid); Total number of DL PDCP non-duplicated packets sent on the MCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1.. I_DLSCG(T,drbid); Total number of DL PDCP non-duplicated packets sent on the SCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1..

We think at least for delays we should treat the existing method as principle. Other metrics are pending on the discussion on CU/DU split topic.

Proposal 2: Capture in TS 37.340 that average delays for split DRBs should be generated in accordance with Section 4.2.1.9 of TS 38.314. (There is no Stage 3 impact.)

  - Miscellaneous

In RAN3#127-bis we agreed that in DC scenario the “Collection Time Duration for UE Performance” start from the completion of SN addition. The semantic description of this IE (in the tabular) obviously needs change, but we missed it then.

Proposal 3: Revise the semantic description of the “Collection Time Duration for UE Performance” IE to cover the DC case.

- Conclusion

Proposal 1: Confirm that essentially per-DRB UE performance should introduced into XnAP DATA COLLECTION UPDATE message for DC purpose.

Proposal 2: Capture in TS 37.340 that average delays for split DRBs should be generated in accordance with Section 4.2.1.9 of TS 38.314. (There is no Stage 3 impact.)

Proposal 3: Revise the semantic description of the “Collection Time Duration for UE Performance” IE to cover the DC case.

The TP for TS 37.340 is shown in the annex, whereas the TP for TS 38.423 is provided in [1].

- Reference

[1] R3-255501; (TP for 38.423) AI/ML in DC scenario; CATT.

- Annex: TP for TS 37.340

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

[y]	3GPP TS 38.314: "NR; Layer 2 Measurements".

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

## 13.x	AI/ML for NR-DC

In NR-DC, an MN that supports AI/ML for NG-RAN as specified in TS 38.300 [3], can configure the collection and reporting of UE performance measurements from the SN after successful SN addition or SN change, if supported.

For UE performance feedback of split bearers:

-	Average packet delay results should be generated in accordance with Section 4.2.1.9 of TS 38.314 [y].