3GPP TSG-RAN WG3 Meeting #129	R3-255519

Bengaluru, India, 25 – 29 August 2025

Agenda item:	31.2

Source:	Nokia

Title:	Enhancement of Split Bearers

WID/SID:	TEI 19

Document for:	Discussion and Decision

# 1	Introduction

NR DC has been introduced to aggregate resources from cells of another node and split bearers can thus offer advantageous high bit rate. However, the efficiency of split bearer feature can be impaired by the use of IPSEC feature when used to enhance the network security.

This paper proposes enhancements to take full advantage of both split bearer feature and network security.

# 2	Using split bearers and IPSEC

NR DC has been introduced to aggregate resources from cells of another node and the key advantage of using split bearers across two nodes is to offer advantageous high bit rate. For example, the SCG cells may be of different frequency band and high throughput.

In case of MCG split bearers, PDCP PDUs are sent from master node to secondary node over Xn-U interface and conversely in case of SCG split bearers. These PDCP SDUs are already encrypted by PDCP in the hosting node.

Besides, the Xn-U interface can also carry a second type of traffic which is the Xn-U data forwarding traffic. This one is not encrypted over Xn-U and therefore an operator may require to turn on IPSEC tunnels over Xn-U to protect it.

However, if IPSEC tunnel is setup over Xn-U interface both split bearer traffic and data forwarding traffic will be carried over the IPSEC tunnel because no standards mechanism exist to segregate them.

This is illustrated below:

Figure 1: Typical deployment of NR-DC with traffic flow

For the Xn-U PDCP data split traffic, the IPsec encryption is however not needed and redundant because PDCP already ciphers the traffic. This redundant encryption leads to inefficiency due to node processing effort and leads to a reduction of bit rate for the split bearer whereas the split bearer feature in NR-DC is typically used to provide high bit rate via a secondary cell. Also, the latency of packets over the split bearer is increased due to the redundant IPsec encryption/decryption processing.

Observation 1: split bearer bit rate degrades whenever IPSEC is used over Xn-U.

It would therefore be beneficial to decouple split bearer traffic from data forwarding traffic over Xn-U so that only data forwarding traffic is carried over the IPSEC tunnel and split bearer traffic outside the IPSEC tunnel. Such enhancement would require to introduce standards means to make the two nodes exchange capability and cooperate in a multi-vendor context.

For example, the master node should propose such segregation of traffic to the secondary node and secondary node should support and accept the segregation to make this work.

Observation 2: segregation of split bearer traffic and data forwarding traffic over Xn-U requires standards support to make it interoperable.

A simple solution could be that the node initiating the Xn interface indicates in the Xn Setup Request whether it supports and want to activate this feature. If the peer Xn node support and accept to seggregate the traffic it can symmetrically indicate this in the Xn Setup Response message.

The Xn CR to implement this simple enhancement is provided in annex A.

Proposal 1: RAN3 to discuss the enhancement to segregate split bearer traffic and data forwarding traffic over Xn-U and the associated CR shown in annex A.

# 3	Conclusion

This paper has explained why it would be highly beneficial to segregate split bearer traffic and data forwarding traffic over Xn-U interface when IPSEC is used. It makes the following proposal:

Observation 1: split bearer bit rate degrades whenever IPSEC is used over Xn-U.

Observation 2: segregation of split bearer traffic and data forwarding traffic over Xn-U requires standards support to make it interoperable.

Proposal 1: RAN3 to discuss the enhancement to segregate split bearer traffic and data forwarding traffic over Xn-U and the associated CR shown in annex A.

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

# 5	CR for TS 38.423

3GPP TSG-RAN WG3 Meeting # 129	R3-25xxxx

# Bengaluru, Malta,  25 – 29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | xxxx | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of split bearer [Enhanced_splitbearer] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Whenever IPSEC is used over Xn-U the efficiency of the split bearer traffic (bit rate, latency, etc..) is impaired. It would be then useful if the master node and secodnary node could seggregate the split bearer traffic over Xn-U so that IPSEC does not apply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A PDCP Data Split Bypass indicator is added in the Xn Setup Request and Response messages to enable master node and secondary node to negotiate the bypass of IPSEC for split bearer traffic only. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR has an impact on the split bearer function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Low performance of split bearer traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.2, 9.2.3.96, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2: Xn Setup, successful operation

The NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to the candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the TNL Configuration Info IE is contained in the XN SETUP REQUEST message, the NG-RAN node2 shall, if supported, take this IE into account for IPSec establishment. In case the IP-Sec Transport Layer Address IE within the Extended UP Transport Layer Addresses To Add List IE is present and the GTP Transport Layer Address Info IE within the GTP Transport Layer Addresses To Add List IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE except the split bearer traffic if the PDCP Data Split Bypass Support IE is included in the XN SETUP REQUEST message and the NG-RAN node2 supports and accepts to segregate the split bearer traffic outside the IP-Sec tunnel. In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the endpoints given by the list of addresses in the GTP Transport Layer Address Info IE within the GTP Transport Layer Addresses To Add List IE if present. If the PDCP Data Split Bypass Support IE is contained in the XN SETUP REQUEST message associated with an IP-Sec Transport Layer Address IE and the NG-RAN node2 supports and accepts to segregate the split bearer traffic outside the IP-Sec tunnel it shall include the PDCP Data Split Bypass Support IE in the XN SETUP RESPONSE message.

If the TNL Configuration Info IE is contained in the XN SETUP RESPONSE message, the NG-RAN node1 shall, if supported, take this IE into account for IPSec establishment. In case the IP-Sec Transport Layer Address IE within the Extended UP Transport Layer Addresses To Add List IE is present and the GTP Transport Layer Address Info IE within the GTP Transport Layer Addresses To Add List IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE. In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the endpoints given by the list of addresses in the GTP Transport Layer Address Info IE within the GTP Transport Layer Addresses To Add List IE if present.

If the Partial List Indicator NR IE or the Partial List Indicator E-UTRA IE is set to "partial" in the XN SETUP REQUEST message the candidate NG-RAN node2 shall, if supported, assume that the List of Served Cells NR IE or the List of Served Cells E-UTRA IE in the XN SETUP REQUEST message includes a partial list of cells.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.96	TNL Configuration Info

This IE is used for signalling IP addresses of GTP-U endpoints and additionally of IPSec endpoints used for establishment of IPSec tunnels.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended UP Transport Layer Addresses To Add List |  | 0..1 |  |  |
| >Extended UP Transport Layer Addresses To Add Item |  | 1..<maxnoofExtTLAs> |  |  |
| >>IP-Sec Transport Layer Address | O |  | Transport Layer Address 9.2.3.29 | Transport Layer Addresses for IP-Sec endpoint. |
| >>PDCP Data Split Bypass Support | O |  | ENUMERATED (supported, …) | Proposed (in Xn Setup Request) or accepts (in Xn Setup Response) that PDCP split data traffic will not be sent over the IPSEC tunnel identified by the IP-Sec Transport Layer Address. |
| >>GTP Transport Layer Addresses To Add List |  | 0..1 |  |  |
| >>>GTP Transport Layer Addresses To Add Item |  | 1..<maxnoofGTPTLAs> |  |  |
| >>>>GTP Transport Layer Address Info | M |  | Transport Layer Address 9.3.2.29 | GTP Transport Layer Addresses for GTP end-points. |
| Extended UP Transport Layer Addresses To Remove List |  | 0..1 |  |  |
| >Extended UP Transport Layer Addresses To Remove Item |  | 0..<maxnoofExtTLAs> |  |  |
| >>IP-Sec Transport Layer Address | O |  | Transport Layer Address 9.2.3.29 | Transport Layer Addresses for IP-Sec endpoint. |
| >>GTP Transport Layer Addresses To Remove List |  | 0..1 |  |  |
| >>>GTP Transport Layer Addresses To Remove Item |  | 1..<maxnoofGTPTLAs> |  |  |
| >>>>GTP Transport Layer Address Info | M |  | Transport Layer Address 9.2.3.29 | GTP Transport Layer Addresses for GTP end-points. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-PDCPDataSplitBypassSupport,

maxEARFCN,

maxnoofAllowedAreas,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

ExtTLAs ::= SEQUENCE (SIZE(1..maxnoofExtTLAs)) OF ExtTLA-Item

ExtTLA-Item ::= SEQUENCE {

iPsecTLA							TransportLayerAddress				OPTIONAL,

gTPTransportLayerAddresses			GTPTLAs								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExtTLA-Item-ExtIEs} } OPTIONAL,

...

}

ExtTLA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDCPDataSplitBypassSupport	CRITICALITY ignore	EXTENSION PDCPDataSplitBypassSupport	PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

PDCPDataSplitBypassSupport:= ENUMERATED {

supported,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-MN-only-MDT-collection												ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo										ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric									ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest								ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation											ProtocolIE-ID ::= 474

id-PDCPDataSplitBypassSupport											ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>