3GPP TSG-RAN WG3 Meeting #129	R3-255560

# Bengaluru, India, 25th - 29th August 2025

Agenda Item:	9.2

Source:	Ericsson, Jio Platforms, China Telecom, CMCC

Title:	Extensions for enumerated type definitions over several Releases - and what can go wrong

Document for:	Discussion, education and approval

# 1	Introduction

Extending extendable ENUMERATED type definitions need to be handled with care when the extension is done simultaneously within different Releases, e.g. in Release N in order to introduce new features and in Release N-1 for correction or alignment, e.g. if some PHY related codepoints were forgotten to be introduced.

Illustrating the situation above can be done as follows:

Release N-1 introduced the following type definition:

Enum1  ::= ENUMERATED {a1, a2, a3, ...}

Release N introduced the following extension

Enum1  ::= ENUMERATED {a1, a2, a3, ..., e4, e5}

Then, for Release N-1, it has been identified, that some codepoints have been forgotten, so a6 and a7 would need to be added. But how to do that in Release N-1 and Release N?

Release N seems to be quite straight forward:

Enum1  ::= ENUMERATED {a1, a2, a3, ..., e4, e5, a6, a7}

Release N-1 may go with the following approach:

Enum1  ::= ENUMERATED {a1, a2, a3, ..., a6, a7}

But, Is that approach for Release N-1 correct?

According to PER encoding rules (ITU-T X.691), ENUMERATED types are encoded like INTEGERs, with the first enumeration assigned the value 0, etc, extensions likewise: first extension value 0, etc.

Figure 1

As shown in Figure 1, If the sending node X has implemented a6/a7 for Release N-1 as shown above and the receiving node Y has implemented a6/a7 for Release N as shown above, the receiving node Y would interpret "a6" as "e4" and "a7" as "e5", which shows, that the implementation of a6/a7 in the sending node X for Release N-1 cannot interoperate with the implementation of a6/a7 in the receiving node Y, so, this approach clearly does not allow nodes having implemented different Releases to interwork correctly.

The only way to introduce a6/a7 in Release N-1 after introducing e4/e5 in Release N works as follows:

Enum1  ::= ENUMERATED {a1, a2, a3, ..., dummy4, dummy5, a6, a7}

With adding semantics descriptions like: "... should not send/should ignore dummy4/5 if received ...".

Figure 2

Why this lengthy, boring introduction, most should be already familiar with?

Unfortunately, such incompatible implementation of ENUMERATED type extensions in different Releases were manufactured in 38.473 and 38.455 in Rel-17.

NOTE:	Detecting such cases is rather straight forward: use the "compare" function of "Word" and search for respective patterns, i.e. new codepoints not added at the end of the codepoint enumeration in the type definition and start to investigate what happened.

# 2	Discussion

## 2.1	Correcting extension of the FR1-Bandwidth type definition in 38.473

#### 2.1.1	Irregularities in current type definition for the FR1-Bandwidth type definition:

TS 38.473 v16.21.0 defines the type FR1-Bandwith as follows (Rel-16 additions in green):

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw15, bw25, bw30, bw60}

TS 38.473 v17.13.0 defines the type FR1-Bandwith as follows (Rel-17 additions in blue):

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw15, bw25, bw30, bw60, bw35, bw45, bw70, bw90}

TS 38.473v18.6.0 defines the type FR1-Bandwidth as follows (Rel-18 additions in red)

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw160, bw200, bw15, bw25, bw30, bw60, bw35, bw45, bw70, bw90}

Lamentation 1:	Along discussion in the Introduction section, nodes having implemented different Releases of that type definition cannot interwork.

#### 2.1.2	Time-line of extending FR1-Bandwidth type definition

38.473v18.1.0 (2024-03) introduced via CR1180 the following (red pseudo revision marks):

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw160, bw200}

38.473v16.20.0 (2024-12) introduced via CR1512, 
38.473v17.11.0 (2024-12) introduced via CR1513
the following (green pseudo revision marks):

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw15, bw25, bw30, bw60 }

38.473v18.4.0 (2024-12) introduced via CR1514 the following (red the previous Rel-18 addition, green pseudo revision marks):

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw160, bw200, bw15, bw25, bw30, bw60}

38.473v17.11.0 (2024-12) introduced via CR1515 the following (blue pseudo revision marks):

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw35, bw45, bw70, bw90}

38.473v18.4.0 (2024-12) introduced via CR1516 the following (blue pseudo revision marks, but the CR was not using latest Rel-18 version):

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw35, bw45, bw70, bw90}

Observation 1:	There have been attempts made by the rapporteur to at least combine CRs 1512/1513/1514 and CRs 1515/1516 consistently throughout the Releases, but the fact that Rel-18 has already included codepoints "bw160" and "bw200" 3 quarters ago for Rel-18 was completely forgotten.

Observation 2: The fact that CR1516 was not based on the latest Rel-18 version of the TS contributed to the fault, hence, this is another example how important it is to use the latest version of the specification as basis for a CR and not just "extrapolate" the actual content of different releases "manually".

Observation 3:	It is not sufficient that just the rapporteur inspects the drafts of the new TSs, more than one pair of eyes is necessary for maintaining a proper specification - especially the main proponents of the respective CRs have the responsibility to check whether their addition to the TS is done is an appropriate way.

#### 2.1.3	How should it have been implemented?

This is how Rel-16 should look like (purple pseudo revision marks):

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., rel-18-dummy1, rel-18-dummy2, bw15, bw25, bw30, bw60}

This is how Rel-17 should look like (purple pseudo revision marks):

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ... , re-18-dummy1, rel-18-dummy2, bw15, bw25, bw30, bw60, bw35, bw45, bw70, bw90}

Again, the Rel-18 version for reference:

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw160, bw200, bw15, bw25, bw30, bw60, bw35, bw45, bw70, bw90}

Proposal 1:	Given the fact, that the codepoints "bw160" and "bw200" have been introduced 3 quarters before the other codepoints have been introduced we propose to correct this error in Rel-16 and Rel-17 specifications (only).

## 2.2	Correcting extension of the BandwidthSRS type definition in 38.455

#### 2.2.1	Irregularities in current type definition for the BandwidthSRS type definition:

TS 38.455 v16.15.0 defines the type FR1-Bandwith as follows (Rel-16 additions in green):

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ..., mHz15, mHz25, mHz30, mHz60},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ...},

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

TS 38.455 v17.10.0 defines the type FR1-Bandwith as follows (Rel-17 additions in blue):

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ..., mHz15, mHz25, mHz30, mHz60, mHz35, mHz45, mHz70, mHz90},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

TS 38.455v18.6.0 defines the type FR1-Bandwidth as follows (Rel-18 additions in red)

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ... ,mHz160, mHz200, mHz15, mHz25, mHz30, mHz60, mHz35, mHz45, mHz70, mHz90},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mHz600, mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

Lamentation 2:	Along discussion in the Introduction section, nodes having implemented different Releases of that type definition cannot interwork.

#### 2.1.2	Time-line of extending BandwidthSRS type definition

38.455v17.7.0 (2024-03) via CR0137 and 38.455v18.1.0 (2924-03) via CR0138 introduced the following:

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ...},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

38.455v18.1.0 (2024-03) introduced via CR0113 the following:

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ... ,mHz160, mHz200},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mHz600, mHz800},

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

BandwidthSRS-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

38.455v16.14.0 (2024-12) introduced via CR0171, (based on "manually fabricated content of v16.13.0)

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ..., mHz15, mHz25, mHz30, mHz60},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ...},

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

38.455v17.9.0 (2024-12) introduced via CR0172, (based on "manually fabricated content of v17.8.0)
the following (green pseudo revision marks):

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ..., mHz15, mHz25, mHz30, mHz60},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mhz800, mHz1600, mHz2000},

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

38.455v18.4.0 (2024-12) introduced via CR0173 the following (red the previous Rel-18 addition, green pseudo revision marks):

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ... ,mHz160, mHz200, mHz15, mHz25, mHz30, mHz60},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mHz600, mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

38.45517.9.0 (2024-12) introduced via CR0174 the following (blue pseudo revision marks):

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ... ,mHz160, mHz200, mHz35, mHz45, mHz70, mHz90},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

38.473v18.4.0 (2024-12) introduced via CR0175 the following (blue pseudo revision marks):

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ... ,mHz160, mHz200, mHz35, mHz45, mHz70, mHz90},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mHz600, mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

Observation 4:		what a chaos.

#### 2.1.3	How to correct this?

Proposal 2:	We propose the same approach as for F1AP, i.e. correct Rel-16 and Rel-17 so that it fits to Rel-18.

This is how Rel-16 should look like:

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ... , rel-18-dummy1, rel-18-dummy2, mHz15, mHz25, mHz30, mHz60},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ...},

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

This is how Rel-17 version should look like (purple pseudo revision marks):

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ..., rel-18-dummy1, rel-18-dummy2, mHz15, mHz25, mHz30, mHz60, mHz35, mHz45, mHz70, mHz90},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., rel-18-dummy3, mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

Again, Rel-18 version as reference:

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ... ,mHz160, mHz200, mHz15, mHz25, mHz30, mHz60, mHz35, mHz45, mHz70, mHz90},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mHz600, mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

## 2.3	Correcting extension of the PosSIB-Type type definition in 38.455

#### 2.2.1	Irregularities in current type definition for the PosSIB-Type type definition:

| TS 38.455v16.15.0 PosSIB-Type ::= ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType2-24, posSibType2-25, posSibType3-1, posSibType4-1, posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3, ... } | TS 38.455v17.10.0 PosSIB-Type ::= ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType2-24, posSibType2-25, posSibType3-1, posSibType4-1, posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3, ..., posSibType1-9, posSibType1-10, posSibType6-4, posSibType6-5, posSibType6-6, posSibType2-17a, posSibType2-18a, posSibType2-20a } | TS38.455v18.6.0 PosSIB-Type ::= ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType2-24, posSibType2-25, posSibType3-1, posSibType4-1, posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3, ..., posSibType1-9, posSibType1-10, posSibType6-4, posSibType6-5, posSibType6-6, posSibType1-11, posSibType1-12, posSibType2-17a, posSibType2-18a, posSibType2-20a, posSibType2-26, posSibType2-27, posSibType6-7, posSibType7-1, posSibType7-2, posSibType7-3, posSibType7-4 } |
| --- | --- | --- |

posSibType2 1-9,1-10,6-4,6-5,6-6 introduced in CR0063 (RP-221131) in v17.6.0,

posSibType 2-17a, 2-18a, 2-20a introduced in CR0146 (RP-241114) in v17.8.0 and in CR0147 in v18.2.1

posSibType1-11, 1-12, 2-26, 2-27, 6-7, 7-1, 7-2, 7-3, 7-4 introduced in CR0148 (RP-241101) in v18.2.1.

Lamentation 3:	Along discussion in the Introduction section, nodes having implemented different Releases of that type definition cannot interwork.

Observation 5:		a bit of another chaos.

#### 2.1.2	How to correct this?

For the Pos SIB issue, Rel-17 needs to be corrected by introducing two dummy values as shown below with purple pseudo revision marks.

| TS 38.455v16.15.0 PosSIB-Type ::= ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType2-24, posSibType2-25, posSibType3-1, posSibType4-1, posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3, ... } | TS 38.455v17.10.0 PosSIB-Type ::= ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType2-24, posSibType2-25, posSibType3-1, posSibType4-1, posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3, ..., posSibType1-9, posSibType1-10, posSibType6-4, posSibType6-5, posSibType6-6, dummy-Rel-18-1, dummy-Rel-18-2, posSibType2-17a, posSibType2-18a, posSibType2-20a } | TS38.455v18.6.0 PosSIB-Type ::= ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType2-24, posSibType2-25, posSibType3-1, posSibType4-1, posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3, ..., posSibType1-9, posSibType1-10, posSibType6-4, posSibType6-5, posSibType6-6, posSibType1-11, posSibType1-12, posSibType2-17a, posSibType2-18a, posSibType2-20a, posSibType2-26, posSibType2-27, posSibType6-7, posSibType7-1, posSibType7-2, posSibType7-3, posSibType7-4 } |
| --- | --- | --- |

Proposal 2:	We propose to correct Rel-17 NRPPa PosSIB-Type type definition by adding two dummy codepoints so that they fit to Rel-18.

# 3	Conclusion and Proposals

Proposal:	Discuss the above shown inconsistencies, internalize the lessions-learned and correct the errors along CRs submitted in documents:

- R3-255561 Correcting the extension of FR1-bandwidth type definition, Rel-16 F1AP CR

- R3-255562 Correcting the extension of FR1-bandwidth type definition, Rel-17 F1AP CR

- R3-255563 Correcting the IE extension definition for bandwidth SRS, Rel-16 NRPPa CR

- R3-255564 Correcting the IE extension definition for bandwidth SRS, Rel-17 NRPPa CR

- R3-255565 Correcting the IE extension definition for SRS Pos SIB Type, Rel-17 NRPPa CR

# 4	References

[1]	"reference"