3GPP TSG-RAN WG3 Meeting #129	     R3-255585

Bengaluru, India, 25 – 29 August, 2025

Title:	[Draft] LS on MBS Communication Service Type

Response to:	S4-251096/R3-255028

Release:	Release 18

Work Item:	NR_QoE_enh-Core

Source:	Nokia [to be RAN3]

To:	SA4, RAN2

Cc:	SA5

Contact Person:

#### Name:	Håkon Helmers

E-mail Address:	hakon.helmers@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA4 for their reply LS on MBS Communication Service Type.

RAN3 understands that, based on the @communicationServiceType attribute, the application layer measurement configuration defined in TS 26.247 may apply for any combination of the following communication service types: unicast service, MBS broadcast service, MBS multicast service.

RAN3 would like to inform SA4 that in current RAN3 specification, a given QMC configuration may apply for one of the listed service types, but any combination of service types is today not supported. However, NGAP and XnAP signalling could as such be enhanced to support combinations of service types, either as correction in Rel-18 or in later releases.

But RAN3 would also like to inform SA4 that the solution retained in Rel-18 is based on particular handling of QMC sessions configured for MBS broadcast service. The configuration for such sessions is transferred to the UE and stored in the UE in RRC_IDLE and retrieved by the gNB when the UE reconnects. When the gNB receives such configuration from the UE, it will as per current specification assume that the corresponding QMC session applies to MBS broadcast service only. One option here could be to update TS 38.300 to specify that QMC configurations uploaded from the UE apply to e.g. all communication services (including unicast), but support of a configurable list of communication services seems to require additional information retrieved by the gNB from the UE.

RAN3 would therefore like to ask RAN2 about feasibility of providing additional information from the UE to the gNB when the UE reconnects from RRC_IDLE so that QMC sessions, applicable to any combination of communication service types, can be supported.

2. Actions:

To RAN2 : 	RAN3 kindly asks RAN2 to provide feedback about feasibility of providing additional information from the UE to the gNB when the UE reconnects from RRC_IDLE so that QMC sessions applicable to any combination of communication service types can be supported.

To SA4: 	RAN3 kindly asks SA4 to take the above into account, and provide further feedback if needed.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #129-bis			13 - 17 October 2025			Prague, CZ

TSG-RAN WG3 Meeting #130			17 - 21 	November 2025			Dallas, US